IESO Charge Types and Equations

Issue 77.0

PUBLIC

LIST

IMP_LST_0001

Public

This document enumerates the various charge types and equations used in the IESO settlements process for IESO-Administered markets that are subject to a functional deferral, and those that are NOT subject to a functional deferral.

This document is provided for stakeholder engagement purposes.  Proposed changes, to be effective for the December 2022 capacity auction, are indicated based on the current version of this document.  Please note that additional changes to this document may be incorporated as part of future engagement on design enhancements to the capacity auction or other IESO activities prior to this document taking effect.

Disclaimer

The posting of documents on this Web site is done for the convenience of market participants and other interested visitors to the IESO Web site. Please be advised that, while the IESO attempts to have all posted documents conform to the original, changes can result from the original, including changes resulting from the programs used to format the documents for posting on the Web site as well as from the programs used by the viewer to download and read the documents. The IESO makes no representation or warranty, express or implied, that the documents on this Web site are exact reproductions of the original documents listed. In addition, the documents and information posted on this Web site are subject to change. The IESO may revise, withdraw or make final these materials at any time at its sole discretion without further notice. It is solely your responsibility to ensure that you are using up-to-date documents and information.

This document may contain a summary of a particular market rule. Where provided, the summary has been used because of the length of the market rule itself. The reader should be aware, however, that where a market rule is applicable, the obligation that needs to be met is as stated in the “Market Rules”.  To the extent of any discrepancy or inconsistency between the provisions of a particular market rule and the summary, the provision of the market rule shall govern.

See also “Notice to Electricity Storage Participants” in Section 2.2.

Document ID	IMP_LST_0001

Document Name	IESO Charge Types and Equations

Issue	Issue 77.0

Reason for Issue	Updated for Baseline 46.0

Effective Date	September 15, 2021

Document Change History

| Issue | Reason for Issue | Date |
| --- | --- | --- |
| - For change history prior to Issue 22.0, see Issue 29.0 of the CT&E. - For change history for Issue 22.0 to Issue 29.0, see Issue 38.0 of the CT&E. - For change history for Issue 30.0 to Issue 49.0, See Issue 59.0 of the CT&E |  |  |
| 50.0 | Updated for Baseline 30.0 | September 11, 2013 |
| 51.0 | Updated for Baseline 30.1 | December 4, 2013 |
| 52.0 | Update for Baseline 31.1 | June 4, 2014 |
| 53.0 | Update for Baseline 32.0 | September 10, 2014 |
| 54.0 | Updated for Baseline 33.0 | March 4, 2015 |
| 55.0 | Updated for Baseline 34.0 | September 9, 2015 |
| 56.0 | Updated for Baseline 34.1 | December 2, 2015 |
| 57.0 | Updated for Baseline 35.0 | March 2, 2016 |
| 58.0 | Updated for Baseline 35.1 | June 1, 2016 |
| 59.0 | Updated for Baseline 36.0 | September 14, 2016 |
| 60.0 | Updated for Baseline 36.1 | December 7, 2016 |
| 61.0 | Updated for Baseline 37.0 | March 1, 2017 |
| 62.0 | Updated for Baseline 38.1 | December 6, 2017 |
| 63.0 | Updated for Baseline 39.0 | March 7, 2018 |
| 64.0 | Updated for Baseline 40.0 | September 12, 2018 |
| 65.0 | Updated for Baseline 41.0 | March 6, 2019 |
| 66.0 | Updated for Baseline 41.1 | June 5, 2019 |
| 67.0 | Updated for Baseline 42.0 | September 11, 2019 |
| 68.0 | Updated for Baseline 42.1 | December 4, 2019 |
| 69.0 | Updated in advance of Baseline 43.1 | April 23, 2020 |
| 70.0 | Updated in advance of Baseline 43.1 for the commitment period beginning May 1, 2021 | May 4, 2020 |
| 71.0 | Updated for Baseline 44.0 | September 16, 2020 |
| 72.0 | Updated in advance of Baseline 44.1 | November 25, 2020 |
| 73.0 | Updated for Baseline 44.1 | December 2, 2020 |
| 74.0 | Updated to meet accessibility requirements pursuant to the Accessibility for Ontarians with Disabilities Act. | December 18, 2020 |
| 75.0 | Issued in advance of Baseline 45.0. Updated to include electricity storage participation. | February 26, 2021 |
| 76.0 | Updated for Baseline 45.1 | June 2, 2021 |
| 77.0 | Updated for Baseline 46.0 | September 15, 2021 |

Related Documents

| Document ID | Document Title |
| --- | --- |
| MDP_PRO_0033 | Market Manual 5: Settlements, Part 5.5: Physical Markets Settlement Statements |

Table of Contents

Table of Contents	i

List of Figures	iii

Table of Changes	iv

1.	Introduction	1

1.1	Purpose	1

1.2	Scope	1

1.3	Tax Treatment	1

1.4	Who Should Use This Document	1

1.5	Conventions	1

1.6	How This Document is Organized	2

2.	IESO Charge Types and Equations that are Part of an Active IESO-Administered Market	3

2.1	Variable Descriptions	3

2.2	Charge Types and Equations	34

2.3	Rounding Conventions – by Settlement Variable	212

2.3.1	Key to the Table of Rounding Conventions for Individual Settlement Variables	212

2.4	Rounding Conventions – by Charge Type	229

2.4.1	General Notes	229

2.4.2	Key to the Table of Rounding Conventions	229

2.5	Settlement of Physical Bilateral Contracts	271

2.5.1	Governing Rules	271

2.5.2	The Nature of the Bilateral Contract Quantity	272

2.5.3	Time Resolution of Bilateral Contract Quantities and Rounding	275

2.5.4	Allocation of Hourly Uplift Components Between Buying and Selling Market Participants	276

2.6	Exemptions from the Day-Ahead Import Failure Charge, Day-Ahead Export Failure Charge, and Day-Ahead Linked Wheel Failure Charge	279

2.6.1	Purpose of this Section	279

2.6.2	Objective of the “Offer Price Test”	279

2.6.3	How the Offer Price Test Works	279

2.6.4	Input Data:	280

2.6.5	Decision Logic Applied During the Offer Price Test for Import Transactions:	281

2.6.6	Decision Logic Applied During the Offer Price Test for Export Transactions:	282

2.6.7	Decision Logic Applied During the Offer Price Test for Linked Wheel Transactions:	284

References	1

List of Figures

Figure 2-1 – Example of an Import Transaction that PASSES the “Offer Price Test”	285

Figure 2-2 – Example of an Import Transaction that FAILS the “Offer Price Test”	286

Figure 2-3 – Example of an Export Transaction that PASSES the “Offer Price Test”	287

Figure 2-4 – Example of an Export Transaction that PASSES the “Offer Price Test”	288

Table of Changes

| Reference (Section and Paragraph) | Description of Change |
| --- | --- |
| 2.1 | Replaced “IESO Portal” with “IESO Gateway” in variable ‘DA_CGC’ description. |
| 2.1 | Updated “HST Tax Treatment for U.S., Manitoba, and Quebec Generation (%)” column for capacity obligation charge types CT 1314, 1315, 1316, 1317, 1318, 1319, 1320. |
| 2.2 | Included new charge type CT 137 “Generation Cost Guarantee - Output Based Pricing System Reimbursement Settlement Amount”. |
| 2.2 | Updated charge type CT 183 “Generation Cost Guarantee Recovery Debit” equation to include new charge type CT 137. |
| 2.4.2 | Added new charge type CT 137 “Generation Cost Guarantee - Output Based Pricing System Reimbursement Settlement Amount”. |

# Introduction

## Purpose

The purpose of this document is to provide the reader with the formulas and variable definitions behind each different charge type implemented in the IESO Settlements process including tax treatment. Furthermore, this document relates each charge type to the high-level description of the settlement amount within the IESO market rules and, where applicable, notes any aspects of the implementation of the charge type itself.

## Scope

- This document provides the formulas and the HST Tax treatment for each charge type implemented in the IESO Settlements System and those charge types which are currently the subject of a Functional Deferral.  This document does not, however, provide the format of the information provided to market participants on settlement statements with respect to each charge type.  For more information on these topics, the reader is directed to the following Technical Interface Document - “Format Specification for Settlement Statement Files and Data Files”.

## Tax Treatment

The IESO is a registrant for purposes of the Excise Tax Act and all or substantially all of the supplies made by the IESO are taxable for GST/HST purposes.

The IESO administers the IESO-administered markets in compliance with the current provisions of the Excise Tax Act and the published rulings, administrative policies, and assessing practices of the Canada Revenue Agency. The IESO conducts regular tax reviews with its advisors to ensure that transactions within the IESO-administered markets comply with the foregoing.

Market participants should consult their own legal and tax advisors for advice with respect to the tax consequences of transactions in the IESO-administered markets.

## Who Should Use This Document

This document is intended for market participants in the IESO-administered markets who are seeking information regarding the calculations of settlement amounts related to each charge type.  Depending on the activity of the market participant in the various IESO-administered markets, these charge types may have varying degrees of relevance to each market participant with respect to their financial settlements.

## Conventions

Usage of an italicized term shall take on the meaning ascribed to that term in the IESO market rules.

Unless otherwise noted, usage of variable subscripts and superscripts within this document shall mirror the same usage with in Chapter 9 of the IESO market rules.  One notable exception is the usage of notation to sum across settlement amounts for charge type “c”.  This is further noted in Section 2.2 of this document.

## How This Document is Organized

This document is divided in 6 major subsections as follows:

Section 2.1:	A table containing a description of each variable used within Section 2.2.

Section 2.2:	A table describing IESO charge types and equations that are part of an active IESO-administered market.

Section 2.3:	This section contains a description of rounding conventions for variables described in Section 2.1.

Section 2.4:	This section contains a description of rounding conventions for charge type calculations described in Section 2.2.

Section 2.5:	This section provides a description of physical bilateral contract quantities, their usage by the settlements system, and their use by market participants as a vehicle for transferring components of hourly uplift.

Section 2.6:	This section describes how Day-Ahead import, export and linked wheel transactions are subject to an “Offer Price Test” in order to determine if they are exempt from the Day-Ahead Failure Charges (charge types, 1135, 1136 and 1134).

– End of Section –

# IESO Charge Types and Equations that are Part of an Active IESO-Administered Market

## Variable Descriptions

The following table contains descriptions of each variable used within Section 2.2, describing IESO charge types and equations that are part of an active IESO-administered market.

| Key to the Table Below |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable used within Section 2 | Data Description | Description | Market Rules Reference | Relation to the corresponding variable description within the IESO Market Rules |
| This column denotes the abbreviated name of each variable used within Section 2.2. | The full name of each variable used within Section 2.2. | A brief description of each variable used within the formulas illustrated within Section 2.2. | The relevant reference to the variable in question within the IESO market rules. The format of each reference is: [Chapter] [Section no.] e.g. Chapter 9 Section 3.1.6 would appear as: 9.3.1.6 | This section notes any aspects of the implementation of the variable within the IESO settlements process which are otherwise not described in the IESO market rules – OR – refers the reader to the appropriate documentation. |

| Key to the Table Below |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable used within Section 2 | Data Description | Description | Market Rules Reference | Relation to the corresponding variable description within the IESO Market Rules |
| AAD | Adjustment Account Disbursement | The total dollar value of all disbursements from the IESO adjustment account authorized by the IESO Board in the current energy market billing period. | 9.6.18.6 | Same as IESO market rules. |
| AQEIk,hm,t | Allocated Quantity of Energy Injected | Allocated quantity in MWh of energy injected by market participant ‘k’ at RWM ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.9 | Represents only those quantities derived from loss-adjusted and totalized metering data. Quantities derived from interchange schedule data is captured in the variable SQEI (see below). |
| AQEWk,hm,t | Allocated Quantity of Energy Withdrawn | Allocated quantity in MWh of energy withdrawn by market participant ‘k’ at RWM ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.9 | Represents only those quantities derived from loss-adjusted and totalized metering data. Quantities derived from interchange schedule data is captured in the variable SQEW (see below). |
| AQORr,k,hm,t | Allocated Quantity of Operating Reserve | Allocated quantity in MW of class r reserve for market participant ‘k’ at RWM ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.9 | Same as IESO market rules and equivalent to DQSR (see below). |
| BE | Energy Offers | A matrix of ‘n’ price-quantity pairs offered by market participant ‘k’ to supply energy during settlement hour ‘h’. Offer prices in this matrix may be altered to a “lower limit” for the purposes of calculating charge type 105 (Congestion Management Settlement Credit for Energy) where any such offer price: - Is associated with a generation facility located within Ontario; or imports and - Is less than a specified “lower limit” where such limit is the lesser of $0.00/MWh and the energy market price for the applicable dispatch interval. The situational criteria and threshold for applying such adjustments are further described in IESO market rules section 9.3.5.6. and 9.3.5.7. | 9.3.5.2, 9.3.5.6 and 9.3.5.7 | Same as IESO market rules. |
| BL | Energy Bids | A matrix of ‘n’ price-quantity pairs bid by market participant ‘k’ to withdraw energy by a dispatchable load during settlement hour ‘h’. | 9.3.5.2 | Same as IESO market rules. |
| BRr | Operating Reserve Offers | A matrix of n price-quantity pairs offered by market participant ‘k’ to supply class r operating reserve during settlement hour ‘h’. | 9.3.5.2 | Same as IESO market rules. |
| BCQs,k,hm,t | Physical Bilateral Contract Quantity of Energy bought | Physical bilateral contract quantity of energy in MWh bought by buying market participant ‘k’ from selling market participant ‘s’ at RWM or intertie metering point ‘m’ for each metering interval ‘t’ in settlement hour ‘h’. | 9.3.1.6 | Same as IESO market rules. |
| BCQk,b,hm,t | Physical Bilateral Contract Quantity of Energy sold | Physical bilateral contract quantity of energy in MWh sold by selling market participant ‘k’ to buying market participant ‘b’ at RWM or intertie metering point ‘m’ for each metering interval ‘t’ in settlement hour ‘h’. | 9.3.1.6 | Same as IESO market rules. |
| CICAPk | Cleared ICAP | An amount in MW per capacity auction resource for capacity market participant ‘k’, and function of cleared UCAP in addition to any applicable performance adjustment as specified in the applicable market manual. | 11 | Same as IESO market rules. |
| CACP | #### Capacity Auction Clearing Price | The capacity auction clearing price for the obligation period and capacity auction resource. | N/A | Refer to Market Manual 5.5 |
| CACPh | #### Hourly Capacity Auction Clearing Price | The capacity auction clearing price for the obligation period and capacity auction resource divided by the hours of availability for the day. | N/A | Refer to Market Manual 5.5 |
| CAEOh | #### Capacity Auction Energy Offer | The energy offer quantity calculated for capacity market participant ‘k’ as the quantity of capacity provided by the associated capacity auction resource delivering the auction capacity. | N/A | Refer to Market Manual 5.5 |
| CBOCk | #### Buy-out Capacity | The buy-out capacity is an amount that is being reduced from the capacity obligation for capacity market participant ‘k’. | N/A | Refer to Market Manual 5.5 |
| CBMPk | #### Total net volume of electricity withdrawn from the IESO-controlled grid by applicable Class B market participant or licensed distributor that is also a market participant for the month | The total net volume of electricity withdrawn from the IESO-controlled grid by applicable Class B market participant (as that term is defined in the regulation) or licensed distributor that is a market participant ‘k’ for the month. | N/A | N/A – See regulations. |
| CBRR | #### Global adjustment Class B recovery rate | Global Adjustment Class B recovery rate for the month per Ontario Regulation 429/04. | N/A | N/A – See regulations. |
| CCOk | #### Capacity Obligation (MW) | The capacity obligation, also refered to as clared UCAP, for the obligation period per capacity auction resource for capacity market participant ‘k’. The initial capacity obligation is acquired through a capacity auction and subject to being reduced via the buy-out process. | N/A | Refer to Market Manual 5.5 |
| CGC | - Submitted Combined Guaranteed Costs | A financial amount consisting of fuel cost components defined on a per-start basis for a given generation unit calculated in a manner consistent with the applicable market manual, and encompassing the following elements: - Fuel and operation and maintenance (O&M) costs associated with unit synchronization to the IESO-controlled grid for a given start-up event (costs submitted via Online IESO). - Fuel and O&M costs associated with moving the generation unit from a valid start to its minimum loading point (costs submitted via Online IESO). | 9.4.7B | Same as IESO market rules. |
| CNPFm | Capacity Auction Non-Performance Factor | The non-performance factor as listed in Section 7.1 of Market Manual 12 that corresponds and applies to the month ‘m’ being settled. | N/A | Refer to Market Manual 5.5 |
| DA_BEk,hm,t | Energy Offer submitted into the schedule of record at a delivery point | Energy offers submitted in day-ahead, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at delivery point ‘m’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price-quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.1.2B.7 | Same as IESO market rules. |
| DA_BEk,hi,t | Energy Offer submitted into the schedule of record at a intertie metering point | Energy offers submitted in day-ahead, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price-quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.8A.2B and 9.3.8B.2 | Same as IESO market rules |
| DA_BLk,hi,t | Energy Bids submitted into the schedule of record | Energy bids submitted in day-ahead, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price-quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.1.2B.7 and 9.3.8D.2 | Same as IESO market rules |
| DA_CGC | Submitted Day-Ahead Combined Guaranteed Costs | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. A financial amount consisting of fuel cost components defined on a per-start basis for a given generation unit calculated in a manner consistent with the applicable market manual, and encompassing the following elements: - Fuel and operation and maintenance (O&M) costs associated with unit synchronization to the IESO-controlled grid for a given start-up event (costs submitted via IESO Gateway). - Fuel and O&M costs associated with moving the generation unit from a valid start to its minimum loading point (costs submitted via IESO Gateway). | 9.4.7D.1 | Same as IESO market rules |
| DA_DQSIk,hm,t | Schedule of Record Dispatch Quantity of Energy Scheduled for Injection at a delivery point | Day-ahead constrained quantity scheduled for injection by market participant ‘k’ at delivery point ‘m’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules. |
| DA_DQSIk,hi,t | Schedule of Record Dispatch Quantity of Energy Scheduled for Injection at an intertie metering point | Day-ahead constrained quantity scheduled for injection by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules. |
| DA_DQSWk,hi,t | Schedule of Record Dispatch Quantity of Energy Scheduled for Withdrawal | Day-ahead constrained quantity scheduled for withdrawal by market participant 'k' at intertie metering point 'i' during metering interval 't' of settlement hour 'h'. | 9.3.1.2A | Same as IESO market rules. |
| DA_ELMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the export zone | Day-ahead constrained schedule intertie price at the delivery point ‘m’ of the sink for the export transaction during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules. |
| DA_ILMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the import zone | Day-ahead constrained schedule intertie price at the delivery point ‘m’ of the source for the import transaction during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules. |
| DA_SNLCk,hm | Speed-no-load costs submitted into the schedule of record at a delivery point | As-offered speed-no-load cost associated with three-part offers for a given settlement hour ‘h’ for market participant ‘k’ at delivery point ‘m’. | 9.3.1.2B.7 | Same as IESO market rules. |
| DA_SNLCk,hp | Speed-no-load costs submitted into the schedule of record at a pseudo-unit | As-offered speed-no-load cost associated with three-part offers for a given settlement hour ‘h’ for market participant ‘k’ at pseudo-unit ‘p’. | 9.3.1.2B.7 | Same as IESO market rules. |
| DA_SUCk,hm | Start-up costs submitted into the schedule of record at a delivery point | As-offered start-up cost associated with three-part offers for a given settlement hour ‘h’ for market participant ‘k’ at delivery point ‘m’ where settlement hour ‘h’ is the initial hour in the DACP start event. | 9.3.1.2B.7 | Same as IESO market rules. |
| DA_SUCk,hp | Start-up costs submitted into the schedule of record at a pseudo-unit | As-offered start-up cost associated with three-part offers for a given settlement hour ‘h’ for market participant ‘k’ at pseudo-unit ‘p’ where settlement hour ‘h’ is the initial hour in the DACP start event. | 9.3.1.2B.7 | Same as IESO market rules. |
| DIPCk,hm,t | Derived Interval Price Curve | Energy price curves derived per interval from submitted hourly day-ahead PSU energy offers, represented as a N by 2 matrix of price-quantity pairs for each market participant ‘k’ at delivery point ‘m’ (where ‘m’ is a CT or ST delivery point) during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.1.11 | Same as IESO market rules. Refer to Market Manual 9.5, Appendix B for a detailed description of DIPC. |
| DIGQk,hm,t | Derived Interval Guaranteed Quantity | Portion of the day-ahead constrained quantity scheduled for injection that is eligible for DA-PCG for market participant ‘k’ at pseudo unit ‘p’ during metering interval ‘t’ of settlement hour ‘h’ | 9.3.1.11 | Same as IESO market rules. Refer to Market Manual 9.5, Appendix C for a detailed description of DIGQ. |
| DQSIk,hm,t | Dispatch Quantity of Energy Scheduled for Injection | Dispatch quantity of energy scheduled for injection in the real-time schedule by market participant ‘k’ at location ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 and 9.3.1.4A | Same as IESO market rules. N.B. Location m is further subject to the functional deferral described in Section 3.1.4A of Chapter 9 of the market rules (ref. 9.3.1.4A). |
| DQSRr,k,hm,t | Dispatch Quantity Schedule of Operating Reserve | Dispatch quantity schedule of class r reserve for market participant ‘k’ at location ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.4 and 9.3.1.4A | Same as IESO market rules. N.B. Location m is further subject to the functional deferral described in Section 3.1.4A of Chapter 9 of the market rules (ref. 9.3.1.4A). |
| DQSWk,hm,t | Dispatch Quantity of Energy Scheduled for Withdrawal | Dispatch quantity of energy scheduled for withdrawal in the real-time schedule by market participant ‘k’ at location ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 and 9.3.1.4A | Same as IESO market rules. N.B. Location m is further subject to the functional deferral described in Section 3.1.4A of Chapter 9 of the market rules (ref. 9.3.1.4A). |
| DRACP | Demand Response Auction Clearing Price | The demand response auction clearing price for the commitment period and zone. | N/A | Refer to Market Manual 5.5 |
| DRACPh | Hourly Demand Response Auction Clearing Price | The demand response auction clearing price for the commitment period and zone divided by the hours of availability for a day. | N/A | Refer to Market Manual 5.5 |
| DRBOCk | Demand Response Buy-Out Capacity | The buy-out capacity is an amount that is being reduced from the demand response capacity obligation for demand response market participant ‘k’. | N/A | Refer to Market Manual 5.5 |
| DRCOk | Demand Response Capacity Obligation (MW) | The demand response capacity obligation amount for the commitment period and zone for demand response market participant ‘k’. The initial capacity obligation is acquired through the demand response auction and subject to being reduced via the buy-out process. | N/A | Refer to Market Manual 5.5 |
| DREBQk | Demand Response Energy Bid Quantity | The demand response energy bid quantity calculated for demand response market participant ‘k’ as the sum of the quantity of demand response capacity provided by all participating demand response resources. | N/A | Refer to Market Manual 5.5 |
| DRNPF | Demand Response Non-Performance Factor | The non-performance factor as listed in Section 7.1 of Market Manual 12 that corresponds and applies to the month being settled. | N/A | Refer to Market Manual 5.5 |
| DRSQty | Demand Response Scheduled Quantity | Calculated as (Total Bid Qty – Schedule) where ‘Total Bid Qty’ is the maximum quantity of the demand response energy bid and where ‘Schedule’ is the real-time constrained schedule quantity. | N/A | Refer to Market Manual 5.5 |
| EEQ | Excluded Energy Quantity | The total volume of energy (MWh) supplied to Fort Frances Power Corporation Distribution Inc. by Abitibi-Consolidated Inc. during the month. | N/A – subject to regulations made pursuant to Bill 100. | N/A – See regulations. |
| EGEIk | Embedded Generator Energy Injection | The total volume of energy (MWh) supplied by embedded generators during the month to distributors who are market participants and to all embedded distributors to whom the market participant ‘k’ is the host distributor, adjusted for losses as required by the OEB, Retail Settlement Code. | N/A – subject to regulations made pursuant to Bill 100. | N/A – See regulations. |
| EIMk,h | Operating Profit Function for the IMPORT of Energy under the Intertie Offer/Bid Guarantee Settlement Credit | This Operating Profit function is used for the calculation of the Intertie Offer/Bid Guarantee Settlement Credit (IOBG) with respect the IMPORT of energy. | 9.3.8A | EIMk,h IS NOT A VARIABLE EIMk,h is the output of a particular usage of the Operating Profit (OP) function defined within Chapter 9, Section 3.8A. EIMk,h Input variables into the Operating Profit (OP) Function include: MQSI, EMP, and BE. |
| EMPhi,t | 5-minute Energy Market Price at the Interties | Energy market price applicable to intertie metering point ‘i’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 | Same as IESO market rules. |
| EMPhm,t | 5-minute Energy Market Price within Ontario | Energy market price applicable to RWM ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 | Same as IESO market rules. |
| EMPhREF,t | 5-minute Energy Market Reference Price | Reference energy market price used to value losses in the calculation of the Transmission Charge Reduction Fund during in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 and 9.3.6.2 | Same as IESO market rules. |
| ETS | Export Transmission Service Tariff Rate | Export Transmission Service Tariff Rate in units of $/MWh. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| FPhm | Fixed Energy Rate | A fixed energy rate for all metering intervals in settlement hour ‘h’. | N/A – subject to regulations made pursuant to Ontario Energy Board Act, 1998 until March 31, 2005 and by the OEB under such regulations commencing April 1, 2005. | N/A – See regulations. |
| FPChm | Rate for a designated group of charge types (see description of charge type 141) | This variable is reserved for charge type 141 and applies with respect to charges for the period commencing December 1, 2002 and ending March 31, 2005. See Ontario Regulation 436/02 and Ontario Regulation 98/05. | N/A – subject to regulations made pursuant to Ontario Energy Board Act, 1998. | N/A – See regulations |
| GA_AQEWg,k,h,Mm,t | Allocated Quantity of Energy Withdrawn for elements of the Global Adjustment distribution | Allocated quantity in MWh of energy withdrawn by market participant or Distributor ‘k’ at RWM ‘m’ in metering interval ‘t’ of settlement hour ‘h’ in month ‘M’ for element “g” Where ‘g’ is 1 for Class A Market Participant or Consumer load, and 2 for energy withdrawn by Generator ‘k’ in the course of providing Ancillary Services. |  |  |
| GARB | Global Adjustment Rate for Class B | GA Class B Rate. | N/A | N/A – See regulations. |
| GRP | Generator Regulated Price | A regulated price ($/MWh) with respect to output of OPG’s regulated generating stations, set by the OEB. | N/A – subject to regulation by the Ontario Energy Board. | N/A – See regulations |
| HDRDCh | Measured hourly demand response capacity | Min (Min (Total Bid Qty, Resource Capability, Capacity Obligation) – Schedule, Delivered Capacity) Where Delivered Capacity: For C&I HDR resources calculated as: - Max (0, HDR Baselineh – Actual consumptionh) For residential HDR resources calculated as: - Max (0, No. of contributors in Treatment Groupm X (Adjusted Control Group Loadh – Treatment Group Loadh)) Where h is an hour within the activation window and m is the month of activation, and Total Bid Qty’ is the maximum quantity of the demand response energy bid, ‘Schedule’ is the real-time constrained schedule quantity, and Resource Capability is the HDR resource’s registered capability. | Chapter 9: Section 4.7J.5 | Refer to Market Manual 5.5, Section 1.6.26.2A |
| HDRBPh | HDR Bid Price | The price from real-time DR energy bid submitted by an HDR resource Where h is an hour within the activation window. | Chapter 9: Section 4.7J.5 | Refer to Market Manual 5.5 |
| HDRTAPR | Out of Market Test Activation Payment Rate | $250 per MWh. | Chapter 9: Section 4.7J.5 | Refer to Market Manual 5.5 |
| HOEPh | Hourly Ontario Energy Price | Hourly Ontario Energy Price in settlement hour ‘h’. | 9.3.1.3 | Same as IESO market rules. |
| IOG_FVk,hi | IOG Floor Value | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. The IOG_FVk,hi is a floor value (in dollars to the nearest cent) derived from: - The day-ahead offer prices for the import transaction submitted by the market participant over the range of the pre-dispatch of record constrained quantity scheduled for that import transaction; and - Real-time offer prices for the import transaction at the corresponding location in the corresponding settlement hour for any additional energy scheduled above and beyond the pre-dispatch of record constrained quantity scheduled for that import transaction: NOTE: The IOG_FVk,hi is formulated in the manner described in Chapter 9, Section 3.8A.8 of the IESO market rules and is used in the formulation of the intertie offer guarantee adjustment (see also, Section 2.2 entry for charge type 1137 within this document). | 9.3.8A.8 | Same as IESO market rules See Chapter 9, Section 3.8A.8 for details concerning its formulation. |
| LCDk,hm | Line Connection Demand (KW) | Billing Demand for Line Connection Transmission Service (KW) for transmission customer ‘k’ at transmission delivery point ‘m’ during settlement hour ‘h’ in which LCDk,hm denotes the non-coincident peak demand for the month. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| MDCAA | Monthly deferred Class A amount to be recovered | The monthly deferred Class A amount to be recovered which equals one twelfth of the total Global Adjustment allocated to Class A customers that was deferred in April, May and June of 2020. | N/A | N/A – See regulations. |
| MDCBA | Monthly deferred Class B amount to be recovered | The monthly deferred Class B amount to be recovered equals one twelfth of the total Global Adjustment allocated to Class B customers that was deferred in April, May and June of 2020. | N/A | N/A – See regulations. |
| MChm | Minimum Consumption | Calculation of the self-induced dispatchable load CMSC clawback under Business Rule 2. The minimum consumption is equal to the quantity in the price quantity pair where the bidding price is MMCP (i.e., $2000) at RWM metering point ‘m’ for settlement hour ‘h’. | 9.3.5.1A |  |
| MI | Ordered matrix of MQSIk,hi,t and corresponding IOG settlement amounts | Used for the calculation of the IOG OFFSET settlement amount. A matrix of X pairs of market schedule quantities scheduled for injection by market participant ‘k’ at all intertie metering points ‘i’ in metering interval ‘t’ of settlement hour ‘h’ (MQSIk,hi,t) paired with the corresponding component of the intertie offer guarantee settlement credit for each intertie metering point ‘i’. See equation in Chapter 9, Section 3.8A.4 of the IESO market rules for further details. | 9.3.8A.4 | Same as IESO market rules. |
| MLPk,hm,t | Minimum Loading Point | Minimum output of energy the market participant ‘k’ at delivery point ‘m’ can maintain without ignition support in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2B.7 | Same as IESO market rules. |
| MLP_CONSk,hm,t | Minimum Loading Point for a steam turbine resource or a combustion turbine resource associated to a pseudo unit | Minimum output of energy the market participant ‘k’ at delivery point ‘m’ can maintain without ignition support in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2B.7 | Same as IESO market rules. Refer to Market Manual 9.4, Section 4.1.2.2 for a detailed description of constraints applied for PCG eligible combined cycle plants. |
| MQSIk,hm,t | Market Quantity Scheduled for Injection | Market quantity scheduled for injection in the market schedule by market participant ‘k’ at RWM or intertie metering point ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 | Same as IESO market rules. |
| MQSI{adj}k,hm,t | Adjusted Market Quantity Scheduled for Injection | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. Used for the calculation of the IOG OFFSET settlement amount. MQSI{adj}k,hi,t is each (and where applicable, adjusted) quantity of energy scheduled for injection in the market schedule by market participant ‘k’ at an intertie metering point ‘i’ in metering interval ‘t’ of settlement hour ‘h’ corresponding with each quantity, MQSIx*,k,hi,t in matrix MI, row x*. | 9.3.8A.4 | Same as IESO market rules. |
| MQSWk,hm,t | Market Quantity Scheduled for Withdrawal | Market quantity scheduled for withdrawal in the market schedule by market participant ‘k’ at RWM or intertie metering point ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.3 | Same as IESO market rules. |
| NSDk,hm | Network Service Demand (KW) | The Billing Demand for Network Transmission Service (KW) is defined as the higher of: transmission customer coincident peak demand (KW) in the hour of the month when the total hourly demand of all PTS customers is highest for the month; and 85% of the customer peak demand in any hour during the peak period 7 AM to 7 PM (local time) on business days defined by the IESO. For the purposes of determining business days for calculating transmission charges, the IESO uses the holidays identified by the Ontario Energy Board. The peak period hours will be between 0700 and 1900 hours Eastern Standard Time during winter (i.e. during standard time) and 0600 to 1800 hours during summer (i.e. during daylight savings time), in conformance with the meter time standard used by the IESO settlement systems. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| OCMWk | Over committed MWs | Represent the over committed MWs of a generator-backed capacity import recource when it is determined that any portion of the MWs offered into the auction are committed to another jurisdiction for the duration of the settlement month of the obligation period. |  |  |
| ONPAO | Ontario Power Generation Non-Prescribed Assets Output | OPG’s Non-Prescribed Assets are those generation assets operated and controlled by Ontario Power Generation in service as of January 1, 2006, excluding Lennox Generating Station, that are not prescribed assets under section 78.1 of the Ontario Energy Board Act, 1998 as amended by the “Electricity Restructuring Act, 2004”. ONPAO refers to the generation output from OPG’s Non-Prescribed Assets, over each hour of the quarter adjusted to take account of volumes sold through forward contracts in effect as of January 1, 2005. For greater certainty, any output from ONPA resulting from fuel conversion by Ontario Power Generation in ONPA, or incremental output from ONPA resulting from refurbishment or expansion is to be excluded from ONPAO. Incremental Output is defined as: generation output x (new total installed capacity – installed capacity as of January 1, 2006) / new total installed capacity. | N/A | The formula for calculating the OPG Rebate is subject to Ministerial Directive made under Order-in-Council 1062/2006 (May 17, 2006). |
| OP | Operating Profit Function | The Operating Profit function is used for the calculation of the Congestion Management Settlement Credit (CMSC) with respect to constrained on/off payments for energy, operating reserve. It is also used for the calculation of the Day-Ahead Production Cost Guarantee components, the Day-Ahead Generator Withdrawal Charge, the Day-Ahead Import and Export failure charges, and the Import Offer Guarantee Settlement Credit. | 9.3.5.2 and 9.3.8A.2 | OP IS NOT A VARIABLE OP is a mathematical function defined within Chapter 9, section 3.5.2. of the IESO market rules Input variables include: MQSI, MQSW, SQROR AQEI, AQEW, AQOR SQEI, SQEW, DSQI, DSQW, DSQR DA_DQSI, DA_DQSW, PD_DQSI, PD_DQSW BE, BL, BRr PD_BE, PD_BL DA_BE, DA_BL EMP MLP, MLP CONS DIPC OPCAP OP is also used within Chapter 9, Section 9.8A.2 of the IESO market rules to derive the Energy Import (EIMk,h) sub-component of the Intertie Offer Settlement Credit (IOG) using the following input variables: MQSI BE EMP |
| OPCAPk,hm,t | Operating Capacity | De-rating of the generation unit by market participant ‘k’ at delivery point ‘m’ in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2B.7 | Same as IESO market rules. |
| OPE{adj}k,hi | Adjusted CMSC component for energy used in the DA-Ahead IOG | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. This congestion management settlement credit settlement amount (CMSC) component is specifically used in the calculation of the Day-Ahead IOG for import transactions that are subject to a constrained-on event in the real-time market. OPE{adj}k,hi is an adjusted component of The congestion management settlement credit settlement amount (CMSC) for market participant ‘k’ at intertie metering point ‘i’ for settlement hour ‘h’ in which the constrained schedule is the lesser of PDR_DQSIk,hi,t or DQSIk,hi,t but in all instances, greater than or equal to MQSIk,hi,t . | 9.3.8A.2A | ‘OP’ is a mathematical function used within Chapter 9, Section 9.3.8A.2A of the IESO market rules to derive Day-Ahead Intertie Offer Guarantee. Please see the market rules for information regarding its formulation. |
| ORL | Ontario Power Generation Revenue Limit | For the period May 1, 2006 to April 30, 2007 ORL is equal to $46/ MWh. For the period May 1, 2007 to April 30, 2008 ORL is equal to $47/ MWh. For the period May 1, 2008 to April 30, 2009 ORL is equal to $48/ MWh. | N/A | The formula for calculating the OPG Rebate is subject to Ministerial Directive made under Order-in-Council 1062/2006 (May 17, 2006). |
| PAA | Pilot Auction Amount | Refers to the Pilot Auction administered by the Ontario Power Authority in the first half of 2006. The volume in MWh over each hour in the quarter that is sold by Ontario Power Generation through the PA. | N/A | The formula for calculating the OPG Rebate is subject to Ministerial Directive made under Order-in-Council 1062/2006 (May 17, 2006). |
| PAORL | Pilot Auction Ontario Power Generation Revenue Limit | For the period May 1, 2006 to April 30, 2007 PAORL is equal to $51/ MWh. For the period May 1, 2007 to April 30, 2008 PAORL is equal to $52/ MWh. For the period May 1, 2008 to April 30, 2009 PAORL is equal to $53/ MWh. | N/A | The formula for calculating the OPG Rebate is subject to Ministerial Directive made under Order-in-Council 1062/2006 (May 17, 2006). |
| PAP | Pilot Auction Price | The weighted average auction price in $/ MWh over each hour of the quarter realized for the PAA by Ontario Power Generation. | N/A | The formula for calculating the OPG Rebate is subject to Ministerial Directive made under Order-in-Council 1062/2006 (May 17, 2006). |
| PB_IMht | Price bias adjustment factor for import transactions | Price bias adjustment factor for import transactions in effect during metering interval ‘t’ of settlement hour ‘h’. | 9.3.8C.3 | Same as IESO market rules |
| PB_EXht | Price bias adjustment factor for export transactions | Price bias adjustment factor for export transactions in effect during metering interval ‘t’ of settlement hour ‘h’. | 9.3.8C.5 | Same as IESO market rules |
| PD_BEk,hi,t | Energy Offer submitted into the Pre-dispatch | Energy offers submitted in Pre-dispatch, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.1.2D | Same as IESO market rules. |
| PD_BLk,hi,t | Energy Bid submitted into the Pre-dispatch | Energy bids submitted in pre-dispatch, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2. | 9.3.1.2D | Same as IESO market rules. |
| PD_DQSIk,hi,t | Pre-dispatch quantity scheduled for injection at an intertie metering point | Pre- dispatch constrained quantity scheduled for injection by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2C | Same as IESO market rules |
| PD_DQSWk,hi,t | Pre-dispatch quantity scheduled for withdrawal at an intertie metering point | Pre- dispatch constrained quantity scheduled for withdrawal by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2C | Same as IESO market rules |
| PD_ELMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the export zone | Pre-dispatch constrained schedule intertie price at the delivery point ‘m ’ of the sink for the export transaction during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2C | Same as IESO market rules. |
| PD_EMPhm,t | Pre-dispatch energy market price for Ontario | Pre-dispatch projected energy market price applicable to all delivery points ‘m’ in the Ontario zone in metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2C | Same as IESO market rules |
| PD_ILMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the import zone | Pre-dispatch constrained schedule intertie price at the delivery point ‘m’ of the source for the import transaction during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2C | Same as IESO market rules. |
| PDFk,m,d | Peak Demand Factor | The Peak Demand Factor for Class A Market Participant or Distributor ‘k’ for month ‘m’ with effectiveness ratio ‘d’. | N/A – subject to regulation by the Ontario Energy Board | N/A – See regulations. |
| PDR_BEk,hi,t | Energy Offer submitted into the pre-dispatch of record | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. Energy offers submitted into the pre-dispatch of record, represented as an n by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price-quantity pair, where offered prices are in column 1 and offered quantities are in column 2. | 9.3.1.2B | Same as IESO market rules |
| PDR_DQSIk,hi,t | Pre-dispatch of record dispatch quantity scheduled for injection at an intertie metering point | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. Pre-dispatch of record constrained quantity scheduled for injection by market participant ‘k’ for an import transaction at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules |
| PDR_DQSIk,hm,t | Pre-dispatch of record dispatch quantity scheduled for injection at a delivery point | EFFECTIVE OCTOBER 13, 2011, THIS VARIABLE IS NO LONGER USED IN THE CALCULATION OF ANY SETTLEMENT. Pre-dispatch of record constrained quantity scheduled for injection by market participant ‘k’ at delivery point ‘m’ during metering interval ‘t’ of settlement hour ‘h’. | 9.3.1.2A | Same as IESO market rules |
| PGSh,M | Allocated Quantity of Energy Withdrawn by OPG at Beck Pump Generating Station | Allocated quantity in MWh of energy withdrawn by OPG at Beck Pump Generating Station in metering interval ‘t’ of settlement hour ‘h’ for month ‘M’. |  |  |
| PRORr,hm,t | 5-minute Operating Reserve Price | Market price in $/MW of class r reserve in metering interval ‘t’ of settlement hour ‘h’ at RWM ‘m’ or intertie metering point ‘m’. | 9.3.1.4 | Same as IESO market rules. |
| PSTk,hp,t | Steam turbine portion from Daily Generator Data | The percentage of the pseudo-unit’s schedule that relates to the steam turbine in association with offer k for market participant ‘k’ at pseudo unit ‘p’ during metering interval ‘t’ of settlement hour ‘h’. | 7.2.2.2 | Same as IESO market rules. |
| PTS-L | Provincial Transmission Service Line Connection Service Rate ($/KW) | Line Connection Transmission Tariff Service Rate in units of dollars per kilowatt. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| PTS-N | Provincial Transmission Service Network Service Rate ($/KW) | Network Transmission Tariff Service Rate in units of dollars per kilowatt. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| PTS-T | Provincial Transmission Service Transformation Connection Service Rate ($/KW) | Transformation Connection Service Transmission Tariff Rate in units of dollars per kilowatt. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| QTRk,hi,j | Quantity of Transmission Rights Owned | Quantity of TRs in MW assigned to market participant ‘k’ for transmission from injection TR zone ‘i’ to withdrawal TR zone ‘j’. | 9.3.1.8 and 8.4.2 | Same as IESO market rules. |
| RACk | Resource Available Capacity (MW) | Available capacity, in MW, per capacity auction resource for capacity market participant ‘k’, represented by: Min (DREBQh,1.15*CCOk, CICAPk, HDR resource’s registered capability) | N/A | Refer to Market Manual 5.5 |
| RPPl | Regulated Price Plan | A fixed energy rate for all metering intervals based on consumption level l. | N/A – subject to regulation by the Ontario Energy Board | N/A – See regulations. |
| RPPVAk | Total volume of electricity distributed to prescribed Class B consumers | The total volume of electricity distributed to Class B consumers whose rates are determined under subsection 79.16 (1) of the Ontario Energy Board Act, 1998 during the month by licensed distributor ‘k’. | N/A | N/A – See regulations. |
| RQ | Reallocate Quantity | A quantity derived from a physical bilateral contract quantity (BCQk,b,hm,t or BCQs,k,hm,t) in order to reallocate a component of hourly uplift from the buying market participant to the selling market participant in direct proportion to the size of the physical bilateral contract. | N/A | See hourly uplift charge types in Section 2.2 |
| SQEIk,hi,t | Scheduled Quantity of Energy Injected at an intertie metering point | Scheduled quantity in MWh of energy injected by market participant ‘k’ at intertie metering point ‘i’ for each metering interval ‘t’ in settlement hour ‘h’. | 9.3.1.9 | This variable is a sub-set of variable AQEI described in Section 3.1.9 of Chapter 9 of the market rules, specifically referring to those quantities derived from interchange schedule data. |
| SQEWk,hi,t | Scheduled Quantity of Energy Withdrawn at an intertie metering point | Scheduled quantity in MWh of energy withdrawn by market participant ‘k’ at intertie metering point ‘i’ for each metering interval ‘t’ in settlement hour ‘h’. | 9.3.1.9 | This variable is a subset of variable AQEW described in Section 3.1.9 of Chapter 9 of the market rules, specifically referring to those quantities derived from interchange schedule data. |
| SQRORr,k,hm,t | Scheduled Quantity of class r Operating Reserve | Market Schedule quantity in MW of class r reserve for market participant ‘k’ in metering interval ‘t’ of settlement hour ‘h’ at RWM ‘m’. | 9.3.1.4 | Same as IESO market rules. |
| TCDk,hm | Transformation Connection Demand (KW) | Billing Demand for Transformation Connection Transmission Service (KW) for transmission customer ‘k’ at transmission delivery point m during settlement hour ‘h’ in which TCDk,hm denotes the non-coincident peak demand for the month. | N/A | Subject to the OEB “Ontario Transmission Rate Order”. |
| TDk,h,c | Total Market Settlement Amount | Total settlement amount (dollars) for the market used in hourly uplift and calculations for various other non-hourly settlement amounts for market participant ‘k’ or transmission customer ‘k’ during settlement hour ‘h’ with respect to charge type ‘c’. | N/A | This is purely a notational term is used within the documentation to describe the aggregation of various settlement amounts. A summation across charge type ‘c’ denotes an aggregation of all settlement amounts for that charge type for the time period concerned. e.g.: cT indicates a summation of all settlement amounts for charge type ‘c’ during all metering intervals ‘T’. |
| TLQ | Threshold Load Quantity | A threshold (kWh) with respect to monthly consumption of regulated customers, set by the OEB. | N/A – subject to regulation by the Ontario Energy Board. | N/A – See regulations. |
| TPc | Tariff price | A stipulated rate ($/MWh, $/KW) used in the calculation of a specific charge type ‘c’. | N/A | This is purely a notational term is used within the documentation to describe the unique per MW or per MWh rate applied to specific quantities in order to calculate various settlement amounts. |
| TRMP | TR Market Clearing Price | The price of each transmission right in a single round of a TR auction. | 8.4.15 | Same as IESO market rules. |
| TRCAD | TR Clearing Account Disbursements | The total dollar value of all disbursements from the TR clearing account authorized by the IESO Board in the current energy market billing period. | 9.4.7.2 | Same as IESO market rules. |
| TRCADE | TR Clearing Account Disbursements for Exporters | The proportion of the total dollar value of all disbursements from the TR clearing account authorized by the IESO Board in the current energy market billing period allocated to exporters. | 9.4.7.2 | Same as IESO market rules. |
| TRCADL | TR Clearing Account Disbursements for Loads | The proportion of the total dollar value of all disbursements from the TR clearing account authorized by the IESO Board in the current energy market billing period allocated to loads. | 9.4.7.2 | Same as IESO market rules. |
| TRCAR | TR Shortfall Recovery Amount | The total dollar value of TR shortfall recovery from the TR clearing account authorized by the IESO Board in the current energy market billing period. | 9.4.7.2 | Same as IESO market rules. |
| Uk | Energy Storage Facility Injection | The total volume of energy (MWh) conveyed back into the IESO-controlled grid during the month by energy storage facilities associated with Class B market participant ‘k’ and the total volume of energy (MWh) conveyed back into the distribution system during the month by energy storage facilities that are Class B consumers of distributor ‘k’. | N/A | N/A – See regulations. |
| Xhm,t | Settlement Floor Price for exports | A settlement floor price for energy applicable to intertie metering point ‘m’ metering interval ‘t’ in settlement hour ‘h’ as set in the applicable market manual. The need for a settlement floor price other than MMCP shall remain in effect only until floor prices for energy offers from registered market participants that are variable generators or nuclear generators go into effect. | 9.3.1.3 | Same as IESO market rules |

## Charge Types and Equations

The following table contains the IESO charge types and equations that are part of an active IESO-administered market.

Notice to Electricity Storage Participants – As of January 2021, substantial amendments to the market rules came into effect allowing for increased participation of electricity storage participants and electricity storage facilities in the IESO-administered markets and on the IESO-controlled grid.  However, the IESO does not anticipate updating the charge types and equations set out in this Section 2.2, the variable descriptions set out in Section 2.1 above, or any other potentially affected parts of this document to reflect those market rule amendments until the IESO’s commercial reconciliation tools shall have been updated.  Therefore, until such time, settlement programs, variable descriptions, charge types and equations will be applied to electricity storage participants and their electricity storage facilities as follows:

- the relevant provisions of Market Manual 5: Settlements, Part 5.5: Physical Markets Settlement Statements and all other relevant  market manuals have been updated to reflect the aforementioned market rule amendments with respect to settlements of transactions and other circumstances relating to electricity storage participants and electricity storage facilities, and;

- based on these updated provisions, the variable descriptions, charge types and equations set out in this document will, as appropriate, be applied to the settlement of all relevant transactions and other circumstances, subject to the making of any alterations to such variable descriptions, charge types and equations as may be necessary to properly apply them in respect of each such transaction or other circumstance.

| Key to the Table Below |  |
| --- | --- |
| Charge Type Number | The designation number for each charge type enumerated below – which correspond to the charge type numbers used on settlement statements and invoices. |
| Charge Type Name | The name of the charge type. |
| Settlement Amount Acronym | The abbreviated name of the variable used to describe the settlement amount within the IESO market rules. |
| Market Rules Reference | The relevant reference to the variable in question within the IESO market rules. The format for each reference is: [Chapter] [Section number] For example: “Chapter 9 Section 3.1.6” would appear as: 9.3.1.6 |
| Equation | The equation used by the IESO settlements process to calculate the settlement amount related to each charge type. |
| Settlement Resolution | The level of granularity by which the IESO settlements process calculates the settlement amount (for which the charge type is related), and provides the supporting data in the settlement data file. Where: “Interval” means that the calculations are performed on the basis of each relevant, 5-minute metering interval; “Hourly” means that the calculations are performed on the basis of each settlement hour; “Daily” means that the calculations are performed on the basis of each calendar day; “Monthly” means that the calculations are performed on the basis of a calendar month (equivalent to a real-time market billing period); “Quarterly” means that the calculations are performed on the basis of 3 month intervals; “Yearly” means that the calculations are performed on the basis of a calendar year. |
| Cashflow | This column indicates whether or not the settlement amount (for which the charge type is related) is: “Due IESO” – which means, owed to the IESO by the market participant; *** or “Due MP” – which means, owed to the market participant by the IESO; *** or “Either Way” – which indicates that the settlement amount in question could be either owed to the IESO by the market participant or owed to the market participant by the IESO in any given time period (according to the applicable “settlements resolution”). ***NOTE in cases where a Cashflow is designated as “Due IESO” or “Due MP” this should be read in the context of its intended use in the normal course of settlements. However, such cashflows can always be REVERSED in situations where an adjustment is applied to a market participant, or the application of a per-unit charge in order to offset an adjustment to another market participant. |
| HST Tax Treatment within Ontario | This column indicates the percentage levy as per the Harmonized Sales Tax (HST). Zone used for Tax Basis is (ONZN) for Ontario. The applicable Zone ID may be found in column 7 of the applicable settlement statement detail record (see also, the Technical Interface Document entitled, “Detail Field Description”). A complete list of Zones may be found in the Technical Interface Document entitled, “Standing Data”. |
| HST Tax Treatment for U.S., Manitoba and Quebec Generation | This column indicates the percentage levy as per the Harmonized Sales Tax (HST). Zones used for Tax Basis are (NYSI) for US Generation, (MBSI) for Manitoba Generation and (PQSI) for Quebec Generation. The applicable Zone ID may be found in column 7 of the applicable settlement statement detail record (see also, the Technical Interface Document entitled, “Detail Field Description”). A complete list of Zones may be found in the Technical Interface Document entitled, “Standing Data”. |
| HST Tax Treatment for US Load | This column indicates the percentage levy as per the Harmonized Sales Tax (HST). Zone used for Tax Basis is (NYSI) for US Load. The applicable Zone ID may be found in column 7 of the applicable settlement statement detail record (see also, the Technical Interface Document entitled, “Detail Field Description”). A complete list of Zones may be found in the Technical Interface Document entitled, “Standing Data”. |
| HST Tax Treatment for Manitoba and Quebec Load | This column indicates the percentage levy as per the Harmonized Sales Tax (HST). Zones used for Tax Basis are (MBSI) for Manitoba Load and (PQSI) for Quebec Load. The applicable Zone ID may be found in column 7 of the applicable settlement statement detail record (see also, the Technical Interface Document entitled, “Detail Field Description”). A complete list of Zones may be found in the Technical Interface Document entitled, “Standing Data”. |
| Effective Start Trade Date | This column indicates the settlement effective start trade date of the charge type. |
| Effective End Trade Date | - This column indicates the settlement effective end trade date of the charge type. |
| Comments | This column notes any charge types that are governed by various documentation other than the IESO market rules. |

| Charge Type Number | Charge Type Name |  | Settlement Amount Acronym |  | Market Rules Reference |  | Equation | Settlement Resolution | Cashflow (See Note at Beginning of this Section) | HST Tax Treatment within Ontario (%) | HST Tax Treatment for U.S., Manitoba, and Quebec Generation (%) | HST Tax Treatment for U.S. Load (%) | HST Tax Treatment for Manitoba and Quebec Load (%) | Effective Start Trade Date | Effective End Trade Date | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Financial Market Charge Types |  |  |  |  |  |  |  |  |  |  |
| 52 | Transmission Rights Auction Settlement Debit |  | N/A |  | 8.4.17 |  |  | Daily | Due IESO | Exempt | Exempt | Exempt | Exempt |  |  |  |
|  |  |  |  |  |  | Physical Market Charge Types |  |  |  |  |  |  |  |  |  |  |
| 100 | Net Energy Market Settlement for Generators and Dispatchable Load |  | NEMSCk,h |  | 9.3.3.2 |  | For dispatchable facilities or an intertie metering point associated with: - An injecting boundary entity; - A withdrawing boundary entity where the associated intertie congestion price is less than zero; - A withdrawing boundary entity conducting a wheeling through transaction that is linked as per Chapter 7, section 3.5.82 of the market rules | Interval | Either Way | 13 | 13 | 0 | 13 |  |  |  |
| 101 | Net Energy Market Settlement for Non-dispatchable Load |  | NEMSCk,h |  | 9.3 |  |  | Hourly | Either Way | 13 | N/A | N/A | N/A |  |  |  |
| 102 | TR Clearing Account Credit |  | TRCACk |  | 9.4.7.2 |  | For loads: TRCACk = TRCADL x H M,T [(AQEWk,hm,t) / K,H M,T (AQEWk,hm,t)] For exporters: TRCACk = TRCADE x H I,T [(SQEWk,hi,t) / K,H I,T (SQEWk,hi,t)] Where TRCADL =(  KTDC / KTDC,C1 ) x TRCAD TRCADE = ( KTDC1 / KTDC,C1 ) x TRCAD Where ‘C’ is the set of all monthly service charge types c as follows: 650,651,652. Where ‘C1’ is the set of all monthly export transmission charge types c as follows:653. Where ‘H’ is the set of all settlement hours ‘h’ in the billing periods immediately preceding the current billing period, as determined by IESO Board. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. Where ‘M’ is the set of all delivery points ‘m’, excluding any intertie metering points. Where ‘I’ is the set of all intertie metering points ‘i’. Where ‘K’ is the set of all market participants ‘k’. | Monthly (when applicable) | Due MP | 13 | N/A | 0 | 13 |  |  | The billing period is defined in Market Manual 5.5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.27 |
| 103 | Transmission Charge Reduction Fund |  | TCRFh |  | 9.3.6.2 and 8.4.18 |  |  | Hourly | Accumulates in the TR Clearing Account | N/A | N/A | N/A | N/A |  |  | See IESO market rules, Chapter 8 Section 4.18 for further details. |
| 104 | Transmission Rights Settlement Credit |  | TRSCk,h |  | 9.3.6.1 |  |  | Hourly | Due MP | 0 | 0 | 0 | 0 |  |  |  |
| 105 | Congestion Management Settlement Credit for Energy |  | CMSCk,h |  | 9.3.5.2 to 9.3.5.7 |  | OP(EMPhm,t, MQSIk,hm,t, BE) – MAX(OP(EMPhm,t, DQSIk,hm,t, BE), OP(EMPhm,t,AQEIk,hm,t, BE)) Subject to the mathematical sign of (DQSI-MQSI) being equal to the mathematical sign of (AQEI-MQSI). AQEIk,hm,t and EMPhm,t may be substituted with SQEIk,h i,t and EMPhi,t respectively, where the application of this equation pertains to intertie metering point ‘i’. or -1OP(EMPhm,t, MQSWk,hm,t, BL) – MAX(-1OP(EMPhm,t, DQSWk,hm,t, BL),-1OP(EMPhm,t,AQEWk,hm,t, BL)) Subject to the mathematical sign of (DQSW-MQSW) being equal to the mathematical sign of (AQEW-MQSW). AQEWk,hm,t and EMPhm,t may be substituted with SQEWk,h i,t and EMPhi,t respectively, where the application of this equation pertains to intertie metering point ‘i’. or For variable generators that are registered market participants whose registered facility is operating under a release notification for any given dispatch interval, and the facility’s market schedule quantity is less than the corresponding quantity in the constrained schedule for the same dispatch interval as a result of the market participant’s offers being partially or fully uneconomic: OPE(EMPhm,t, MQSIk,hm,t, BE) -OP(EMPhm,t,AQEIk,hm,t, BE) See 9.3.5.2 for the definition of the Operating Profit (OP) function referenced above. | Interval | Either Way | 13 | 13 | 13 | 13 |  |  | This charge type holds the market participant to the expected profits implied by the market schedule derived on dispatch data provided by that market participant. Offer prices in matrix ‘BE’ may be revised down to a lower limit as described in 9.3.5.6. See also: description of variable ‘BE’ in Section 2.2. The bid prices in the matrix BL may be revised as described in Market Manual 5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.8. |
| 106 | Congestion Management Settlement Credit for 10 Minute Spinning Reserve |  | CMSCr,k,h |  | 9.3.5.2 |  | OP(PRORr,hm,t,SQRORr,k,hm,t, BRr) – MAX(OP(PRORr,hm,t, DQSRr,k,hm,t, BRr),OP(PRORr,hm,t, AQORr,k,hm,t, BRr)) See 9.3.5.2 for the definition of the Operating Profit (OP) function referenced above. | Interval | Either Way | 13 | N/A | N/A | N/A |  |  | This charge type holds the market participant to the expected profits implied by the market schedule derived on dispatch data provided by that market participant. |
| 107 | Congestion Management Settlement Credit for 10 Minute Non-spinning Reserve |  | CMSCr,k,h |  | 9.3.5.2 |  | OP(PRORr,hm,t,SQRORr,k,hm,t, BRr) – MAX(OP(PRORr,hm,t, DQSRr,k,hm,t, BRr),OP(PRORr,hm,t, AQORr,k,hm,t, BRr)) See 9.3.5.2 for the definition of the Operating Profit (OP) function referenced above. | Interval | Either Way | 13 | N/A | N/A | N/A |  |  | This charge type holds the market participant to the expected profits implied by the market schedule derived on dispatch data provided by that market participant. |
| 108 | Congestion Management Settlement Credit for 30 Minute Operating Reserve |  | CMSCr,k,h |  | 9.3.5.2 |  | OP(PRORr,hm,t,SQRORr,k,hm,t, BRr) – MAX(OP(PRORr,hm,t, DQSRr,k,hm,t, BRr),OP(PRORr,hm,t, AQORr,k,hm,t, BRr)) See 9.3.5.2 for the definition of the Operating Profit (OP) function referenced above. | Interval | Either way | 13 | N/A | N/A | N/A |  |  | This charge type holds the market participant to the expected profits implied by the market schedule derived on dispatch data provided by that market participant. |
| 111 | Northern Pulp and Paper Mill Electricity Transition Program Settlement Amount |  | N/A |  | N/A |  | =  M HT (AQEWmht) x (Tprate) Where: Tprate is the transition program rate ‘M’ is the set of all delivery points ‘m’ for all market participant-eligible facilities. ‘H’ is the set of all settlement hours ‘h’ in the settlement period. ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours 5 H’. ‘AQEW’ is limited to a maximum of 1,000,000 MWh annually per eligible market participant. | Quarterly | Due MP | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to Ministry of Natural Resources specifications. This program ends on September 30, 2010. |
| 112 | Ontario Power Generation Rebate |  | N/A |  | N/A |  | ** CALCULATIONS FOR CHARGE TYPE 112 END April 30, 2009 ** = TD162 x [(AQEWk,ht) / K,HT (AQEWk,ht)] Where: ‘K’ is the set of all Ontario market participants ‘k’ ‘H’ is the set of all settlement hours ‘h’ in the applicable quarter. ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | May 1, 2006 to April 30, 2009 | Due MP | 13 | N/A | N/A | N/A |  |  | The Ontario Power Generation Rebate payments will be based on the allocated quantity of energy withdrawn for the applicable quarter. |
| 113 | Additional Compensation for Administrative Pricing Credit |  | N/A |  | 7.8.4A.16 or 7.8.4A.10 or 7.13.6.2 |  | Manual Entry as per 7.8.4A.16, or 7.8.4A.10, or 7.13.6.2. | Monthly | Due MP | 13 | 13 | 0 | 13 |  |  | This charge will still be used for market suspension events |
| 114 | Outage Cancellation/Deferral Settlement Credit. |  | N/A |  | 5.6.7.4 |  | Manual Entry as per 5.6.7.4. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 115 | Unrecoverable Testing Costs Credit |  | N/A |  | 9.4.8.1.1 and4.5.3.4 |  | Manual Entry as per 4.5.3.4. | Monthly | Due MP | 13 | 13 | 13 | 13 |  |  |  |
| 116 | Tieline Maintenance Reliability Credit |  | N/A |  | 9.4.8.1.2 and 5.5.3.4 |  | Manual Entry as per 5.5.3.4. | Monthly | Due MP | 13 | 13 | 13 | 13 |  |  |  |
| 118 | Emergency Energy Rebate |  | N/A |  | 9.4.8.2 and 5.4.4A.1 |  | = H,cM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 119 | Station Service Reimbursement Credit |  | N/A |  | 9.4.8.1.6 and 9.2.1A.9 -2.1A14 |  | = {TDC,k,h m,T x [∑T2 (AQEWk,hM,t) / ∑K,hT (AQEWk,hm,t + SQEWk,hi,t )]} + {TDC2,k,H m,T x [∑H2T2 (AQEWk,hM,t) / ∑K,HT (AQEWk,hm,t + SQEWk,hi,t )]} + {TDC3,k,H m,T x [∑H4T2 (AQEWk,hM,t) / ∑K,H3T (AQEWk,hm,t + SQEWk,hi,t )]} Where: ‘T’ is the set of all metering intervals in settlement hour ‘h’. ‘M’ is the eligible generation station service delivery point ‘m’ of market participant ‘k’ ‘C’ is the set of the following hourly uplift charge type c as follows: 150, 155, 250, 252, 254, 451 ‘T2’ is the set of all metering intervals in settlement hour ‘h’ where the eligible generation facility was a net injector of energy into the IESO-controlled grid. ‘K’ is the set of all market participants ‘C2’ is the set of the following non-hourly monthly charge type ‘c’ as follows: 163,164,165,166,167,168,169,183, 184,450,452,454,460,550,1188, 1650 ‘C3’ is the set of the following daily charge type ‘c’ as follows: 1550, 1560 ‘H’ is the set of all settlement hours ‘h’ in the billing period ‘H2’ is the set of all settlement hours ‘h’ in the billing period where the eligible generation facility was a net injector of energy into the IESO-controlled grid. ‘H3’ is the set of all settlement hours ‘h’ in the day ‘H4’ is the set of all settlement hours ‘h’ in the day where the eligible generation facility was a net injector of energy into the IESO-controlled grid. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 120 | Local Market Power Debit |  | N/A |  | 9.4.8.2.2 and Ch. 7, Appendix 7.6 |  |  | Monthly | Due IESO | 13 | 13 | 0 | 13 |  |  |  |
| 121 | Northern Industrial Electricity Rate Program Settlement Amount |  | N/A |  | N/A |  | =  M HT (AQEWmht) x (Rate) Where: Rate is the program rate ‘M’ is the set of all delivery points ‘m’ for all market participant-eligible facilities. ‘H’ is the set of all settlement hours ‘h’ in the settlement period. ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours 5 H’. | Quarterly | Due MP | 0 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to Ministry of Northern Development, Mines and Forestry specifications. |
| 122 | Ramp Down Settlement Amount |  | RDSAk,h |  | 9.3.5A.1 |  | Let ‘BE’ be a matrix of n price-quantity pairs offered by market participant ‘k’ to supply energy during the settlement hour immediately before the hour in which ramp-down begins, adjusted by a ramp-down factor (RDF) as specified in the applicable market manual. Let OP(P,Q,B) be a function of Price (P), Quantity (Q) and an n x 2 matrix (B) of offered price-quantity pairs: OP(P,Q,B) = P∙Q - i=1s*Pi∙(Qi – Qi-1) – (Q - Qs*)∙Ps*+1 Where: s* is the highest indexed row of BE such that Qs*  Q  Qn and where, Q0=0 Using the terms below, let RDCk,hm,t be expressed as follows: RDCk,hm,t = MAX[0, [OP(EMPhm,t, MQSIk,hm,t, BE) – MAX(OP(EMPhm,t, DQSIk,hm,t, BE), OP(EMPhm,t,AQEIk,hm,t, BE))]] RDSAk,hm,t= MIN(-1 x RDCBk,hm,t, RDCk,hm,t) | Interval | Either Way | 13 | N/A | N/A | N/A |  |  | The RDF is defined in Market Manual 5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.31. |
| 123 | MACD Enforcement Activity Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by MACD | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 124 | SEAL Congestion Management Settlement Credit Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by MACD | Monthly | Due MP | 13 | 13 | 13 | 13 |  |  |  |
| 130 | Intertie Offer Guarantee Settlement Credit – Energy |  | IOGk,h and IOGk,hOFFSET |  | 9.3.8A.1 9.3.8A.3 and 7.3.5.8.1 |  | **CALCULATIONS FOR CHARGE TYPE 130 END OCTOBER 12, 2011. CHARGE TYPE 130 REPLACED BY CHARGE TYPE 1131 EFFECTIVE OCTOBER 13, 2011. The Intertie Offer Guarantee settlement amount is derived from an hourly Energy Import sub component (EIMk,h) as follows: I (-1)MIN[0, TOP(EMPhi,t, MQSIk,hi,t, BE)] See 9.3.8A.2 for the definition of the Operating Profit (OP) function referenced above. Where ‘I’ is the set of relevant intertie metering points ‘i’. Where ‘T’ is the set of all metering intervals ‘t’ during settlement hour ‘h’. The IOG_OFFSET component of this charge type applied on a monthly basis and is calculated as follows:  DA_IOGk,h + EIMk,hI (-1) * MIN[0,T OP(EMPhi,t, QSI{adj}k,hi,t, BE k,hi,t or PDR_BE k,hi,t) + TQSI{adj}k,hi,t ∕ TMIk,ht[n,1] * OPE’k,hi] (See 9.3.8A.4 for the derivation of the variable QSI{adj}k,hi,t , OPE’k,hi and the proper context of the matrix notation MIk,ht[n,1] used above ). | Hourly (the IOG Offset is debited) | Either Way | N/A | 13 | 13 | 13 |  |  | Compensation for cumulative, hourly financial losses as implied by the market schedule for Imports of energy at an intertie metering point. This amount is reduced by the IOG Offset when the import is part of an implied “wheeling through” transaction as described in Section 3.5.8.1 of Chapter 7. |
| 133 | Generation Cost Guarantee Payment |  | N/A |  | 9.4.7B |  | Dispatchable delivery points: MAX[0, (CGC + RT_COST - TEMPhm,t xAQEI{limited}k,hm,t -T CMSC_ REVk,hm,t] Subject to: AQEI{limited}k,hm,t = MIN[AQEIk,hm,t , minimum loading point] Where ‘CGC’ is a Submitted Combined Guaranteed Costs variable, assessed in accordance with the applicable market manual (see also Section 2.1 “Variable Description”). Where ‘m’ is delivery point ‘m’ at which the generation unit incurring the relevant costs is located. Where ‘T’ is a set of metering intervals ‘t’ from a valid start time until the earlier of: - the end of minimum generation block run-time; or - the end of the unit’s minimum run-time. Where AQEI{limited}k,hm,t shall denote all allocated quantities in MWh of energy injected at delivery point ‘m’ irrespective of any submission of physical allocation data by market participant ‘k’ in metering interval ‘t’ of settlement hour ‘h’, up to the generation unit’s minimum loading point. Where RT_COST is fuel and O&M cost component related to operation of the generation unit at its minimum loading point during its minimum generation block run-time (these costs are calculated based on the offer price associated with real-time dispatch). RT_COSTk = Σ T*H1 COST(AQEI{limited} k,h m,t, BE) - Where the COST function is defined as follows: where: - B is the n x 2 matrix (B) of offered price-quantity pairs (Pi , Qi) - s* is the highest indexed row of B such that Qs*-1 ≤ Q ≤ Qs* and where Q0=0 - Where ‘H1’ is the set of all settlement hours ‘h’ during the period from beginning of the minimum generation block run-time until the end of the calculated minimum run time. We consider that the minimum generation block run-time starts with the first hour after we add the submitted number of ramp intervals to the valid start-up hour. - Where ‘T*’ is the set of metering intervals ‘t’ in the set of all settlement hours ‘H1’ Where CMSC_REV k,hm,t is any real-time CMSC(TD k,h,105m,t) payment associated with allocated quantities in MWh of energy injected at delivery point ‘m’ irrespective of any submission of physical allocation data by market participant ‘k’ in metering interval ‘t’ of settlement hour ‘h’ up to the generation unit’s minimum loading point. CMSC_REV is calculated using the following rules: - Real-time CMSC (TD k,h,105m,t) for the same interval is greater than zero. - If MQSI k,hm,t and max(DQSI k,hm,t,AQEI k,hm,t) >= MLP, then CMSC_REVk,hm,t = 0. - In the case of a constrained-off event: - If MQSI k,hm,t < MLP, then CMSC_REV k,hm,t = TD k,h,105m,t - If MQSI k,hm,t >= MLP and max(DQSI k,hm,t,AQEI k,hm,t) <= MLP, then CMSC_REV k,hm,t = OP(EMP hm,t,MLP,BE) – OP(EMP,max(DQSI k,hm,t,AQEI k,hm,t),BE). - In the case of a constrained-on event: - If MQSI k,hm,t < MLP and min(DQSI k,hm,t,AQEI k,hm,t) < MLP, then CMSC_REV k,hm,t = TD k,h,105m,t - If MQSI k,hm,t <= MLP and min(DQSI k,hm,t, AQEI k,hm,t) >=MLP, then CMSC_REV k,hm,t = OP(EMP hm,t,MQSI k,hm,t,BE) – OP(EMP hm,t,MLP,BE) (See applicable market manual) | Hourly | MP | 13 | N/A | N/A | N/A |  |  |  |
| 134 | Demand Response Credit |  | N/A |  | 9.4.7C 9.4.7F |  | Manual Entry for TDRP (Refer to “Market Manual 5: Settlements, Part 5.10: Transitional Demand Response Program”. Manual Entry for ELRP (Refer to “Market Manual 10: Emergency Load Reduction Program (ELRP)”. | Monthly | Either way | 13 | N/A | NA | N/A |  |  | TDRP and ELRP suspended by the IESO. |
| 135 | Real-time Import Failure Charge |  | RT_IFCk,h |  | 9.3.8C.3 |  | I,T (-1) * MIN[MAX[ 0, (EMPhm,t + PB_IMht – PD_EMPhm,t) * RT_ISDk,hi,t], (MAX(0, EMPhm,t)* RT_ISDk,hi,t)] Where: ‘I’ is the set of all intertie metering points ‘i’. ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. RT_ISDk,hi,t = MAX (PD_DQSIk,hi,t – DQSIk,hi,t, 0) | Hourly | Due IESO | N/A | 13 | N/A | N/A |  |  | Subject to exemptions under the provisions of 9.3.8C.2.2. |
| 136 | Real-time Export Failure Charge |  | RT_EFCk,h |  | 9.3.8C.5 |  | I,T (-1) * MIN[MAX[ 0, (PD_EMPhm,t – EMPhm,t – PB_EXht) * RT_ESDk,hi,t], (MAX(0, PD_EMPhm,t)* RT_ESDk,hi,t)] Where: ‘I’ is the set of all intertie metering points ‘i’ ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’ RT_ESDk,hi,t = MAX (PD_DQSWk,hi,t – DQSWk,hi,t, 0) | Hourly | Due IESO | N/A | N/A | 0 | 13 |  |  | Subject to exemptions under the provisions of 9.3.8C.4.2. |
| 137 | Generation Cost Guarantee - Output Based Pricing System Reimbursement Settlement Amount |  | N/A |  | 9.4.7B.1.2 7.2.2B |  | Manual entry based on the calculations outlined in Market Manual 4: Market Operations Part 4.6: Real-Time Generation Cost Guarantee Program, section 5.4 Fuel Cost Recovery Methodology. | Monthly | Due MP | 13 | N/A | N/A | N/A | March 3, 2021 |  |  |
| 140 | Fixed Energy Rate Settlement Amount |  | N/A |  | N/A |  | ** CHARGE TYPE 140 REPLACED BY CHARGE TYPE 142 EFFECTIVE JANUARY 1, 2005 ** NOTE: The equations identified below apply to low volume and designated consumers (as defined in Ontario Energy Board Act, 1998 and associated regulations) in the IESO-administered market. For distributors, charge type 140 is applied once a month based on the values submitted by the distributor on IMO_FORM_1562 (monthly adjustment) and IMO_FORM_1505 (May-Nov 2002 refund). For IESO’s low volume and designated customers a fixed rate adjustment with a rate of 5.5 cents per kWh is applied on an interval basis using the equation below. A manual adjustment is applied at the end of the month to apply a rate of 4.7 cents per kWh for energy withdrawn up to 750 kWhs. Fixed Energy Rate Settlement Amount (dispatchable locations): Where net uncovered consumption > 0: T,m (EMPhm,t – FPh m )  (AQEWk,hm,t – AQEIk,hm,t- s BCQs,k,hm,t) Where net uncovered consumption = 0: T,m (EMPhm,t – FPh m )  ( -AQEIk,hm,t) SUBJECT TO: Net uncovered consumption = MAX [T,m (AQEWk,hm,t - s BCQs,k,hm,t),0] Fixed Energy Rate Settlement Amount (non-dispatchable locations): Where net uncovered consumption > 0: (HOEPh – FPh m)  m,T ( AQEWk,hm,t – AQEIk,hm,t - s BCQs,k,hm,t ) Where net uncovered consumption = 0: (HOEPh – FPh m)  m,T (- AQEIk,hm,t) SUBJECT TO: Net uncovered consumption = MAX [T,m (AQEWk,hm,t - s BCQs,k,hm,t),0] SUBJECT TO: Net uncovered consumption = MAX [T,m (AQEWk,hm,t - s BCQs,k,hm,t),0] | Hourly (type ‘DP’ records only. See: “Format Spec. for Settlement Statement Files and Data Files” for further details) | Either Way | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 141 | Fixed Wholesale Charge Rate Settlement Amount |  | N/A |  | N/A |  | ** CALCULATIONS FOR CHARGE TYPE 141 END MARCH 31, 2005 ** NOTE: The equations identified below apply to distributors, low volume and designated consumers (as defined in Bill 4 and associated regulations) in the IESO-administered market. For distributors an additional charge type 141 record is provided to reflect any monthly submission of IMO_FORM_1562. See IMO_FORM_1562 for further details. TDk,C AQEWk,hm,t * (FPC) Where: ‘H’ is all settlement hours ‘h’ during the billing period; and, ‘C’ is a designated group of charge types ‘c’ prescribed by government regulation (and associated rulings by the Ontario Energy Board) and consisting of the cumulative sum of the following charge types: 150, 155, 168, 170, 182, 183, 184, 250, 252, 254, 450, 452, 454, 550, 753, 9990 | Monthly | Either Way | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 142 | Regulated Price Plan Settlement Amount |  | N/A |  | N/A |  | NOTE: The equation identified below applies to low volume and designated consumers (as defined in Ontario Energy Board Act, 1998 and associated regulations) in the IESO-administered market. For distributors, charge type 142 is applied once a month based on the values submitted by the distributor via On-line settlement forms: “Regulated Price Plan vs. Market Price – Variance for Conventional Meters”, “Regulated Price Plan vs. Market Price – Variance for Smart Meters” and “Regulated Price Plan – Final Variance Settlement Amount”. Regulated Price Plan Settlement Amount: NEMSCk,H – { MIN [ TLQ , H M,T (AQEWk,hm,t – AQEIk,hm,t - s BCQs,k,hm,t) ] x RPPl=1 + MAX [0, H M,T (AQEWk,hm,t – AQEIk,hm,t - s BCQs,k,hm,t) – TLQ] x RPPl=2 } | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 143 | NUG Contract Adjustment Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by OEFC via On-line settlement form “NUG Adjustment Amount Information”, subject to Regulation. | Monthly | Due OEFC | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 144 | Regulated Nuclear Generation Adjustment Amount |  | N/A |  | N/A |  | For dispatchable delivery points: (GRP– EMPhm,t ) x AQEIk,hm,t For non-dispatchable delivery points: (GRP– HOEPh ) x T AQEIk,hm,t Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. | Interval or Hourly | Due OPG | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 145 | Regulated Hydroelectric Generation Adjustment Amount |  | N/A |  | N/A |  | NEMSCk,H – { H M,T [ ( MWAvgT x GRP) + ( ( AQEIk,hm,t – AQEWk,hm,t ) – MWAvgT ) x EMPhm,t ] } Where ‘M’ is the set of all delivery points ‘m’ of OPG’s regulated hydroelectric generating stations. ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. ‘H’ is the set of all settlement hours ‘h’ in the month. MWAvg is the average hourly net energy production within a given month. | Monthly | Due OPG | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to OEB regulation. |
| 146 | Global Adjustment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 146 END DECEMBER 31, 2010. CHARGE TYPE 146 REPLACED BY CHARGE TYPES 147 AND 148 EFFECTIVE JANUARY 1, 2011. For Fort Frances Power Corporation Distribution Inc.: H,M,CTD x (  HM,T AQEWk,hm,t + EGEIk -EEQ) / (K,HM,T AQEWk,hm,t +K EGEIk - EEQ ) For other market participants: H,M,CTD x ( HM,T AQEWk,hm,t +EGEIk) / (K,HM,T AQEWk,hm,t +K EGEIk - EEQ) Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘K’ is the set of all market participants ‘k’. Where ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. Where ‘C’ is the set of the following charge types ‘c’: 193, 194, 195, 197, 198, 1380, 1381, 1382, 1383, 1384, 1385, 1386, 1390, 1391, 1392, 1393, 1394, 1395, 1396, 1397, 1398, 1450, 1460, 1461, 1462 and 1464. | Monthly | Due MPs | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 147 | Class A – Global Adjustment Settlement Amount |  | N/A |  | N/A |  | H,M,CTD * PDFk,m,d Where ‘d’ is the ratio of the number of days in the month the Peak Demand Factor was effective compared to the total number of days in the month and ‘C’ is the set of the following charge types ‘c’: 193, 194, 195, 1380, 1381, 1382, 1383, 1384, 1385, 1386, 1390, 1391, 1392, 1393, 1394, 1395, 1396, 1397, 1398, 1466, 1450, 1460, 1461, 1462, 1464, 1468, 1469, 1471, 1472, 1473, 1474, 1475. | Monthly | Either Way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 148 | Class B – Global Adjustment Settlement Amount |  | N/A |  | N/A |  | For Fort Frances Power Corporation Distribution Inc.: H,M,CTD – TD147 )x MAX((  HM,T AQEWk,hm,t + EGEIk -EEQ),0) / Class B Load For other Class B Market Participants and Distributors: H,M,CTD – TD147 ) x MAX(( HM,T AQEWk,hm,t + EGEIk - GA_AQEWg,k,h,Mm,t – PGSh,M ),0) / Class B Load Class B Load = (K (MAX(HM,T AQEWk,hm,t +EGEIk - EEQ - HM,T GA_AQEWg,k,h,Mm,t -  PGSh,M ,0))) -K Uk Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘K’ is the set of all market participants ‘k’. Where ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. Where ‘C’ is the set of the following charge types ‘c’: 193, 194, 195, 1380, 1381, 1382, 1383, 1384, 1385, 1386, 1390, 1391, 1392, 1393, 1394, 1395, 1396, 1397, 1398, 1466, 1450, 1460, 1461, 1462, 1464, 1468, 1469, 1471, 1472, 1473, 1474, 1475. | Monthly | Either Way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 149 | Regulated Price Plan Retailer Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via On-line settlement form “Retailer Payments for Contract Price vs. HOEP for Regulated Consumers with a Retail Contract”. | Monthly | Due LDCs | 13 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 150 | Net Energy Market Settlement Uplift |  | N/A |  | 9.3.9.1 |  | C M,T TDk,h,c x [(AQEWk,hm,t + SQEWk,hi,t + RQ k,hm,t) / k M,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘C’ is the set of the following charge types ‘c’ as follows: 100, 101, 103, 104, 1131 ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the NEMSC component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Either Way | 13 | N/A | 0 | 13 |  |  |  |
| 155 | Congestion Management Settlement Uplift |  | N/A |  | 9.3.5.2 and 9.3.5.7 |  | cM,T TDk,h,(105, 106, 107, 108,122, 124, 1050, 1051) x [(AQEWk,hm,t + SQEWk,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘T’ is the set of 12 metering intervals‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the CMSC component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly or Monthly (see 9.3.5.7) | Either Way | 13 | N/A | 0 | 13 |  |  | Pursuant to market rules, Section 9.3.5.7, during an interim period, the disbursements of charge type 105 amounts adjusted as per Section 9.3.5.6 may be made on a monthly basis. |
| 161 | Northern Pulp and Paper Mill Electricity Transition Program Balancing Amount |  | N/A |  | N/A |  | K TDk,111 Where ‘k’ is part of a subset of eligible market participants ‘k’. | Quarterly | Due IESO | 0 | N/A | N/A | N/A |  |  | This program ends on September 30, 2010. |
| 162 | Ontario Power Generation Rebate Debit |  | N/A |  | N/A |  | ** CALCULATIONS FOR CHARGE TYPE 162 END April 30, 2009 ** Payment (n) = H [(HOEPh – ORL) x (ONPAOh x 0.85 – PAA) + (PAP – PAORL) x PAA)] OPG rebate (n) = Max [ 0, Payment (n) – Payment (n-1) + NCF (n-1) ] Where: ‘H’ is the set of all settlement hours ‘h’ from May 1, 2006 to the end of the applicable quarter. ‘n’ is the current quarter. ‘n-1’ is the previous quarter. NCF is the negative amount carried forward and calculated as NCF (n) = Min [ 0, Payment (n) – Payment (n-1) + NCF (n-1) ] | May 1, 2006 to April 30, 2009 | Due IESO | N/A | N/A | N/A | N/A |  |  | The OPG rebate quarterly payment will be based on a cumulative calculation commencing May 1, 2006 to the end of each quarter less the same cumulative calculation to the end of the previous quarter. Where the payment formula results in an amount owing to OPG for any quarter, no such payment will be made to OPG and any such amount will be carried forward into subsequent quarters. |
| 163 | Additional Compensation for Administrative Pricing Debit. |  | N/A |  | 7.8.4A.16 or 7.8.4A.10 or 7.13.6.2, and 9.4.8 |  | c,HM,T TDk,H,(113) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t )] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  | This charge will still be used for market suspension events. |
| 164 | Outage Cancellation/ Deferral Debit. |  | N/A |  | 5.6.7.4 and 9.4.8.1.3 |  | c,HM,T TDk,H,(114) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours H. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 165 | Unrecoverable Testing Costs Debit |  | N/A |  | 9.4.8.1.1 and 4.5.3.4 |  | = H,cM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 166 | Tieline Reliability Maintenance Debit |  | N/A |  | 9.4.8.1.2 and 5.5.3.4 |  | = H,cM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 167 | Emergency Energy Debit |  | N/A |  | 9.4.8.1.5 9.4.2.3A and 5.2.3.3A |  | = H,cM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘c’ is any payments made for emergency energy during the applicable period. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 168 | TR Market Shortfall Debit |  | N/A |  | 9.4.8.1.7 and 9.6.14.5.2 |  | For loads: TRCACk = TRCADL x H M,T [(AQEWk,hm,t) / K,H M,T (AQEWk,hm,t)] For exporters: TRCACk = TRCADE x H I,T [(SQEWk,hi,t) / K,H I,T (SQEWk,hi,t)] Where TRCADL =(  KTDC / KTDC,C1 ) x TRCAR TRCADE = ( KTDC1 / KTDC,C1 ) x TRCAR Where ‘C’ is the set of all monthly service charge types c as follows: 650,651,652. Where ‘C1’ is the set of all monthly export transmission charge types c as follows:653. Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. Where ‘M’ is the set of all delivery points ‘m’, excluding any intertie metering points. Where ‘I’ is the set of all intertie metering points ‘i’. Where ‘K’ is the set of all market participants ‘k’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 169 | Station Service Reimbursement Debit |  | N/A |  | 9.4.8.1.6 and 9.2.1A.12.2(a) |  | = H,cM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 170 | Local Market Power Rebate |  | N/A |  | 9.4.8.2.2 9.4.8.2.3 9.3.8A.5 9.3.8A.6 and Ch. 7, Appendix 7.6 |  | = H,CM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘c’ denotes charge type 120 and that portion of charge type 130 related to the IOG OFFSET settlement amount. Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 171 | Northern Industrial Electricity Rate Program Balancing Amount |  | N/A |  | N/A |  | K TDk,121 Where ‘k’ is part of a subset of eligible market participants ‘k’. | Quarterly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 173 | MACD Enforcement Activity Balancing Amount |  | N/A |  | N/A |  | ΣKTDk123 Where ‘K’ is the set of all market participants ‘k’. Where TDk123 is the settlement amount of charge type 123 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 183 | Generation Cost Guarantee Recovery Debit |  | N/A |  | 9.4.8.1.9 |  | = H,CM,T TDh,c x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘C’ is the set of the following charge types ‘c’ as follows: 133, 137 ‘H’ is the set of all settlement hours ‘h’ in the month. ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | IESO | 13 | N/A | 0 | 13 |  |  |  |
| 184 | Demand Response Debit |  | N/A |  | 9.4.7C 9.4.7F |  | k,H, (TD134) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘H’ is all settlement hours ‘h’ during the billing period. | Monthly | Either way | 13 | N/A | 0 | 5 |  |  | TDRP and ELRP suspended by the IESO. |
| 186 | Intertie Failure Charge Rebate |  | HUSAk,h |  | 9.3.9.1 |  | CM,T TDc x [(AQEWk,hm,t + SQEWk,hi,t + RQ k,hm,t) / k M,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘C’ is the set of the following charge types ‘c’ as follows: 135, 136, 1134, 1135, 1136 ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the IFCR component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 190 | Fixed Energy Rate Balancing Amount |  | N/A |  | N/A |  | ** CHARGE TYPE 190 REPLACED BY CHARGE TYPE 192 EFFECTIVE JANUARY 1, 2005 ** k,H,c (TD140) Where: ‘H’ is all settlement hours ‘h’ during the trading day for all trading days during the interim period beginning December 1, 2002. | Hourly (type ‘DP’ records only. See: “Format Spec. for Settlement Statement Files and Data Files” for further details) | Either Way | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 191 | Fixed Wholesale Charge Rate Balancing Amount |  | N/A |  | N/A |  | ** CALCULATIONS FOR CHARGE TYPE 191 END MARCH 31, 2005 ** k,H,c (TD141) Where: ‘H’ is all settlement hours ‘h’ during the billing period. | Monthly | Either Way | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 192 | Regulated Price Plan Balancing Amount |  | N/A |  | N/A |  | KTDk,142 Where ‘K’ is the set of all market participants ‘k’. Where TDk,142 is the total settlement amount of charge type 142 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 193 | NUG Contract Adjustment Balancing Amount |  | N/A |  | N/A |  | TD143 | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 194 | Regulated Nuclear Generation Balancing Amount |  | N/A |  | N/A |  | TD144 | Interval or Hourly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 195 | Regulated Hydroelectric Generation Balancing Amount |  | N/A |  | N/A |  | TD145 | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to OEB regulation. |
| 196 | Global Adjustment Balancing Amount |  | N/A |  | N/A |  | K,TDk,147, 148 -∑197 Where ‘K’ is the set of all market participants ‘k’. Where TDk,147, 148is the settlement amount of charge type 147 and 148 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 197 | Global Adjustment - Special Programs Balancing Amount |  | N/A |  | N/A |  | K TDk,1466 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1466 is the settlement amount of charge type 1466 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 198 | Renewable Generation Balancing Amount |  | N/A |  | N/A |  | ** CALCULATIONS FOR CHARGE TYPE 198 END DECEMBER 31, 2010 **. K TDk,148 Where ‘K’ is the set of all market participants ‘k’. Where TDk,148 is the settlement amount of charge type 148 for the month for market participant ‘k’. | Pending | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 199 | Regulated Price Plan Retailer Balancing Amount |  | N/A |  | N/A |  | KTDk,149 Where ‘K’ is the set of all market participants ‘k’. Where TDk,149 is the settlement amount of charge type 149 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 200 | 10 Minute Spinning Reserve Market Settlement Credit |  | ORSCk,h |  | 9.3.4.1 |  | m,t,r AQORr,k,hm,t x PRORr,hm,t | Interval | Due MP | 13 | 13 | N/A | N/A |  |  |  |
| 201 | 10 Minute Spinning Reserve Market Shortfall Rebate |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(251) x [(AQEWk,hm,t + SQEWhi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEW k,hi,t)] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 202 | 10 Minute Non-spinning Reserve Market Settlement Credit |  | ORSCk,h |  | 9.3.4.1 |  | m,t,r AQORr,k,hm,t x PRORr,hm,t | Interval | Due MP | 13 | 13 | N/A | N/A |  |  |  |
| 203 | 10 Minute Non-spinning Reserve Market Shortfall Rebate |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(253) x [(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEW k,hi,t)] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 204 | 30 Minute Operating Reserve Market Settlement Credit |  | ORSCk,h |  | 9.3.4.1 |  | m,t,r AQORr,k,hm,t x PRORr,hm,t | Interval | Due MP | 13 | 13 | N/A | N/A |  |  |  |
| 205 | 30 Minute Operating Reserve Market Shortfall Rebate |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(255) x [(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 250 | 10 Minute Spinning Market Reserve Hourly Uplift |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(200) x [(AQEWk,hm,t + SQEWk,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t )] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 251 | 10 Minute Spinning Market Reserve Shortfall Debit |  | ORSSDk,r,h |  | 9.3.8.2 |  | Manual Entry as per 9.3.8.2 where the value below which ORESFk,r,hm,t shall be set at zero equals ∞. | Interval | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 252 | 10 Minute Non-spinning Market Reserve Hourly Uplift |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(202) x [(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 253 | 10 Minute Non-spinning Market Reserve Shortfall Debit |  | ORSSDk,r,h |  | 9.3.8.2 |  | Manual Entry as per 9.3.8.2 where the value below which ORESFk,r,hm,t shall be set at zero equals ∞ | Interval | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 254 | 30 Minute Operating Reserve Market Hourly Uplift |  | HUSAh |  | 9.3.9.1 |  | cM,T TDk,h,(204) x [(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. Where RQk,hm,t is a reallocated quantity whereby market participant ‘k’ is a party to one or more physical bilateral contracts for settlement hour ‘h’ in which the operating reserve component of hourly uplift is to be reallocated between market participant ‘k’ and the other market participant that is a party to the contract in which: RQ k,hm,t = s,b [BCQk,b,hm,t - BCQs,k,hm,t] | Hourly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 255 | 30 Minute Operating Reserve Market Shortfall Debit |  | ORSSDk,r,h |  | 9.3.8.2 |  | Manual Entry as per 9.3.8.2 where the value below which ORESFk,r,hm,t shall be set at zero equals ∞ | Interval | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 400 | Black Start Capability Settlement Credit |  | N/A |  | 9.4.2.2 |  | Manual Entry as per 9.4.2.2 | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 404 | Regulation Service Settlement Credit |  | N/A |  | 9.4.2.3 |  | Manual Entry as per 9.4.2.3 | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 406 | Emergency Demand Response Program Credit |  | N/A |  | 9.4.2.3A |  | Manual Entry as per 9.4.2.3A | Monthly | Due MP | N/A | N/A | N/A | N/A |  |  | EDRP no longer contracted by the IESO. |
| 410 | IESO-Controlled Grid Special Operations Credit |  | N/A |  | 5.8.2.6 |  | Manual Entry as per 5.8.2.6 | Monthly | Either way | 13 | N/A | N/A | N/A |  |  |  |
| 450 | Black Start Capability Settlement Debit |  | N/A |  | 9.4.2.2 |  | = H,cM,T TDh,(400) x [(AQEWk,hm,t + SQEWk,,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 451 | Hourly Reactive Support and Voltage Control Settlement Debit |  | N/A |  | 9.4.2.4 |  | = CM,T TDh,c x [(AQEWk,hm,t + SQEWk,hi,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘C’ is the set of the following charge types ‘c’ as follows: 1401, 1402, 1404, 1405, 1451 Where ‘T’ is the set of all metering intervals ‘t’ during settlement hour ‘h’. | Hourly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 452 | Monthly Reactive Support and Voltage Control Settlement Debit |  | N/A |  | 9.4.2.4 |  | = H,CM,T TDh,c x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘C’ is the set of the following charge types ‘c’ as follows: 1403, 1406, 1407, 1408, 1409, 1417 Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 454 | Regulation Service Settlement Debit |  | N/A |  | 9.4.2.3 |  | = H,cM,T TDh,(404) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 460 | IESO-Controlled Grid Special Operations Debit |  | N/A |  | 5.8.2.6 |  | = H,cM,T TDh,(410) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Either way | 13 | N/A | 0 | 13 |  |  |  |
| 500 | Must Run Contract Settlement Credit |  | N/A |  | 9.4.2.1 |  | Manual Entry as per 9.4.2.1 | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 550 | Must Run Contract Settlement Debit |  | N/A |  | 9.4.2.1 |  | = H,cM,T TDh,(500) x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 600 | Network Service Credit |  | N/A |  | 9.4.1 / 9.4.3 |  | k,H,c (TD650) Where ‘H’ is the set of the settlement hours ‘h’ in the month during which the Network Service Demand occurs at every delivery point defined for Transmission Network Service charges. | Monthly | Due applicable transmitters | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 601 | Line Connection Service Credit |  | N/A |  | 9.4.1 / 9.4.3 |  | k,H,c (TD651) Where ‘H’ is the set of all settlement hours ‘h’ in the month during which the Line Connection Service Demand occurs at every delivery point defined for Transmission Line Connection Service charges. | Monthly | Due applicable transmitters | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 602 | Transformation Connection Service Credit |  | N/A |  | 9.4.1 / 9.4.3 |  | k,H,c (TD652) Where ‘H’ is the set of all settlement hours ‘h’ in the month during which the Transformation Connection Demand occurs at every delivery point defined for Transmission Transformation Connection Service charges. | Monthly | Due applicable transmitters | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 603 | Export Transmission Service Credit |  | N/A |  | 9.4.1 / 9.4.3 |  | k,H,c (TD653i ) Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘i’ is an intertie metering point ‘i’ where an export transaction occurred during the month Each charge type 603 line detail record line item is therefore totaled on the basis of TD653 per intertie metering point ‘i’ per month. | Monthly | Due applicable transmitter | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 650 | Network Service Charge |  | N/A |  | 9.4.1 / 9.4.3 |  | NSDk,hm x PTS-N The Billing Demand for Network Transmission Service (kW) is defined as the higher of: Transmission customer coincident peak demand (kW) in the hour of the month when the total hourly demand of all PTS customers is highest for the month; and 85% of the customer peak demand in any hour during the peak period. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 651 | Line Connection Service Charge |  | N/A |  | 9.4.1 / 9.4.3 |  | LCDk,hm x PTS-L Where ‘h’ is the settlement hour of the current billing period in which LCDk,hm denotes the non-coincident peak demand for the month. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 652 | Transformation Connection Service Charge |  | N/A |  | 9.4.1 / 9.4.3 |  | TCDk,hm x PTS-T Where ‘h’ is the settlement hour of the current billing period in which TCDk,hm denotes the non-coincident peak demand for the month. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 653 | Export Transmission Service Charge |  | N/A |  | 9.4.1 / 9.4.3 |  | H T SQEWk,hi,t x ETS Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ during the set of settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  | Subject to the OEB “Ontario Transmission Rate Order”. |
| 700 | Dispute Resolution Settlement Amount |  | N/A |  | 3.2.7 |  | Manual Entry as per 3.2.7 | Monthly | Due MP | 13 | 13 | 0 | 13 |  |  | Note: tax would follow original disputed transaction |
| 702 | Debt Retirement Credit |  | N/A |  | 9.4.6 |  | k,H,c TD752 | Monthly | Due Ministry of Finance | 0 | N/A | N/A | N/A |  |  | Ontario Regulations 493/01 and 494/01 See Ministry of Energy website for details. |
| 703 | Rural and Remote Settlement Credit |  | N/A |  | 9.4.4 |  | Manual Entry as per Reg | Monthly | Due MP as per Reg | 13 | N/A | N/A | N/A |  |  | Ontario Regulation 442/01 See Ministry of Energy website for details. |
| 704 | OPA Administration Credit |  | N/A |  | N/A |  | K TDk,754 Where ‘K’ is the set of all market participants ‘k’. Where TDk,754 is the settlement amount of charge type 754 for the month for market participant ‘k’. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 705 | Ontario Fair Hydro Plan First Nations On-reserve Delivery Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “First Nations On-Reserve Delivery Credit (FNDC)”; | Monthly | Due LDCs either way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 706 | Ontario Fair Hydro Plan Distribution Rate Protection Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Distribution Rate Protection (DRP)”; | Monthly | Due LDCs either way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 750 | Dispute Resolution Balancing Amount (IESO) |  | N/A |  | 3.2.7 |  | ∑KTDk,700, where applicable | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  |  |
| 751 | Dispute Resolution Board Service Debit |  | N/A |  |  |  |  |  |  | 13 | 13 | 13 | 13 |  |  |  |
| 752 | Debt Retirement Charge |  | N/A |  | 9.4.6 |  | AQEWk,hm,tx TP Where ‘k’ is part of a subset of market participants meeting the criteria of any government regulation defining the ultimate consumers of energy. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Ontario Regulations 493/01 and 494/01 See Ministry of Energy website for details. |
| 753 | Rural and Remote Settlement Debit |  | N/A |  | 9.4.4 |  | AQEWk,hm,tx TP | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Ontario Regulation 442/01 See Ministry of Energy website for details. |
| 754 | OPA Administration Charge |  | N/A |  | N/A |  | H T AQEWk,hm,t x TP Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in settlement hour ‘h’. Where TP is the rate ($/MWh) for the OPA Administration Charge set by OEB. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government regulation. |
| 755 | MOE - Ontario Fair Hydro Plan First Nations On-reserve Delivery Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,705 Where ‘K’ is the set of all market participants ‘k’. Where TDk,705 is the total settlement amount of charge type 705 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 756 | MOE - Ontario Fair Hydro Plan Distribution Rate Protection Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,706 Where ‘K’ is the set of all market participants ‘k’. Where TDk,706 is the total settlement amount of charge type 706 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 850 | Market Participant Default Settlement Debit (recovery) |  | N/A |  | 2.8.6 |  | Manual Entry as per 2.8.6 | Monthly | Due IESO | 13 | 13 | 13 | 13 |  |  |  |
| 851 | Market Participant Default Interest Debit |  | N/A |  | 2.8.3, 2.8.5 |  | Manual Entry as per 2.8.3 and 2.8.5 | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  |  |
| 900 | GST/HST Credit |  | N/A |  | N/A |  | C TDk,c A summation of all Goods and Services Tax Credits or Harmonized Sales Tax Credits payable to market participant ‘k’ across all charge types ‘c’. Where ‘C’ is the set of all charge types ‘c’. |  | Due MP | N/A | N/A | N/A | N/A |  |  | Only appear as “SC” record types. |
| 950 | GST/HST Debit |  | N/A |  | N/A |  | C TDk,c A summation of all Goods and Services Tax Debits or Harmonized Sales Tax Debits payable by market participant ‘k’ across all charge types ‘c’. Where ‘C’ is the set of all charge types ‘c’. |  | Due IESO | N/A | N/A | N/A | N/A |  |  | Only appear as “SC” record types. |
| 1050 | Self-Induced Dispatchable Load CMSC Clawback |  | N/A |  | 9.3.5.1A |  | - BUSINESS RULES are used in conjunction with the definitions below to specify the criteria by which the IESO will recover constrained off CMSC paid to dispatchable load facilities. - Business Rule 1 – Materiality: Constrained off CMSC is allowed for an interval during a constrained off event if the total amount of CMSC paid for the trade day to that dispatchable load is less than $4000. The daily total includes negative CMSC. - **BUSINESS RULE 1 – - MATERIALITY THRESHOLD END JUNE 1, 2019 - Business Rule 2 – Non-Dispatchable Portion of Load: Constrained off CMSC is not allowed for an interval during a constrained off event if the CMSC is paid for portions of the dispatch where the load has bid greater than or equal to MMCP, indicating that it is a non-dispatchable in that range. [-1OP(EMPhm,t, MQSWk,hm,t, BL) – MAX (-1OP(EMPhm,t, DQSWk,hm,t, BL), -1 OP(EMPhm,t,AQEWk,hm,t, BL)] – [-1OP(EMPhm,t, MQSWk,hm,t, BL) – MAX (-1OP(EMPhm,t, DQSWk,hm,t, BL),-1OP (EMPhm,t,AQEWk,hm,t, BL), -1OP(EMPhm,t,MChm, BL)] Where ‘MC’ is minimum consumption level and is equal to the quantity in the price quantity pair where the bidding price is MMCP (i.e., $2000). - This business rule applies unless CMSC is allowed because of materiality (defined by Business Rule 1). - Business Rule 3 – Dispatch Deviation: Constrained off CMSC is not allowed for an interval during a constrained off event if the current 5-minute constrained schedule exceeds the revenue meter value in the previous interval plus 2.5 minutes of ramping. This business rule applies unless CMSC is allowed because of: Materiality (defined by Business Rule 1); or The load has been constrained off economically (defined below – ‘Economically constrained off interval’); or Operating reserve has been activated (defined below – ‘Operating Reserve Activation interval’); or The load is ramping (defined below – ‘Ramping interval’); or The load has been manually dispatched down for reliability (defined below – ‘Manual Dispatch for Reliability’). - Business Rule 4 – Facility off-line or unable to follow dispatch instructions: Constrained off CMSC is not allowed for an interval during a constrained off event if the constrained schedule is 0 MW and the consumption is less than 1 MW, or if the consumption is 0 MW. - This business rule applies unless CMSC is allowed because of: Materiality (defined by Business Rule 1); or The load has been constrained off economically (defined below – ‘Economically constrained off interval’); or Operating reserve has been activated (defined below – ‘Operating Reserve Activation interval’); or The load has been manually dispatched down for reliability (defined below – ‘Manual Dispatch for Reliability’). - In addition to the Business Rules 1 to 4 described above, constrained off CMSC is not allowed for hour ‘h’ if a dispatchable load changes its energy bid that results in a change in the facility’s market schedule and the ramping up or down of the dispatchable load. Definitions – There are a number of definitions that are used in the specification of criteria for recovery of constrained off CMSC paid to dispatchable load facilities. These are: - Constrained-off event: A constrained off event comprises one or more consecutive intervals where the market schedule is greater than the constrained schedule and the market schedule is greater than the actual quantity of energy withdrawn. Both conditions must exist to be considered a constrained off event. - Economic Constrained–off interval: A dispatchable load is considered to be ‘economically constrained off’ in an interval if the relevant nodal price is greater than or equal to the bid price for either the current interval, the next interval or the previous interval. The inequality should be applied to the last MW constrained off. - Operating Reserve Activation Interval (ORA): A dispatchable load is considered to be dispatched in an interval as part of an activation of operating reserve if one or more of the following conditions exist: The constrained schedule is labeled with the reason code ‘ORA’. The interval is 1-3 intervals before an interval with the ‘ORA’ code. The interval is 1-3 intervals after an interval with the ‘ORA’ code. - Ramping Interval: A generation unit is considered to be ramping up or ramping down when the unconstrained schedule differs between consecutive hours. A dispatchable load is considered to be ‘ramping’ in an interval if one of the following exist: - It is one of the first 3 intervals of the second hour when ramping up. - It is one of the last 3 intervals of the first hour when ramping down. Manual Dispatch for Reliability: A dispatchable load is considered to be a ‘manually constrained off for reliability’ if the IESO Control Room logs indicate that the IESO needed to constrain off the load for system or for local requirements. | Interval | Due IESO | 13 | N/A | N/A | N/A |  |  | The decision rule for ramping up or down is described in Market Manual 5.5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.9.3. |
| 1051 | Ramp-Down CMSC Claw Back |  | RDCBk,h |  | 9.3.5.1G |  | RDCBk,hm,t = -1 x TD k,h,105m,t (See applicable market manual) | Interval | Either Way | 13 | N/A | N/A | N/A |  |  | Conditions for the Ramp-Down CMSC Claw Back are described in Market Manual 5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.31. |
| 1130 | Day-Ahead Intertie Offer Guarantee Settlement Credit |  | DA_IOGk,h |  | 9.3.8A.2A |  | **CALCULATIONS FOR CHARGE TYPE 1130 END OCTOBER 12, 2011. CHARGE TYPE 1130 REPLACED BY CHARGE TYPE 1131 EFFECTIVE OCTOBER 13, 2011. The Day-Ahead Intertie Offer Guarantee settlement amount is derived as follows: For all day-ahead import transactions other than those that are subject to a constrained on event in the real-time market: I (-1) * MIN[0, TOP(EMPhi,t, MIN(PDR_DQSIk,hi,t , DQSIk,hi,t), PDR_BEk,hi,t) + TDk,h,105i] Or, in the case of an import transaction subject to a constrained on event in the real-time market: I (-1) * MIN[0, TOP(EMPhi,t, MIN(PDR_DQSIk,hi,t , DQSIk,hi,t), PDR_BEk,hi,t) + OPE{adj}k,hi,t] See 9.3.8A.2A for the definition of the Operating Profit (OP) function referenced above. Where: ‘I’ is the set of relevant intertie metering points ‘i’. ‘T’ is the set of all metering intervals ‘t’ during settlement hour ‘h’. TDk,h,105i is that component of charge type 105 (“Congestion Management Settlement Credit for Energy”) applicable to market participant ‘k’ at intertie metering point ‘i’ during settlement hour ‘h’. | Hourly | Due MP | N/A | 13 | 13 | 13 |  |  | Subject to IOG OFFSET process under the provisions of 9.3.8A.3 (see also, entry for charge type 130 for further details) |
| 1131 | Intertie Offer Guarantee Settlement Credit |  | IOGk,h |  | 9.3.8A |  | The Day-Ahead Intertie Offer Guarantee settlement amount is derived as follows: I MAX[0,  T (DA_IOG_COMP1 + DA_IOG_COMP2 – DA_IOG_COMP3)] Where DA_IOG_COMP1: -1 x OP(EMPhi,t, MIN(DA_DQSIk,hi,t, DQSIk,hi,t), DA_BEk,hi,t) DA_IOG_COMP2: XDA_BEk,hi,t – MAX(0, XBEk,hi,t) DA_IOG_COMP3: Component 3 is calculated when: the CMSC for energy (TDk,h,105m,t) for the same metering interval is a value other than zero. For Component 3 (DA_IOG_COMP3), the six scenarios of the possible orderings of the generator’s DA_DQSI, DQSI and MQSI are as follows: - DQSI >= MQSI >= DA_DQSI - MQSI >= DQSI >= DA_DQSI - DQSI > DA_DQSI > MQSI - MQSI > DA_DQSI > DQSI - DA_DQSI >= DQSI > MQSI - DA_DQSI >= MQSI > DQSI Scenario 1 and 2: 0 Scenario 3: OP(EMPhi,t, MQSIk,hi,t, BE) – OP(EMPhi,t, DA_DQSIk,hi,t, BE) Scenario 4: OP(EMPhi,t, DA_DQSIk,hi,t, BE) – OP(EMPhi,t, DQSIk,hi,t, BE) Scenario 5 and 6: TDk,h,105m,t Where ‘I’ is the set of relevant intertie metering points ‘i’. ‘T’ is the set of all metering intervals ‘t’ during settlement hour ‘h’. ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8A.2. XDA_BEk,hi,t = (-1) * [OP(EMPhi,t, DA_DQSIk,hm,t, DA_BE) – OP(EMPhi,t, min(DA_DQSIk,hm,t, DQSIk,hm,t, DA_BE)] XBEk,hi,t = (-1) * [OP(EMPhi,t, DA_DQSIk,hi,t, BE) – OP(EMPhi,t, min(DA_DQSIk,hi,t, DQSIk,hi,t, BE)] Where EMPhi,t = 0 The Intertie Offer Guarantee settlement amount is derived from an hourly Energy Import sub component (EIMk,h) as follows: RT-IOGk,h = EIMk,h The Real-Time Intertie Offer Guarantee (RT-IOGk,h) settlement amount is derived as follows: I (-1)*MIN[0, TOP(EMPhi,t, MQSIk,hi,t, BE)] Where ‘I’ is the set of relevant intertie metering points ‘i’. ‘T’ is the set of all metering intervals ‘t’ during settlement hour ‘h’. ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8A.2. The IOG_OFFSET component of this charge type is calculated as follows: The Day-Ahead IOG rate: DA_IOG_RATE = IF [DA_IOG is not NULL, DA_IOG / min(DA_DQSI, DQSI), 0] The Real-Time IOG rate: RT_IOG_RATE = IF[RT_IOG is NULL, 0, RT_IOG/DQSI] The matrix is arranged in ascending order on DA_IOG_RATE and the day-ahead import quantities are offset against the day-ahead export schedule quantities: DA_DQSW_REM = [MAX[0, DA_OFFSET_DQSW)]] DA_OFFSET_DQSW = MIN[DA_DQSI, DQSI, DA_DQSW_REM] The day-ahead IOG offset flag: DA_OFFSET_FLAG = IF(DA_OFFSET_DQSW > [50% X MIN(DA_DQSI,DQIS)],Y,N) The IOG offset rate: IOG_SETTLEMENT_RATE = IF[DA_OFFSET_FLAG = ‘Y’, RT_IOG_RATE, MAX(RT_IOG_RATE, DA_IOG_RATE)] Subject to: MI[n,9] >= MIN[n-1,9] MI[1,9] = MIN[MI[1 to N,9]] MI[1 to N,9] <> 0 The Gross IOG amount: IOG = IOG dollar amount associated with the used to calculate IOG_SETTLEMENT_RATE The matrix is arranged in ascending order on IOG_SETTLEMENT_RATE and the real-time import quantities are offset against the real-time export schedule quantities: RT_DQSW_REM = [MAX[0, DQSW – RT_OFFSET_DQSW)]] RT_OFFSET_DQSW = MIN[DQSI, RT_DQSW_REM] The IOG offset settlement amount: IOG_OFFSET = (IOG_SETTLEMENT_RATE * RT_OFFSET_DQSW) The IOG settlement amount: NET_IOG = (IOG – IOG_OFFSET) | Hourly | Due MP | N/A | 13 | N/A | N/A |  |  |  |
| 1133 | Day-Ahead Generation Cost Guarantee Payment |  | DA_GCGk,h |  | 9.4.7D |  | **CALCULATIONS FOR CHARGE TYPE 1133 END OCTOBER 12, 2011. Dispatchable delivery points: MAX[0, (DA_CGC + DA_COST - TEMPhm,t xAQEI{limited}k,hm,t T CMSC REVk,h,m,t] Subject to: AQEI{limited}k,hm,t = MIN[AQEIk,hm,t , minimum loading point] Where ‘DA_CGC’ is a Day-Ahead Combined Guaranteed Costs variable, assessed in accordance with the applicable market manual (see also Section 2.1 “Variable Description”). Where ‘m’ is delivery point ‘m’ at which the generation unit incurring the relevant costs is located. Where ‘T’ is a set of metering intervals ‘t’ from a valid start time to the end of minimum generation block run-time. Where AQEI{limited}k,hm,t shall denote all allocated quantities in MWh of energy injected at delivery point ‘m’ irrespective of any submission of physical allocation data by market participant ‘k’ in metering interval ‘t’ of settlement hour ‘h’ up to the generation unit’s minimum loading point. Where DA_COST is fuel and O&M cost component related to operation of the generation unit at its minimum loading point during its minimum generation block run-time (these costs are calculated based on the offer price associated with Pre-dispatch of record). DA_COSTk = Σ T*H2 COST(AQEI{limited} k,h m,t, PDR_BEk,h m,t ) - Where the COST function is defined as follows: where: - B is the n x 2 matrix (B) of offered price-quantity pairs (Pi , Qi) - s* is the highest indexed row of B such that Qs*-1 ≤ Q ≤ Qs* and where Q0=0 - Where H2 is the set of all settlement hours ‘h’ during the period from the Pre-dispatch of Record ‘start hour’ until the end of minimum generation block run - Where ‘T*’ is the set of metering intervals ‘t’ in the set of all settlement hours ‘H2’ Where CMSC_REV k,hm,t is any real-time CMSC(TD k,h,105m,t) payment associated with allocated quantities in MWh of energy injected at delivery point ‘m’ irrespective of any submission of physical allocation data by market participant ‘k’ in metering interval ‘t’ of settlement hour ‘h’ up to the generation unit’s minimum loading point. CMSC_REV is calculated using the following rules: - Real-time CMSC (TD k,h,105m,t) for the same interval is greater than zero. - If MQSI k,hm,t and max(DQSI k,hm,t,AQEI k,hm,t) >= MLP, then CMSC_REVk,hm,t = 0. - In the case of a constrained-off event: - If MQSI k,hm,t < MLP, then CMSC_REV k,hm,t = TD k,h,105m,t - If MQSI k,hm,t >= MLP and max(DQSI k,hm,t,AQEI k,hm,t) <= MLP, then CMSC_REV k,hm,t = OP(EMP hm,t,MLP,BE) – OP(EMP,max(DQSI k,hm,t,AQEI k,hm,t),BE). - In the case of a constrained-on event: - If MQSI k,hm,t < MLP and min(DQSI k,hm,t,AQEI k,hm,t) < MLP, then CMSC_REV k,hm,t = TD k,h,105m,t - If MQSI k,hm,t <= MLP and min(DQSI k,hm,t, AQEI k,hm,t) >=MLP, then CMSC_REV k,hm,t = OP(EMP hm,t,MQSI k,hm,t,BE) – OP(EMP hm,t,MLP,BE) (See applicable market manual) | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1134 | Day-Ahead Linked Wheel Failure Charge |  | DA_LWFCk,h |  | 9.3.8E |  | MAX[(-1) * [(DA_LWSDk,hi) * MAX[0,( DA_PSk,hi – PD_PSk,hi)]], (RT_IFC_DALWk,hi + RT_EFC_DALWk,hi)] Where: ###### DA_LWSDk,hi,t = MAX[MAX (DA_DQSIk,hi,t – PD_DQSIk,hi,t, DA_DQSWk,hi,t – PD_DQSWk,hi,t),0] RT_IFC_DALWk,hi = I,T (-1) * MIN[MAX[ 0, (EMPhm,t + PB_IMht – PD_EMPhm,t) * MAX (DA_DQSIk,hi,t – PD_DQSIk,hi,t, 0)], (MAX(0, EMPhm,t)* MAX (DA_DQSIk,hi,t – PD_DQSIk,hi,t, 0))] RT_EFC_DALWk,hi = I,T (-1) * MIN[MAX[ 0, (PD_EMPhm,t – EMPhm,t – PB_EXht) * MAX (DA_DQSWk,hi,t – PD_DQSWk,hi,t, 0)], (MAX(0, PD_EMPhm,t) * MAX (DA_DQSWk,hi,t – PD_DQSWk,hi,t, 0))] Where: ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. ‘I’ is the set of all intertie metering points ‘i’. | Hourly | Due IESO | N/A | 13 | 13 | - 13 |  |  |  |
| 1135 | Day-Ahead Import Failure Charge |  | DA_IFCk,h |  | 9.3.8B |  | I,T (-1) * MIN[MAX[ 0, OP(PD_EMPhm,t, DA_DQSIk,hi,t, DA_BEk,ki,t) – OP(PD_EMPhm,t, PD_DQSIk,hi,t, DA_BEk,ki,t)] , (MAX(0, XPD_BEk,hi,t – XDA_BEk,hi,t)], (MAX(0, PD_EMPhm,t)* DA_ISDk,hi,t)] Where: ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. ‘T’ is the set of all metering intervals ‘t’ in settlement hour ‘h’. ‘I’ is the set of all intertie metering points ‘i’. DA_ISDk,hi,t = MAX (DA_DQSIk,hi,t – PD_DQSIk,hi,t, 0) XDA_BEk,hi,t = (-1) * [OP(0,DA_DQSI,DA_BE) -OP(0,PD_DQSI,DA_BE)] XPD_BEk,hi,t = (-1) * [OP(0,DA_DQSI,PD_BE) - OP(0,PD_DQSI,PD_BE)] | Hourly | Due IESO | N/A | 13 | N/A | N/A |  |  | Subject to exemptions under the provisions of 9.3.8B.1.2 |
| 1136 | Day-Ahead Export Failure Charge |  | DA_EFCk,h |  | 9.3.8D |  | I,T (-1) * MIN[MAX[ 0,(–1)* OP(PD_EMPhm,t, DA_DQSWk,hi,t, DA_BLk,ki,t) – (–1)* OP(PD_EMPhm,t, PD_DQSWk,hi,t, DA_BLk,ki,t)] , (MAX(0, XDA_BLk,hi,t – XPD_BLk,hi,t), (MAX(0, XDA_BLk,hi,t)] Where: ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. ‘T’ is the set of all metering intervals ‘t’ in settlement hour ‘h’. ‘I’ is the set of all intertie metering points ‘i’. XDA_BLk,hi,t = [OP(0,DA_DQSW,DA_BL) -OP(0,PD_DQSW,DA_BL)] XPD_BLk,hi,t = [OP(0,DA_DQSW,PD_BL) - OP(0,PD_DQSW,PD_BL)] | Hourly | Due IESO | N/A | N/A | 0 | 13 |  |  |  |
| 1137 | Intertie Offer Guarantee Reversal |  | Context 1: IOG_REVk,h Context 2: DA_IOG {adj}k,hi |  | 9.3.8A.1.2 and 9.3.8A.7 to 9.3.8A.9 |  | **CALCULATIONS FOR CHARGE TYPE 1137 END OCTOBER 12, 2011. NOTE: This charge type is used in two separate contexts as follows: Context 1: When a day-ahead Intertie Offer Guarantee and a real-time Intertie Offer Guarantee apply to the same import transaction, the lower of the two is reversed by this charge type. -1 x TDk,h,ci Where: ‘c’ is charge type 130 or 1130 as the case may be such that: TDk,h,c i = MIN (TDk,h,130i ,TDk,h,1130i) Context 2: In cases where this charge type is used for the purposes of applying the intertie offer guarantee adjustment (DA_IOG{adj}k,hi), the settlement amount applied is DA_IOG{adj}k,hi and is calculated as follows: DA_IOG{adj}k,hi = MAX [0 , IOG_FVk,hi – TDk,h,100i – MAX(TDk,h,1130i, TDk,h,130i ) – TDk,h,105 i] Where: TDk,h,100i, TDk,h,1130i , TDk,h,130i and TDk,h,105i are the settlement amounts for charge types 100, 1130, 130 and 105 respectively, that are applicable to market participant ‘k’ during settlement hour ‘h’ at intertie metering point ‘i’. | Context 1: Hourly Context 2: Hourly, but reported on the last trading day of the billing period | Context 1: Due IESO Context 2: Due MP | N/A | 13 | 13 | 13 |  |  | Note: Context 1 and Context 2 can both be applied to the same import. |
| 1138 | Day-Ahead Fuel Cost Compensation Credit |  | DA_FCCk,h |  | 9.4.7E |  | Manual entry as per 9.4.7E.2 | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1139 | Intertie Failure Charge Reversal |  | IFC_REVk,h |  | 9.3.8C.6 |  | **CALCULATIONS FOR CHARGE TYPE 1139 END OCTOBER 12, 2011. When a Day-Ahead Import Failure Charge and a Real-time Import Failure Charge apply to the same import transaction, the lower of the two is reversed by this charge type. -1 x TDk,h,ci Where: ‘c’ is charge type 135 or 1135 as the case may be such that: TDk,h,c i = MIN (-1 x TDk,h,135i ,-1 * TDk,h,1135i) | Hourly | Due IESO | N/A | 13 | N/A | N/A |  |  |  |
| 1142 | Ontario Fair Hydro Plan Eligible RPP Consumer Discount Settlement Amount |  | N/A |  | N/A |  | ** CHARGE TYPE 1142 REPLACED BY CHARGE TYPE 142 EFFECTIVE NOVEMBER 1, 2019 ** Manual entry based on: (1) the values submitted via on-line settlement forms “Regulated Price Plan vs. Market Price – Variance for Conventional Meters”, “Regulated Price Plan vs. Market Price – Variance for Smart Meters” and “Regulated Price Plan – Final Variance Settlement Amount”; or (2) For eligible IESO market participant consumers: NEMSCk,H – { MIN [ TLQ , ΣH M,T (AQEWk,hm,t – AQEIk,hm,t - Σs BCQs,k,hm,t) ] x RPPl=1 + MAX [0, ΣH M,T (AQEWk,hm,t – AQEIk,hm,t - Σs BCQs,k,hm,t) – TLQ] x RPPl=2 } | Monthly | Due LDCs, Unit Sub-Meter Providers and eligible MPs either way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 1143 | Ontario Fair Hydro Plan Eligible Non-RPP Consumer Discount Settlement Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Ontario Fair Hydro Plan (OFHP) for Eligible Non-RPP Customers”; | Monthly | Due LDCs, Unit Sub-Meter Providers and eligible MPs either way | 13 | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 1144 | Ontario Fair Hydro Plan Financing Entity Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Ontario Fair Hydro Plan – Financing Entity Funding Expenses”; | Monthly | Due Financing Entity | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 1145 | Ontario Fair Hydro Plan Financing Entity Interest |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Ontario Fair Hydro Plan – Financing Entity Funding Expenses”; | Monthly | Due Financing Entity | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 1148 | GA Energy Storage Injection Reimbursement |  | N/A |  | N/A |  | Uk x GARB | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Eligibility and other implementation details subject to government regulation. |
| 1188 | Day-Ahead Fuel Cost Compensation Debit |  | DA_FCC_Uk,h |  | 9.4.8.1.12 |  | = K,H,c M,T TDc x [(AQEWk,hm,t + SQEWk,hi,t) / K,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘c’ is charge type 1138. ‘K’ is the set of all market participants ‘k’. ‘M’ is the set of all delivery points ’m’ and intertie metering points ‘i’. ‘H’ is the set of all settlement hours ‘h’ in the month. ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 1192 | Ontario Fair Hydro Plan Eligible RPP Consumer Discount Balancing Amount |  | N/A |  | N/A |  | ** CHARGE TYPE 1192 REPLACED BY CHARGE TYPE 192 EFFECTIVE NOVEMBER 1, 2019 ** ΣKTDk,1142 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1142 is the total settlement amount of charge type 1142 for the month for market participant ‘k’. | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 1193 | Ontario Fair Hydro Plan Eligible Non-RPP Consumer Discount Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,1143 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1143 is the total settlement amount of charge type 1143 for the month for market participant ‘k’. | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  | Eligibility, rates, and other implementation details subject to government and OEB regulations. |
| 1194 | Ontario Fair Hydro Plan Financing Entity Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,1144 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1144 is the total settlement amount of charge type 1144 for the month for market participant ‘k’. | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 1195 | Ontario Fair Hydro Plan Financing Entity Balancing Interest |  | N/A |  | N/A |  | ΣKTDk,1145 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1145 is the total settlement amount of charge type 1145 for the month for market participant ‘k’. | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 1300 | Capacity Based Demand Response Program Availability Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1300 ENDED ON OCTOBER, 2018. = HAH x MCMWh x AAR Where: ‘AAR’ means ‘Adjusted Availability Rate’. ‘H’ is the total hours a DRMP is available in a program month. ‘HA’ means ‘Hours of Availability’. ‘MCMW’ means ‘Monthly Contracted MW’. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1301 | Capacity Based Demand Response Program Availability Over-Delivery Settlement Amt |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1301 ENDED ON OCTOBER, 2018. H (CMWh – MCMWh) x AODRh Applicable only in response to an ‘Open Standby Notification’. Where: ‘AODR’ means ‘Availability Over-Delivery Rate’. ‘CMW’ means ‘Confirmed MW’. ‘H’ is the set of all hours ‘h’ in the month where the ‘CMW’ exceeded the ‘MCMW’. ‘MCMW’ means ‘Monthly Contracted MW’. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1302 | Capacity Based Demand Response Program Availability Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1302 ENDED ON OCTOBER, 2018. The charge to a DRMP is highest of A, B or C: A: Availability Set-Off (Reliability) = H PSOh x AAR x MCMWh This formula applies when the Reliability Rate for a given Demand Response Account is less than 85% during any interval of an Activation Hour, or where the Participant is not Fully Available for Curtailment. Where: ‘AAR’ and ‘MCMW’ have the same meaning as in CT1300. ‘H’ is the set of all activation hours ‘h’ for the activation period. ‘PSO’ means ‘Performance Set-Off Factor’ as described in the market manual. B: Availability Set-Off (Timely Confirmation) = PSO x AAR x MCMWh x CDP This formula applies when the Participant, regardless of Activation, has failed to deliver, or delivers late, a Confirmation that is required by the IESO. Where: ‘AAR’ and ‘MCMW’ have the same meaning as in CT1300. ‘CDP’ (Contracted Dispatch Period) means four consecutive hours. Each Contracted Dispatch Period shall occur within the hours of Availability, and shall occur within and no more than once in accordance with the Daily Schedule. ‘PSO’ has the same meaning as defined above. C: Availability Set-Off (Low Confirmation) = H (PSO x AAR x (MCMWh – CMW) This formula applies when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for a Confirmed Hour of the Contracted Dispatch Period. Where: ‘AAR’ and ‘MCMW’ have the same meaning as in CT1300. ‘CMW’ has the same meaning as in CT1301. ‘H’ is the set of all confirmed hours ‘h’ when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for the Contracted Dispatch Period. ‘PSO’ has the same meaning as defined above. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1303 | Capacity Based Demand Response Program Utilization Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1303 ENDED ON OCTOBER, 2018. = [H (AAMh × URh)] – [H (NGh x MIN(HOEP, URh))] Where: ‘AAM’ (Actual Activated MWh), means the number of MWh Curtailed by a Participant when requested by the IESO, as measured through the use of electricity meter(s). Curtailment shall not exceed the product of the Activation MW and the activation period requested by the IESO, plus the lesser of an additional 15% of the Activation MW per hour of the activation period, OR 15 MWh per hour of the activation period. ‘H’ is the total hours ‘h’ a DRMP is activated in a program month. ‘HOEP’ means Hourly Ontario Energy Price. ‘NG’ (Net Generation), means the MWh of net electricity generated by any contributor that is a behind the meter generator. ‘UR’ (Utilization Rate), means the rates, expressed in $/MWh, as specified in the Demand Response Schedule. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1304 | Capacity Based Demand Response Program Utilization Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1304 ENDED ON OCTOBER, 2018. The charge to a DRMP is highest of A, B or C: A: Utilization Set-Off (Reliability) = H PSOh x UR x MCMWh This formula applies when the Reliability Rate for a given Demand Response Account is less than 85% during any interval of an Activation Hour. Where: ‘H’ is the set of all activation hours ‘h’ for the activation period. ‘PSO’ has the same meaning as in CT 1301. ‘UR’ has the same meaning as in CT1303. ‘MCMW’ has the same meaning as in CT1300. B: Utilization Set-Off (Timely Confirmation) = PSO x UR x MCMWh x CDP This formula applies when the DRMP, regardless of Activation, has failed to deliver, or delivers late, a Confirmation that is required by the IESO. Where: ‘CDP’ (Contracted Dispatch Period) means four consecutive hours. Each Contracted Dispatch Period shall occur within the hours of Availability, and shall occur within and no more than once in accordance with the Daily Schedule. ‘MCMW’ has the same meaning as defined above. ‘PSO’ has the same meaning as defined above. ‘UR’ has the same meaning as defined above. C: Utilization Set-Off (Low Confirmation) = H (PSO x UR x (MCMWh – CMW) This formula applies when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for a Confirmed Hour of the Contracted Dispatch Period. Where: ‘CMW’ has the same meaning as in CT1301. ‘H’ is the set of all confirmed hours ‘h’ when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for the Contracted Dispatch Period. ‘MCMW’ has the same meaning as defined above. ‘PSO’ has the same meaning as defined above. ‘UR’ has the same meaning as defined above. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1305 | Capacity Based Demand Response Program Planned Non-Performance Event Set-Off Amt |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1305 ENDED ON OCTOBER, 2018. The Planned Non-Performance Availability Set-Off applies for any day for which a participant has requested a Non-Performance Event as part of either a Single Day Non-Performance Event or a part of an Extended Period Planned Non-Performance Event. The monthly set-off calculation is the sum of all: - Non-Activation Day Non-Performance Availability Set-Off s and - Activation Day Non-Performance Availability Set-Offs. For 1.) The Non-Activation Day Non-Performance Availability Set-Off amount is: = (AAR x MCMWh x HANEH) Where: ‘AAR’ has the same meaning as in CT1300. ‘HANE’ (Hours of Availability for a Non-Performance Event), represents the Hours of Availability for all days in the contract month for which a planned Non-Performance Event is requested and for which an Activation Notice is not received by the participant. ‘MCMW’ has the same meaning as in CT1300. For 2.) The Activation Day Non-Performance Availability Set-Off amount is: = (OH x AAR x MCMWh x NEWFH) Where: ‘AAR’ and ‘MCMW’ have the same meaning as in CT1300. ‘OH’ (Opportunity Hours), means 64 if Option A is applicable to the Demand Response Account; or 32 if Option B is applicable to the Demand Response Account. ‘NEWF’ (Non-Performance Event Weighting Factor), means 10%, unless the Actual Activated MWh per interval, as averaged over all of the Intervals in the Contracted Dispatch Period for the Activation, is greater than or equal to the product of the Monthly Contracted MW and 1/12 of an hour in which case ‘NEWF’ means 50%. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1306 | Capacity Based Demand Response Program Measurement Data Set-Off Settlement Amt |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1306 ENDED ON OCTOBER, 2018. = MDSF x (HAH x MCMWh x AAR) This formula applies when the complete set of weekly measurement data for a Demand Response Account are not received as per the CBDR Processing Timelines. The formula recovers a percentage of the availability payment for the applicable week. Where: ‘MDSF’ (Measurement Data Set-Off Factor), is an increasing factor for every week that the full data remains undelivered. The factor is equal to: - 20% for the first week that the full data remains undelivered; - 33% for the second week that the full data remains undelivered; - 50% for the third week that the full data remains undelivered; and - 100% for the fourth week that the full data remains undelivered. ‘AAR’, ‘HA’ and ‘MCMW’ have the same meaning as in CT1300. ‘H’ is the total hours a DRMP is available for the applicable week. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1307 | Capacity Based Demand Response Program Buy-Down Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1307 ENDED ON OCTOBER, 2018. Buy-Down means the act by the DRMP chooses to reduce its Monthly Contracted MW and/or remove up to three Daily Schedules from participation in CBDR. For the Buy-Down of Monthly Contracted MW the payment is: = (MCMWR x BDR x HAE) Where: ‘MCMWR’ (Monthly Contracted MW Reduction), means the MW of demand reduction in the Monthly Contracted MWs. ‘BDR’ (Buy-Down Rate), means the Buy-Down Rate, expressed in $/MW. ‘HAE’ (Hours of Availability Elapsed), means the number of Hours of Availability that have elapsed in the Schedule Term up to the date that the reduction takes effect. For the Buy-Down of the Daily Schedules the payment is: = (MCMW x RD x BDR x HAE) Where: ‘BDR’ has the same meaning as defined above. ‘HAE’ has the same meaning as defined above. ‘MCMW’ has the same meaning as in CT1300. ‘RD’ (Requested Days), means the number of Business Days per week from which the Hours of Availability are to be removed. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1308 | Capacity Based Demand Response Program Performance Breach Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1308 ENDED ON OCTOBER, 2018. Performance breach amounts are calculated as defined in the market manual. | Monthly | Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1309 | Demand Response Pilot – Availability Payment |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1309 ENDED ON APRIL, 2018. Calculated as per demand response pilot contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Demand Response Pilot |
| 1310 | Demand Response Pilot – Availability Clawback |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1310 ENDED ON APRIL, 2018. Calculated as per demand response pilot contracts. | Hourly | Due IESO | 13 | N/A | N/A | N/A |  |  | Demand Response Pilot |
| 1311 | Demand Response Pilot – Availability Charge |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1311 ENDED ON APRIL, 2018. Calculated as per demand response pilot contracts. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Demand Response Pilot |
| 1312 | Demand Response Pilot – Availability Adjustment |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1312 ENDED ON APRIL, 2018. Calculated as per demand response pilot contracts. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Demand Response Pilot |
| 1313 | Demand Response Pilot – Demand Response Bid Guarantee |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1313 ENDED ON APRIL, 2018. Calculated as per demand response pilot contracts. Notes: - Bid guarantee as a payment is Due MP; bid guarantee as a clawback is Due IESO. Bid guarantee is calculated per unit commitment period/event. | Monthly | Either Way | 13 | N/A | N/A | N/A |  |  | Demand Response Pilot |
| 1314 | Capacity Obligation – Availability Payment |  | N/A |  | N/A |  | ∑hn CCOk x CACPh Where ‘h’ is an hour within the hours of availability for the month. Where ‘n’ is the number of hours of availability during a business day multiplied by the number of business days in the month which the settlement is for. | Monthly | Due MP | 13 | 13 | N/A | N/A |  |  |  |
| 1315 | Capacity Obligation – Availability Charge |  | N/A |  | N/A |  | For capacity dispatchable load resources and hourly demand response resources: ∑hn (-1) x Max( 0, (CCOk - DREBQh)) x CACPh x CNPFm For capacity generation resources, system-backed capacity import resources, generator-backed capacity import resources and capacity storage resources: ∑hn (-1) x Max( 0, (CCOk - CAEOh)) x CACPh x CNPFm Where ‘h’ is an hour within the hours of availability for the day. Where ‘n’ is the number of hours of availability for the day. | Daily | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 1316 | Capacity Obligation – Administration Charge |  | N/A |  | N/A |  | (-1) x Availability Paymentm Where ‘m’ is the month that is being settled. Where ‘Availability Payment’ is the settlement amount calculated for CT1314. | Monthly | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 1317 | Capacity Obligation – Dispatch Charge |  | N/A |  | N/A |  | (-1) x DRSQtyh x CACPh x CNPFm Where ‘h’ is an hour in which the hourly demand response resource failed to follow its dispatch instruction. | Hourly | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 1318 | Capacity Obligation – Capacity Charge |  | N/A |  | N/A |  | For capacity auction capacity test activations: (-1) x Availability Paymentm Where ‘m’ is the month that is being settled. Where ‘Availability Payment’ is the settlement amount as calculated for CT1314. | Monthly | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 1319 | Capacity Obligation – Buy-Out Charge |  | N/A |  | N/A |  | =50% x∑dn CBOCk x CACP x (1 - CNPFm) Where ‘d’ is a business day as defined in the Market Rules Chapter 11. Where ‘n’ is the range of business days from the buy-out effective date to the end of the commitment period. Where ‘m’ is the month that corresponds to the business day. | Monthly | Due IESO | 13 | 13 | N/A | N/A |  |  |  |
| 1320 | Capacity Obligation – Out of Market Activation Payment |  | N/A |  | Chapter 9, Section 4.7J.5 |  | For capacity auction dispatch test activations: HDRTAPR X HDRDCh For emergency operating state activations: Max(0, HDRBPh – Max(0,HOEPh)) X HDRDCh Where h is an hour within the activation window | Hourly | Due MP | 13 | 13 | N/A | N/A |  |  |  |
| 1321 | Capacity Obligation – Availability Charge True Up Payment |  | N/A |  | TBD |  | For capacity dispatchable load resources and hourly demand response resources: Min ((-1) x ∑mTD1315, ∑m∑d∑h Max (0,(RACk– CCOk) x CACPh x CNPFm)) For capacity generation resources, system-backed capacity import resources, generator-backed capacity import resources and capacity storage resources: Min ((-1) x ∑mTD1315, ∑m∑d∑h Max (0,(Min (CAEOh ,1.15*CCOk, CICAPk) – CCOk) x CACPh x CNPFm)) Where TD1315 is the total dollars charged during all months of the obligation period for the capacity auction resource under CT1315. Where m is a month within the obligation period, d is a business day within the month, h is an hour within the availability window for the day. | Bi-annually (At the end of each obligation period) | Due MP | TBD | TBD | TBD | TBD |  |  |  |
| 1322 | Capacity Obligation – Augmented Availability Charge |  |  |  |  |  | *For declaration of potential declaration/potential declaration and HDRs standby notices* For capacity dispatchable load resources and hourly demand response resources: ∑hn (-1) x Max( 0, (CCOk - DREBQh)) x CACPh x (10-CNPFm) For capacity generation resources, system-backed capacity import resources, generator-backed capacity import resources and capacity storage resources: ∑hn (-1) x Max( 0, (CCOk - CAEOh)) x CACPh x (10-CNPFm) Where ‘h’ is an hour within the hours of declaration of potential emergency operating state or HDRs standby notice for the day. Where ‘n’ is the number of hours of the declaration of potential emergency operating state or HDRs standby notice for the day. | Monthly | Due MP | TBD | TBD | TBD | TBD |  |  |  |
| 1323 | Capacity Obligation – Capacity Call Failure Charge |  |  |  |  |  | For capacity call, applied only to generator-backed capacity import resources: (-2) x Availability Paymentm Where ‘m’ is the month that is being settled. | Monthly | Due MP | TBD | TBD | TBD | TBD |  |  |  |
| 1324 | Capacity Obligation – Capacity Deficiency Charge |  |  |  |  |  | For generator-backed capacity import resources: ∑hn (-1.5) x OCMWk x CACPh Where ‘h’ is an hour within the hours of availability for the month. Where ‘n’ is the number of hours of availability during a business day multiplied by the number of business days in the month multiplied by the number of months in the applicable obligation period. | Monthly | Due MP | TBD | TBD | TBD | TBD |  |  |  |
| 1330 | On behalf of the former OPA for the DR2 Program - Availability Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1330 ENDED ON FEBRUARY 28, 2015. = H CoMWh x AR x ILSR Where: ‘CoMW’ (Contracted MW), means the MW specified in the DR2 Schedule(s) for a given Settlement Account which the Participant agrees to Load Shift in each On-Peak Contract hour. ‘AR’ (Availability Rate), means the availability rate, expressed in $/MW, in the amount as specified by the OPA from time to time on the OPA Website pursuant to the DR2 Program Rules. ‘H’ is the total On-Peak contract hours in a Contract Month. ‘ILSR’ (Implied Load Shift Ratio), has the meaning as defined in OPA’s DR2 Program Rules and is calculated as follows: ILSR = (-1) x [Implied Load Shift - ((3/4)(Load Shift Credit))] / Implied Load Shift Requirement | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1331 | On behalf of the former OPA for the DR2 Program - Availability Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1331 ENDED ON FEBRUARY 28, 2015. The charge to a DR participant is the highest of amounts A, B or C plus amount D; where A, B and C cannot occur within an on-peak period that was subject to D. A: Availability Set-Off (Reliability) = H PSOh x AR x CoMWh x ILSR This formula applies when the Actual MW Reliability Ratio for a given Settlement Account is less than 95% during the Summer and Winter seasons and less than 90% during the shoulder seasons. The Actual MW Reliability Ratio, which shall not be greater than 100%, shall be calculated as follows: - For each On-Peak Contract Hour, the Actual MW Reliability Ratio is defined as the result of the baseline MW minus the actual MW divided by the confirmed MW. ‘PSO’ (Performance Set-Off Factor) refers to a set of factors defined in the OPA DR2 Program Rules. ‘AR’ has the same meaning as in CT1330. ‘CoMW’ has the same meaning as in CT1330. ‘H’ is the set of all hours ‘h’ in the On-Peak Contract period where the required reliability is not met. ‘ILSR’ has the same meaning as in CT1330. B: Availability Set-Off (Timely Confirmation) = PSO x AR x CoMWh x H x ILSR This formula applies when the Participant has failed to deliver, or delivers late, a Confirmation that is required by the IESO pursuant to the DR2 Program Rules. Where: ‘PSO’ has the same meaning as defined above. ‘AR’ has the same meaning as in CT1330. ‘CoMW’ has the same meaning as in CT1330. ‘H’ is the set of all hours in the On-Peak Contract period. ‘ILSR’ has the same meaning as in CT1330. C: Availability Set-Off (Low Confirmation) = H PSO x AR x (CoMWh – CMW) x ILSR This formula applies when the Confirmed MW is less than the product of the Required Reliability Ratio and the Contracted MW for one or more On-Peak Contract hours. Where: ‘PSO’ has the same meaning as defined above. ‘AR’ has the same meaning as in CT1330. ‘CoMW’ has the same meaning as in CT1330. ‘CMW’ (Confirmed MW) means the number of MW available to shift by the Participant. ‘H’ is the set of all confirmed hours ‘h’ when the Confirmed MW’s are: - Less than 95% during the Summer and Winter seasons or - Less than 90% during the shoulder seasons of the Contracted MW. ‘ILSR’ has the same meaning as in CT1330. D: Availability Set-Off (Non-Performance) = PSO x AR x CoMWh x H x ILSR This formula applies when the Participant has taken an Extended Planned Non-Performance Event or Single Day Planned Non-Performance Event. Where: ‘PSO’ has the same meaning as defined above. ‘AR’ has the same meaning as in CT1330. ‘CoMW’ has the same meaning as in CT1330. ‘H’ is the set of all hours in the On-Peak Contract period. ‘ILSR’ has the same meaning as in CT1330. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA Program Rules. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1332 | On behalf of the former OPA for the DR2 Program - Utilization Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1332 ENDED ON FEBRUARY 28, 2015. The monthly Utilization Payment to a DR2 participant is the sum of the weekly utilization payments for the contract month and calculated as follows: Weekly Utilization payment = P Max[(GHDiff – AHDiff),0] x Min[(CoMWh x 1.15),(Curt p)] x ILSR Where: ‘GHDiff’ (Guaranteed weekly HOEP Differential), means the weekly differential rate, expressed in $/MWh, as specified by the OPA ‘AHDiff’ (Actual weekly HOEP Differential), is equal to the average actual HOEP for all hours of the useable On-Peak Contract Periods in the Week less the average actual HOEP for all hours in the Off-Peak Period for the same Week. ‘CoMWh’ (Contracted MWh), means the MWh specified in the DR2 Schedule(s) for a given Settlement Account which the Participant agrees to Load Shift in each On-Peak Contract Period. ‘Curt’ (Curtailment), means the number of MWh Curtailed by a Participant for each useable on-peak contract period, and shifted to the off-peak period as measured through the use of electricity meter(s). ‘P’ is the total number of On-Peak Contract Periods ‘p’ for a Participant in a Contract Week ‘ILSR’ has the same meaning as in CT1330. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1333 | On behalf of the former OPA for the DR2 Program - Utilization Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1333 ENDED ON FEBRUARY 28, 2015. The charge to a DR participant is highest of A, B or C where A, B and C cannot occur within an on-peak period that was subject to an Availability Set-Off (Non-Performance) event: A: Utilization Set-Off (Reliability) = P PSO x Max[(GHDiff – AHDiff),0] x CoMWh p x ILSR This formula applies when the Actual MWh Reliability Ratio for a given Settlement Account is less than 95% during the Summer and Winter seasons and less than 90% during the shoulder seasons. The Actual MWh Reliability Ratio, which shall not be greater than 100%, shall be calculated as follows: - For each On-Peak Contract Period, the Actual MWh Reliability Ratio is defined as the result of the baseline MWh minus the actual MWh divided by the product of the confirmed MW and the On-Peak Contract Hours. Where: ‘PSO’ (Performance Set-Off Factor) refers to a set of factors defined in the OPA’s Program Rules. ‘GHDiff’ has the same meaning as in CT1332. ‘AHDiff’ has the same meaning as in CT1332. ‘CoMWh’ has the same meaning as in CT1332. ‘P’ is the total number of On-Peak Contract Periods ‘p’ for a Participant in a Contract Month. ‘ILSR’ has the same meaning as in CT1330. B: Utilization Set-Off (Timely Confirmation) = P PSO x Max[(GHDiff – AHDiff),0] x CoMWh p x ILSR This formula applies when the Participant has failed to deliver, or delivers late, a Confirmation that is required by the IESO pursuant to the DR2 Program Rules. Where: ‘PSO’ has the same meaning as defined above. ‘GHDiff’ has the same meaning as in CT1332. ‘AHDiff’ has the same meaning as in CT1332. ‘CoMWh’ has the same meaning as in CT1332. ‘P’ is the total such On-Peak Contract Periods ‘p’ for a Participant in a Contract Month when the Participant has failed to deliver, or delivers late, a Confirmation. ‘ILSR’ has the same meaning as in CT1330. C: Utilization Set-Off (Low Confirmation) = P PSO x Max[(GHDiff – AHDiff),0] x (CoMWh – CMWhp) x ILSR This formula applies when the Confirmed MWh are less than the product of the Required Reliability Ratio and the Contracted MWh for an On-Peak Contract Period. Where: ‘PSO’ has the same meaning as defined above. ‘GHDiff’ has the same meaning as in CT1332. ‘AHDiff’ has the same meaning as in CT1332. ‘CoMWh’ has the same meaning as in CT1332. ‘CMWh’ (Confirmed MWh) means the MWh available confirmed for shifting by the Participant. ‘P’ is the total such On-Peak Contract Periods ‘p’ for a Participant in a Contract Month. ‘ILSR’ has the same meaning as in CT1330. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1334 | On behalf of the former OPA for the DR2 Program – Meter Data Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1334 ENDED ON FEBRUARY 28, 2015. = MDSF x TDk,1330 / NoWk) This formula applies when the complete set of weekly meter data for a Settlement Account is not received by 15:00 EST on the first Business Day of the following week. The formula recovers a percentage of the Availability Payment, as pro-rated for that week in question. Where: ‘MDSF’ (Meter Data Set-Off Factor), is an increasing factor for every week that the full data remains undelivered. The factor is equal to: - 20% for the first week that the full data remains undelivered; - 33% for the second week that the full data remains undelivered; - 50% for the third week that the full data remains undelivered; and - 100% for the fourth week that the full data remains undelivered. TDk,1330 is the settlement amount of charge type 1330 for month ‘k’ for the DR2 participant. ‘NoW’ (Number of Weeks) means the number of Weeks contained in the Contract month. ‘k’ is the Contract month. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1335 | On behalf of the former OPA for the DR2 Program - Buy-Down Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1335 ENDED ON FEBRUARY 28, 2015. Buy-Down means the act by the Participant of reducing its Contracted MW and/or the number of On-Peak Contract hours from participation in DR2. For the Buy-Down of Seasonal Contracted MW the payment is: = (SCMWR x BDR x CHE) Where: ‘SCMWR’ (Seasonal Contracted MW Reduction), means the MW of demand reduction in the Seasonal Contracted MWs. ‘BDR’ (Buy-Down Rate), means the Buy-Down Rate, expressed in $/MW. ‘CHE’ (on-peak Contract Hours Elapsed), means the number of On-Peak Contract Hours that have elapsed in the Schedule Term up to the date that the reduction takes effect. For the Buy-Down of the number of On-Peak Contract hours, the payment is: = (CoMW x PRCH x BDR x CHE) Where: ‘CoMW’ has the same meaning as in CT1330. ‘PRCH’ (Percent Reduction in Contract Hours), means the percent reduction in On-Peak Contract Hours requested. ‘BDR’ has the same meaning as defined above. ‘CHE’ has the same meaning as defined above. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1336 | On behalf of the former OPA for the DR2 Program - Miscellaneous Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1336 ENDED ON FEBRUARY 28, 2015. Reserved for DR2 payments or charges of a miscellaneous nature not specifically covered under Charge Types 1330 through 1335. | Monthly | Due DR2-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1340 | On behalf of the former OPA for the DR3 Program – Availability Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1340 ENDED ON APRIL 30, 2015. = HAH x MCMWh x AAR Where: ‘HA’ (Hours of Availability), means those hours within which a Participant shall maintain a Contracted Dispatch Period to be available for potential Curtailment of that Participant’s Monthly Contracted MW. ‘MCMW’ (Monthly Contracted MW), means the MW of demand reduction capacity for a specific Contract Month as identified in one or more DR3 Contact Schedule(s). ‘AAR’ (Adjusted Availability Rate), means an amount equal to the Availability Rate, expressed in $/MWh, as increased by the Availability Premium or as decreased by the Availability Discount, as the case may be. ‘H’ is the total hours a Participant is available in a Contract Month. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1341 | On behalf of the former OPA for the DR3 Program – Availability Over-Delivery Settlement Amt |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1341 ENDED ON APRIL 30, 2015. = H (CMWh – MCMWh) x AODRh Applicable only in response to an open standby notification. Where: ‘CMW’ (Confirmed MW), means the number of MW available for Curtailment by the Participant. ‘CMW’ is limited to the lesser of the Monthly Contracted MW plus 15 MW and 130% of the Monthly Contracted MW. ‘MCMW’ has the same meaning as in CT1340. ‘AODR’ (Availability Over-Delivery Rate), means the over-delivery rate as specified by the OPA. ‘H’ is the set of all hours ‘h’ in the Contract month where the ‘CMW’ exceeded the ‘MCMW’. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1342 | On behalf of the former OPA for the DR3 Program – Availability Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1342 ENDED ON APRIL 30, 2015. The charge to a DR participant is highest of A, B or C: A: Availability Set-Off (Reliability) = H PSOh x AAR x MCMWh This formula applies when the Reliability Rate for a given Settlement Point is less than 85% during any meter interval of an Activation Hour, or where the Participant is not Fully Available for Curtailment as defined in the OPA DR3 Program Rules. Where: For each metered interval, the Reliability Rate at a settlement point is defined as the actual reduction divided by the requested reduction; however, the Reliability Rate cannot exceed 100%. ‘PSO’ (Performance Set-Off Factor) refers to a set of factors defined in the OPA DR3 Program Rules. ‘AAR’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. ‘H’ is the set of all activation hours ‘h’ for the activation period. B: Availability Set-Off (Timely Confirmation) = PSO x AAR x MCMWh x CDP This formula applies when the Participant, regardless of Activation, has failed to deliver, or delivers late, a Confirmation that is required by the IESO pursuant to the DR3 Program Rules. Where: ‘CDP’ (Contracted Dispatch Period) means four consecutive hours. Each Contracted Dispatch Period shall occur within the hours of Availability, and shall occur within and no more than once in accordance with the Daily Schedule. ‘PSO’ has the same meaning as defined above. ‘AAR’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. C: Availability Set-Off (Low Confirmation) = H (PSO x AAR x (MCMWh – CMW) This formula applies when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for a Confirmed Hour of the Contracted Dispatch Period. Where: ‘PSO’ has the same meaning as defined above. ‘AAR’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. ‘CMW’ has the same meaning as in CT1341. ‘H’ is the set of all confirmed hours ‘h’ when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for the Contracted Dispatch Period. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1343 | On behalf of the former OPA for the DR3 Program – Utilization Payment Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1343 ENDED ON APRIL 30, 2015. = [H (Curth × URh)] – [H (NGh x MIN(HOEP, URh))] Where: ‘Curt’ (Curtailment), means the number of MWh Curtailed by a Participant when requested by the IESO, as measured through the use of electricity meter(s). Curtailment shall not exceed the product of the Activation MW and the activation period requested by the IESO, plus the lesser of an additional 15% of the Activation MW per hour of the activation period, OR 15 MWh per hour of the activation period. ‘UR’ (Utilization Rate), means the rates, expressed in $/MWh, as specified by the OPA. ‘NG’ (Net Generation), means the MWh of net electricity generated by any contributor that is a behind the meter generator. ‘H’ is the total hours ‘h’ a Participant is activated in a Contract Month. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1344 | On behalf of the former OPA for the DR3 Program – Utilization Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1344 ENDED ON APRIL 30, 2015. The charge to a DR participant is highest of A, B or C: A: Utilization Set-Off (Reliability) = H PSOh x UR x MCMWh This formula applies when the Reliability Rate for a given Settlement Point is less than 85% during any meter interval of an Activation Hour. Where: For each metered interval, the Reliability Rate at a settlement point is defined as the actual reduction divided by the requested reduction; however, the Reliability Rate cannot exceed 100%. ‘PSO’ (Performance Set-Off Factor) refers to a set of factors defined in the OPA’s Program Rules. ‘UR’ has the same meaning as in CT1343. ‘MCMW’ has the same meaning as in CT1340. ‘H’ is the set of all activation hours ‘h’ for the activation period. B: Utilization Set-Off (Timely Confirmation) = PSO x UR x MCMWh x CDP This formula applies when the Participant, regardless of Activation, has failed to deliver, or delivers late, a Confirmation that is required by the IESO pursuant to the DR3 Program Rules. Where: ‘CDP’ (Contracted Dispatch Period) means four consecutive hours. Each Contracted Dispatch Period shall occur within the hours of Availability, and shall occur within and no more than once in accordance with the Daily Schedule. ‘PSO’ has the same meaning as defined above. ‘UR’ has the same meaning as in CT1343. ‘MCMW’ has the same meaning as in CT1340 C: Utilization Set-Off (Low Confirmation) = H (PSO x UR x (MCMWh – CMW) This formula applies when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for a Confirmed Hour of the Contracted Dispatch Period. Where: ‘PSO’ has the same meaning as defined above. ‘UR’ has the same meaning as in CT1343. ‘MCMW’ has the same meaning as in CT1340. ‘CMW’ has the same meaning as in CT1341. ‘H’ is the set of all confirmed hours ‘h’ when the Confirmed MW’s are less than 95% of the Monthly Contracted MW for the Contracted Dispatch Period. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1345 | On behalf of the former OPA for the DR3 Program – Planned Non-Performance Event Set-Off Amt |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1345 ENDED ON APRIL 30, 2015. The Planned Non-Performance Availability Set-Off applies for any day for which a participant has requested a Non-Performance Event as part of either a Single Day Non-Performance Event or a part of an Extended Period Planned Non-Performance Event. The monthly set-off calculation is the sum of all: - Non-Activation Day Non-Performance Availability Set-Off s and - Activation Day Non-Performance Availability Set-Offs. For 1.) The Non-Activation Day Non-Performance Availability Set-Off amount is: = (AAR x MCMWh x HANEH) Where: ‘AAR’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. ‘HANE’ (Hours of Availability for a Non-Performance Event), represents the Hours of Availability for all days in the contract month for which a planned Non-Performance Event is requested and for which an Activation Notice is not received by the participant. For 2.) The Activation Day Non-Performance Availability Set-Off amount is: = (OH x AAR x MCMWh x NEWFH) Where: ‘OH’ (Opportunity Hours), means 64 if Option A is applicable to the Settlement Account; or 32 if Option B is applicable to the Settlement Account. ‘AAR’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. ‘NEWF’ (Non-Performance Event Weighting Factor), means 50%, if the Actual Activated MWh per interval, as averaged over all of the Intervals in the Contracted Dispatch Period for the Activation, is greater than or equal to the product of the Monthly Contracted MW and 1/12 of an hour; or 100% otherwise. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1346 | On behalf of the former OPA for the DR3 Program – Meter Data Set-Off Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1346 ENDED ON APRIL 30, 2015. = MDSF x (HAH x MCMWh x AAR) This formula applies when the complete set of weekly meter data and proof of any Forced Outage(s) for a Settlement Account is not received by 15:00 EST on the first Business Day of the following week. The formula recovers a percentage of the availability payment for the applicable week. Where: ‘MDSF’ (Meter Data Set-Off Factor), is an increasing factor for every week that the full data remains undelivered. The factor is equal to: - 20% for the first week that the full data remains undelivered; - 33% for the second week that the full data remains undelivered; - 50% for the third week that the full data remains undelivered; and - 100% for the fourth week that the full data remains undelivered. ‘HA’ has the same meaning as in CT1340. ‘MCMW’ has the same meaning as in CT1340. ‘AAR’ has the same meaning as in CT1340. ‘H’ is the total hours a Participant is available for the applicable week. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1347 | On behalf of the former OPA for the DR3 Program – Buy-Down Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1347 ENDED ON APRIL 30, 2015. Buy-Down means the act by the Participant of reducing its Monthly Contracted MW and/or removing Daily Schedules from participation in DR3. For the Buy-Down of Monthly Contracted MW the payment is: = (MCMWR x BDR x HAE) Where: ‘MCMWR’ (Monthly Contracted MW Reduction), means the MW of demand reduction in the Monthly Contracted MWs. ‘BDR’ (Buy-Down Rate), means the Buy-Down Rate, expressed in $/MW. ‘HAE’ (Hours of Availability Elapsed), means the number of Hours of Availability that have elapsed in the Schedule Term up to the date that the reduction takes effect. For the Buy-Down of the Daily Schedules the payment is: = (MCMW x RD x BDR x HAE) Where: ‘MCMW’ has the same meaning as in CT1340. ‘RD’ (Requested Days), means the number of Business Days per week from which the Hours of Availability are to be removed. ‘BDR’ has the same meaning as defined above. ‘HAE’ has the same meaning as defined above. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1348 | On behalf of the former OPA for the DR3 Program – Miscellaneous Settlement Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1348 ENDED ON APRIL 30, 2015. Reserved for DR3 payments or charges of a miscellaneous nature not specifically covered under Charge Types 1340 through 1347. | Monthly | Due DR3-participants Either way | 13 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1350 | Capacity Based Recovery Amount for Class A Loads |  | N/A |  | N/A |  | H,M,CTD * PDFk,m,d Where: ‘d’ is the ratio of the number of days in the month the Peak Demand Factor was effective compared to the total number of days in the month. ‘C’ is the set of the following charge types ‘c’: 1300, 1301, 1302, 1303, 1304, 1305, 1306, 1307,1308, 1309, 1310, 1311, 1312, 1313 and 1314 to 1320. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | See comments under charge type 147 |
| 1351 | Capacity Based Recovery Amount for Class B Loads |  | N/A |  | N/A |  | For Fort Frances Power Corporation Distribution Inc.: H,M,CTD – TD1350 ) x MAX((  HM,T AQEWk,hm,t + EGEIk - EEQ),0) / Class B Load For other Class B Market Participants and Distributors: H,M,CTD – TD1350 ) x MAX(( HM,T AQEWk,hm,t + EGEIk – GA_AQEWg,k,h,Mm,t - PGSh,M ),0) / Class B Load Where: Class B Load = (K (MAX(HM,T AQEWk,hm,t +EGEIk - EEQ - HM,T GA_AQEWg,k,h,Mm,t -  PGSh,M ,0))) -K Uk ‘H’ is the set of all settlement hours ‘h’ in the month. ‘K’ is the set of all market participants ‘k’. ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. ‘C’ is the set of the following charge types ‘c’: 1300, 1301, 1302, 1303, 1304, 1305, 1306, 1307 and 1308, 1309, 1310, 1311, 1312 and 1313 and 1314 to 1320. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | See comments under charge type 148 |
| 1380 | Demand Response 2 Availability Payment Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1380 ENDED ON FEBRUARY 28, 2015. KTDk,1330 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1330 is the settlement amount of charge type 1330 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1381 | Demand Response 2 Availability Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1381 ENDED ON FEBRUARY 28, 2015. KTDk,1331 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1331 is the settlement amount of charge type 1331 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1382 | Demand Response 2 Utilization Payment Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1382 ENDED ON FEBRUARY 28, 2015. KTDk,1332 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1332 is the settlement amount of charge type 1332 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1383 | Demand Response 2 Utilization Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1383 ENDED ON FEBRUARY 28, 2015. KTDk,1333 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1333 is the settlement amount of charge type 1333 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1384 | Demand Response 2 Meter Data Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1384 ENDED ON FEBRUARY 28, 2015. KTDk,1334 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1334 is the settlement amount of charge type 1334 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1385 | Demand Response 2 Buy-Down Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1385 ENDED ON FEBRUARY 28, 2015. KTDk,1335 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1335 is the settlement amount of charge type 1335 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1386 | Demand Response 2 Miscellaneous Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1386 ENDED ON FEBRUARY 28, 2015. KTDk,1336 Where ‘K’ is the set of all DR2 participants ‘k’. Where TDk,1336 is the settlement amount of charge type 1336 for the month for DR2 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR2 Contract. The DR2 program was last settled on the February 2015 settlement statements and invoice. |
| 1390 | Demand Response 3 Availability Payment Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1390 ENDED ON APRIL 30, 2015. KTDk,1340 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1340 is the settlement amount of charge type 1340 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1391 | Demand Response 3 Availability Over-Delivery Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1391 ENDED ON APRIL 30, 2015. KTDk,1341 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1341 is the settlement amount of charge type 1341 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1392 | Demand Response 3 Availability Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1392 ENDED ON APRIL 30, 2015. KTDk,1342 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1342 is the settlement amount of charge type 1342 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1393 | Demand Response 3 Utilization Payment Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1393 ENDED ON APRIL 30, 2015. KTDk,1343 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1343 is the settlement amount of charge type 1343 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1394 | Demand Response 3 Utilization Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1394 ENDED ON APRIL 30, 2015. KTDk,1344 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1344 is the settlement amount of charge type 1344 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1395 | Demand Response 3 Planned Non-Event Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1395 ENDED ON APRIL 30, 2015. KTDk,1345 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1345 is the settlement amount of charge type 1345 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1396 | Demand Response 3 Meter Data Set-Off Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1396 ENDED ON APRIL 30, 2015. KTDk,1346 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1346 is the settlement amount of charge type 1346 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1397 | Demand Response 3 Buy-Down Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1397 ENDED ON APRIL 30, 2015. KTDk,1347 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1347 is the settlement amount of charge type 1347 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1398 | Demand Response 3 Miscellaneous Balancing Amount |  | N/A |  | N/A |  | **CALCULATIONS FOR CHARGE TYPE 1398 ENDED ON APRIL 30, 2015. KTDk,1348 Where ‘K’ is the set of all DR3 participants ‘k’. Where TDk,1348 is the settlement amount of charge type 1348 for the month for DR3 participant ‘k’. | Monthly | Due OPA | 0 | N/A | N/A | N/A |  |  | Former OPA DR3 Contract. The DR3 program was last settled on the April 2015 settlement statements and invoice. |
| 1400 | OPA Contract Adjustment Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by the former OPA via On-line settlement form “Global Adjustment Amount Information”, subject to Regulation. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation |
| 1401 | Incremental Loss Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1402 | Hourly Condense System Constraints Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1403 | Speed-no-load Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1404 | Condense Unit Start-up and OM&A Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1405 | Hourly Condense Energy Costs Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1406 | Monthly Condense Energy Costs Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1407 | Condense Transmission Tariff Reimbursement Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1408 | Condense Availability Cost Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1409 | Monthly Condense System Constraints Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1410 | Renewable Energy Standard Offer Program Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via On-line settlement forms: “Licenced Distributor Claims for the Renewable Energy Standard Offer Program” and “Embedded Distributor Claims for the Renewable Energy Standard Offer Program”. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1411 | Clean Energy Standard Offer Program Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via future On-line settlement form “Clean Energy Standard Offer Program”. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1412 | Feed-In Tariff Program Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via On-line settlement form “Feed-In Tariff Program”. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1413 | Renewable Generation Connection – Monthly Compensation Settlement Credit |  | N/A |  | N/A |  | Manual entry based on the values submitted by the OEB. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  | Recipients, compensation amounts and other implementation details subject to OEB regulation. |
| 1414 | Hydroelectric Contract Initiative Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by the market participant. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1415 | Conservation Assessment Recovery |  | N/A |  | N/A |  | H,M, TD x ( HM,T AQEWk,hm,t / (K,HM,T AQEWk,hm,t) Where ‘H’ is the set of all settlement hours ‘h’ in the year 2009. Where ‘K’ is the set of all non-LDC load market participants ‘k’. Where ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. Where ‘TD’ equals the value assessed by the OEB. | Monthly | Due Non-LDC Load | 13 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation. |
| 1416 | Conservation and Demand Management – Compensation Settlement Credit |  | N/A |  | N/A |  | Manual entry based on the values submitted by the OEB and/or as stipulated by contracts held with the IESO. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1417 | Daily Condense Energy Costs Settlement Credit |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1418 | Biomass Non-Utility Generation Contracts Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via Online IESO. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1419 | Energy from Waste (EFW) Contracts Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via Online IESO. | Monthly | Due LDCs Either way | 13 | N/A | N/A | N/A |  |  |  |
| 1420 | Ontario Electricity Support Program Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via Online IESO | Monthly | Due LDCs, USMPs and service providers | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 314/15 |
| 1421 | Capacity Agreement Settlement Credit |  | N/A |  | N/A |  | Calculated as per capacity contracts. | Monthly | Either way | 13 | 13 | N/A | 13 |  |  |  |
| 1422 | Capacity Agreement Penalty Settlement Amount |  | N/A |  | N/A |  | Calculated as per capacity contracts. | Monthly | Either way | 13 | 13 | N/A | 13 |  |  |  |
| 1423 | Energy Sales Agreement Settlement Credit |  | N/A |  | N/A |  | Calculated as per energy sales contracts. | Monthly | Either way | 13 | 13 | N/A | 13 |  |  |  |
| 1424 | Energy Sales Agreement Penalty Settlement Amount |  | N/A |  | N/A |  | Calculated as per energy sales contracts. | Monthly | Either way | 13 | 13 | N/A | 13 |  |  |  |
| 1425 | Hydroelectric Standard Offer Program Settlement Amount |  | N/A |  | N/A |  | Manual Entry. | Monthly | Due LDCs either way | 13 | N/A | N/A | N/A |  |  |  |
| 1427 | Non-Hydro Renewables Funding Amount |  | N/A |  | N/A |  | Manual entry as per Ontario Transfer Payment Agreement. | Monthly | Due IESO | 13 | N/A | N/A | N/A | January 1, 2021 | March 31, 2022 | Ontario Regulation 735/20 |
| 1450 | OPA Contract Adjustment Balancing Amount |  | N/A |  | N/A |  | TD1400 | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government regulation |
| 1451 | Incremental Loss Offset Settlement Amount |  | N/A |  | 9.4.2.4 |  | Calculated as per ancillary service contracts. | Hourly | Due IESO | 13 | N/A | N/A | N/A |  |  | Reactive Support and Voltage Control Service |
| 1457 | Ontario Electricity Rebate Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,9983 Where ‘K’ is the set of all market participants ‘k’. Where TDk,9983 is the settlement amount of charge type 9983 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 363/16 and 364/16 |
| 1460 | Renewable Energy Standard Offer Program Balancing Amount |  | N/A |  | N/A |  | KTDk,1410 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1410 is the total settlement amount of charge type 1410 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1461 | Clean Energy Standard Offer Program Balancing Amount |  | N/A |  | N/A |  | KTDk,1411 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1411 is the total settlement amount of charge type 1411 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1462 | Feed-In Tariff Balancing Amount |  | N/A |  | N/A |  | KTDk,1412 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1412 is the total settlement amount of charge type 1412 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1463 | Renewable Generation Connection – Monthly Compensation Settlement Debit |  | N/A |  | N/A |  | KTDk,1413 x ( HM,T AQEWk,hm,t + EGEIk) / (K,HM,T AQEWk,hm,t +KEGEIk) Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘K’ is the set of all market participants ‘k’. Where ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. Where TDk,1413 is the total settlement amount of charge type 1413 for the month for market participant ‘k’. | Monthly | Due MPs | 13 | N/A | N/A | N/A |  |  | Cost recovery implementation details set out in Ontario Regulation 330/09 |
| 1464 | Hydroelectric Contract Initiative Balancing Amount |  | N/A |  | N/A |  | KTDk,1414 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1414 is the total settlement amount of charge type 1414 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1465 | Ontario Clean Energy Benefit (-10%) Program Balancing Amount |  | N/A |  | N/A |  | KTDk,9992 Where ‘K’ is the set of all market participants ‘k’. Where TDk,9992 is the settlement amount of charge type 9992 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 495/10. |
| 1466 | Conservation and Demand Management – Compensation Balancing Amount |  | N/A |  |  |  | K TDk,1416 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1416 is the settlement amount of charge type 1416 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1467 | Ontario Rebate for Electricity Consumers (8% Provincial Rebate) Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,9982 Where ‘K’ is the set of all market participants ‘k’. Where TDk,9982 is the settlement amount of charge type 9982 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 363/16 |
| 1468 | Biomass Non-Utility Generation Contracts Balancing Amount |  | N/A |  | N/A |  | KTDk,1418 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1418 is the total settlement amount of charge type 1418 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1469 | Energy from Waste (EFW) Contracts Balancing Amount |  | N/A |  | N/A |  | KTDk,1419 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1419 is the total settlement amount of charge type 1419 for the month for market participant ‘k’ | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1470 | Ontario Electricity Support Program Balancing Amount |  | N/A |  | N/A |  | ** CHARGE TYPE 1470 REPLACED BY CHARGE TYPE 2470 EFFECTIVE FEBRUARY 1, 2018 ** H M,T(AQEWk,hm,t + EGEIk) x TP Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 314/15. TP rate subject to OEB regulation. |
| 1471 | Capacity Agreement Balancing Amount |  | N/A |  | N/A |  | KTDk,1421 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1421 is the total settlement amount of charge type 1421 for the month for market participant ‘k’ | Monthly | Either way | 0 | N/A | N/A | N/A |  |  |  |
| 1472 | Capacity Agreement Penalty Balancing Amount |  | N/A |  | N/A |  | KTDk,1422 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1422 is the total settlement amount of charge type 1422 for the month for market participant ‘k’ | Monthly | Either way | 0 | N/A | N/A | N/A |  |  |  |
| 1473 | Energy Sales Agreement Balancing Amount |  | N/A |  | N/A |  | KTDk,1423 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1423 is the total settlement amount of charge type 1423 for the month for market participant ‘k’ | Monthly | Either way | 0 | N/A | N/A | N/A |  |  |  |
| 1474 | Energy Sales Agreement Penalty Balancing Amount |  | N/A |  | N/A |  | KTDk,1424 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1424 is the total settlement amount of charge type 1424 for the month for market participant ‘k’ | Monthly | Either way | 0 | N/A | N/A | N/A |  |  |  |
| 1475 | Hydroelectric Standard Offer Program Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,1425 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1425 is the total settlement amount of charge type 1425 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A |  |  |  |
| 1477 | COVID-19 Energy Assistance Program (CEAP) Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted via the relevant on-line settlement form “COVID-19 Energy Assistance Program” for residential consumers. | Monthly | Due LDCs and USMPs | 0 | N/A | N/A | N/A |  |  | Implementation details subject to OEB order EB-2020-0186 and EB-2020-0163 |
| 1487 | Non-Hydro Renewables Funding Balancing Amount |  | N/A |  | N/A |  | TD1427 | Monthly | Due IESO | 13 | N/A | N/A | N/A | January 1, 2021 | March 31, 2022 | Ontario Regulation 735/20 |
| 1500 | Day-Ahead Production Cost Guarantee Payment – Component 1 and Component 1 Clawback |  | DA_PCG_COMP1 |  | 9.4.7D.4 |  | T (Component 1 – Component1 Clawback) Component 1: -1 x OP(EMPhm,t, MIN(DA_DQSIk,hm,t, DQSIk,hm,t, AQEIk,hm,t), DA_BE) + DA_SNLCk,hm/12 Component 1 Clawback: -1 x OP(EMPhm,t, MIN(MLPk,hm,t, AQEIk,hm,t), DA_BE) + DA_SNLCk,hm/12 Where: T is the set of metering intervals in the settlement hour h. ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. For a combustion turbine resource associated to a pseudo unit: Component 1: -1 x OP(EMPhm,t, MIN(DA_DQSIk,hm,t, DQSIk,hm,t, AQEIk,hm,t), DIPCk,hm,t) + (DA_SNLCk,hm/12) * (1 – PSTk,hp,t) Component 1 Clawback: -1 x OP(EMPhm,t, MIN(MLP_CONSk,hm,t, AQEIk,hm,t), DIPCk,hm,t) + (DA_SNLCk,hm/12) * (1 – PSTk,hp,t) For a steam turbine resource associated to a pseudo unit: Component 1: -1 x OP(EMPhm,t, MIN(DIGQk,hm,t, DQSIk,hm,t, AQEIk,hm,t), DIPCk,hm,t) + (DA_SNLCk,hm/12) * PSTk,hp,t Component 1 Clawback: -1 x OP(EMPhm,t, MIN(MLP_CONSk,hm,t, AQEIk,hm,t), DIPCk,hm,t) + (DA_SNLCk,hm/12) * PSTk,hp,t | Hourly | Either Way | 13 | N/A | N/A | N/A |  |  | Component 1 applies to Variants 1, 2 and 3. Component 1 Clawback applies to Variant 2 only. For a description of Production Cost Guarantee Variants, see Market Rules 9.4.7D.2.1 |
| 1501 | Day-Ahead Production Cost Guarantee Payment – Component 2 |  | DA_PCG_COMP2 |  | 9.4.7D.4 |  | T (XDA_BEk,hm,t – MAX(0, XBEk,hm,t)) Where: T is the set of metering intervals in the settlement hour h. XDA_BEk,hm,t = (-1) * [OP(EMPhm,t,min(DA_DQSIk,hm,t, OPCAPk,hm,t), DA_BE) – OP(EMPhm,t, min(DA_DQSIk,hm,t, OPCAPk,hm,t, max(DQSIk,hm,t, AQEIk,hm,t)), DA_BE)] XBEk,hm,t = (-1) * [OP(EMPhm,t,min(DA_DQSIk,hm,t, OPCAPk,hm,t), BE) – OP(EMPhm,t, min(DA_DQSIk,hm,t, OPCAPk,hm,t, max(DQSIk,hm,t, AQEIk,hm,t)), BE)] Where: ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. EMPhm,t = 0. For a combustion turbine and a steam turbine resources associated to a pseudo unit: DA_BE is replaced with DIPCk,hm,t. For a steam turbine resource associated to a pseudo unit: DA_DQSIk,hm,t is replaced with the DIGQk,hm,t | Hourly | Either Way | 13 | N/A | N/A | N/A |  |  | Component 2 applies to Variants 1, 2 and 3. For a description of Production Cost Guarantee Variants, see Market Rules 9.4.7D.2.1 |
| 1502 | Day-Ahead Production Cost Guarantee Payment – Component 3 and Component 3 Clawback |  | DA_PCG_COMP3 |  | 9.4.7D.4 |  | T (-1)*(Component 3 + Component 3 Clawback) Where: T is the set of metering intervals in the settlement hour h. For Component 3, the six scenarios of the possible orderings of the generator’s DA_DQSI, DQSI and MQSI are as follows: - DQSI >= MQSI >= DA_DQSI - MQSI >= DQSI >= DA_DQSI - DQSI > DA_DQSI > MQSI - MQSI > DA_DQSI > DQSI - DA_DQSI >= DQSI > MQSI - DA_DQSI >= MQSI > DQSI Component 3: Component 3 is calculated when: the CMSC for energy (TDk,h,105m,t) for the same metering interval is a value other than zero; and the mathematical sign of (DQSI-MQSI) is equal to the mathematical sign of (AQEI-MQSI). Scenario 1 and 2: 0 Scenario 3: OP(EMPhm,t, MQSIk,hm,t, BE) – MAX(OP(EMPhm,t, DA_DQSIk,hm,t, BE), OP(EMPhm,t,AQEIk,hm,t, BE)) Scenario 4: OP(EMPhm,t, DA_DQSIk,hm,t, BE) – MAX(OP(EMPhm,t, DQSIk,hm,t, BE), OP(EMPhm,t,AQEIk,hm,t, BE)) Scenario 5 and 6: TDk,h,105m,t Refer to Market Rules for a description of Scenarios 1 through 6. Component 3 Clawback: Component 3 Clawback is calculated when: the event is a constrained-on event (i.e. Scenarios 3 and 5); the minimum loading point is greater than the real-time unconstrained schedule; and Component 3 (PCG_COMP3k,hm,t) for the same interval is a value other than zero. MAX(OP(EMPhm,t, MLPk,hm,t, BE), OP(EMPhm,t,AQEIk,hm,t, BE)) – OP(EMPhm,t, MQSIk,hm,t, BE) For combustion turbine resources associated to a pseudo unit: DA_BE is replaced with DIPCk,hm,t; and MLP is replaced with MLP_CONS. For steam turbine resources associated to a pseudo unit: DA_BE is replaced with DIPCk,hm,t, MLP is replaced with MLP_CONS, and DA_DQSIk,hm,t is replaced with the DIGQk,hm,t. Where ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. | Hourly | Either Way | 13 | N/A | N/A | N/A |  |  | Component 3 applies to Variants 1, 2 and 3. Component 3 Clawback applies to Variant 2 only. For a description of Production Cost Guarantee Variants, see Market Rules 9.4.7D.2.1 |
| 1503 | Day-Ahead Production Cost Guarantee Payment – Component 4 |  | DA_PCG_COMP4 |  | 9.4.7D.4 |  | T ((-1) x [OP(PRORr1,hm,t, 30R_SQRORr1,k,hm,t, BRr1,k,hm,t ) + OP(PRORr2,hm,t, 10NS_SQRORr2,k,hm,t, BRr2,k,hm,t ) + OP(PRORr3,hm,t, 10S_SQRORr3,K,hm,t, BRr3,k,hm,t )]) Where T is the set of metering intervals in the settlement hour h. ‘OP’ is the operating profit function defined in IESO market rules Section 9.3.8B.2. r1 = 30-minute operating reserve r2 = 10-minute non-spinning operating reserve r3 = 10-minute spinning operating reserve 30R_SQRORr1,k,hm,t = MAX[0,MIN(DA_DQSIk,hm,t – MQSIk,hm,t, SQRORr1,k,hm,t)] 10NS_SQRORr2,k,hm,t = MAX[0,MIN(DA_DQSIk,hm,t – MQSIk,hm,t - 30R_SQRORr1,k,hm,t , SQRORr2,k,hm,t)] 10S_SQRORr3,k,hm,t = MAX[0,MIN(DA_DQSIk,hm,t – MQSIk,hm,t - 30R_SQRORr1,k,hm,t – 10NS_SQRORr2,k,hm,t , SQRORr3,k,hm,t)] For combustion turbine resources and steam turbine resources associated to a pseudo unit: DA_DQSIk,hm,t is replaced with the DIGQk,hm,t | Hourly | Either Way | 13 | N/A | N/A | N/A |  |  | Component 4 applies to Variants 1, 2 and 3. For a description of Production Cost Guarantee Variants, see Market Rules 9.4.7D.2.1 |
| 1504 | Day-Ahead Production Cost Guarantee Payment – Component 5 |  | DA_PCG_COMP5 |  | 9.4.7D.4 |  | If first hour of the DACP start event is not HE24, then the start-up cost is calculated as follows: Scenario 1 (achieves MLP before the 7th interval): DA_SUCk,hm Scenario 2 (achieves MLP between the 7th and 18th interval): DA_SUCk,hmDA_SUCk,hm x 1/12 x SUC_INT) Where SUC_INT is the number of 5-minute intervals between and including Interval 7 and 18 the market participant takes to achieve MLP Scenario 3 (achieves MLP after the start of the 18th interval): 0 For a combustion turbine resource associated to a pseudo unit: Scenario 1 (achieves MLP before the 7th interval): DA_SUCk,hp * (1 – PSTk,hp,t) Scenario 2 (achieves MLP between the 7th and 18th interval): DA_SUCk,hp * MLP_MF * (1 – PSTk,hp,t) Scenario 3 (achieves MLP after the start of the 18th interval): 0 Where MLP_MF = 1/12 * (12 - SUC_INT) For a steam turbine resource associated to a pseudo unit: Scenario 1 (achieves MLP before the 7th interval): DA_SUCk,hp * (PSTk,hp,t) Scenario 2 (achieves MLP between the 7th and 18th interval): DA_SUCk,hp * MLP_MF * (PSTk,hp,t) Scenario 3 (achieves MLP after the start of the 18th interval): 0. If first hour of the DACP start event is HE24 and the resource has not achieved MLP before Interval 12, then the start-up cost is calculated as follows: DA_SUCk,hm * 50% For a combustion turbine resource associated to a pseudo unit: DA_SUCk,hm * (1 – PSTk,hp,t) * 50% For a steam turbine resource associated to a pseudo unit: DA_SUCk,hm * (PSTk,hp,t) * 50% | Hourly | Due IESO | 13 | N/A | N/A | N/A |  |  | Component 5 applies to Variant 1 only. For a description of Production Cost Guarantee Variants, see Market Rules 9.4.7D.2.1 |
| 1505 | Day-Ahead Production Cost Guarantee Reversal |  |  |  | 9.4.7D.6 |  | For each DACP start event If H,C TDk,h,c < 0 Then H,C TDk,h,c Else 0 Where: 'C' is the set of the following charge types 'c' as follows: 1500, 1501, 1502, 1503, 1504 ‘H’ is the set of all settlement hours ‘h’ in the DACP start event. | Hourly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1510 | Day-Ahead Generator Withdrawal Charge |  | DA_GWC |  | 9.3.8F.2 |  | The Day-Ahead Generator Withdrawal Charge is calculated as follows: If notification of the withdrawal is received 4 or more hours prior to first withdrawal hour: i=1 n (-1) * OP([MIN(PD_EMPhm,t, EMPhm,t), MLPk,hm,t, DA_BEk,hm,t)) Where: n is the set of all metering intervals ‘t’ in settlement hour ‘h’ for the total number of hours with a committed day-ahead schedule for the DACP start event that are withdrawn If notification of the withdrawal is received less than 4 hours prior to first withdrawal hour: i=1 n (-1) * OP(EMPhm,t, MLPk,hm,t, DA_BEk,hm,t) Where: n is the set of all metering intervals ‘t’ in settlement hour ‘h’ for the total number of hours with a committed day-ahead schedule for the DACP start event that are withdrawn For resources associated to a pseudo unit, the DA_BE is replaced with DIPCk,hm,t; and the MLP is replaced with MLP_CONS. | Daily | Due IESO | 13 | N/A | N/A | N/A |  |  |  |
| 1550 | Day-Ahead Production Cost Guarantee Recovery Debit |  |  |  | 9.4.8.1.12 |  | H,c M,T TDk,h,c x [(AQEWk,hm,t + SQEWk,hi,t) / k M,T (AQEWk,hm,t + SQEWk,hi,t)] Where: 'C' is the set of the following charge types 'c' as follows: 1500, 1501, 1502, 1503, 1504, 1505 'K' is the set of all market participants 'k'. 'M' is the set of all delivery points 'm' and intertie metering points 'i'. ‘H’ is the set of all settlement hours ‘h’ in the day. ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. | Daily | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 1560 | Day-Ahead Generator Withdrawal Rebate |  |  |  | 9.4.8.2.14 |  | H,c M,T TDk,h,c x [(AQEWk,hm,t + SQEWk,hi,t) / K M,T (AQEWk,hm,t + SQEWk,hi,t)] Where: ‘c’ is charge type 1510. 'K' is the set of all market participants 'k'. 'M' is the set of all delivery points 'm' and intertie metering points 'i'. ‘H’ is the set of all settlement hours ‘h’ in the day. ‘T’ is the set of 12 metering intervals ‘t’ during settlement hour ‘h’. | Daily | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 1600 | Forecasting Service Settlement Amount |  | N/A |  | 9.1.1.2.16, 9.4.7G , 9.4.7G.1, 9.4.8.1.16, 9.6.3.17, 9.6.11.5 |  | Manual entry based on the values submitted by the forecasting entity. | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 1650 | Forecasting Service Balancing Amount |  | N/A |  | 9.1.1.2.16, 9.4.7G , 9.4.7G.1, 9.4.8.1.16, 9.6.3.17, 9.6.11.5 |  | = H,CM,T TDh,c x [(AQEWk,hm,t + SQEWk,hi,t) / k,HM,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘C’ is charge type ‘c’ 1600. Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  |  |
| 1750 | Dispute Resolution Balancing Amount (Market) |  | N/A |  | 3.2.7 and 9.6.8.5 (if applicable) |  | ∑H,cM,T TDh,(700) x [(AQEWk,hm,t + SQEWk,hi,t) / ∑k,HM,T (AQEWk,hm,t + SQEWk,hi,t)], where applicable Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due MP | 13 | N/A | 0 | 13 |  |  |  |
| 1753 | MOE - Rural and Remote Settlement Debit |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Rural or Remote Rate Protection (RRRP) – Fixed Rate Credit”; | Monthly | Due Ministry of Energy | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government and OEB regulations. |
| 2148 | Class B Global Adjustment Prior Period Correction Settlement Amount |  | N/A |  | N/A |  | Manual entry based on post-final changes to input data for charge type 148 | Monthly | Due MP | 13 | N/A | N/A | N/A |  |  |  |
| 2470 | MOE - Ontario Electricity Support Program Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,1420 Where ‘K’ is the set of all market participants ‘k’. Where TDk,1420 is the settlement amount of charge type 1420 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | 0 | N/A | N/A | N/A |  |  | Implementation details subject to government and OEB regulations. |
| 6000 | Ontario Fair Hydro Plan - Regulatory Asset Transfer Amount |  | N/A |  | N/A |  | Manual Entry | Monthly | Due Financing Entity | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 6050 | Ontario Fair Hydro Plan - Regulatory Asset Transfer Balancing Amount |  | N/A |  | N/A |  | Manual Entry | Monthly | Due IESO | N/A | N/A | N/A | N/A |  |  | Implementation details subject to government regulations |
| 6147 | Class A Global Adjustment Deferral Recovery Amount |  | N/A |  | N/A |  | MDCAA × (PDFk,m,d / ∑K PDFk,m,d ) Where ‘K’ is the set of all market participants ‘k’. | Monthly | Due IESO | 13 | N/A | N/A | N/A | January 1. 2021 | December 31, 2021 | Ontario Regulation 429/04 |
| 6148 | Class B Global Adjustment Deferral Recovery Amount |  | N/A |  | N/A |  | CBRR × CBMPk Where: CBRR = MDCBA / (Class B Load – ∑K RPPVAk) Class B Load = (ΣK,HM,T AQEWk,hm,t + ΣKEGEIk - ΣK EEQ - ΣK GA_AQEWg,k,h,Mm,t - ΣK PGSh,M - ΣK Uk) For Fort Frances Power Corporation Distribution Inc.: CBMPk = ΣHM,T AQEWk,hm,t + EGEIk – EEQ – RPPVAk For other applicable Class B market participants or licensed distributors that are also market participants : CBMPk = ΣHM,T AQEWk,hm,t + EGEIk - GA_AQEWg,k,h,Mm,t - PGSh,M –RPPVAk Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘K’ is the set of all market participants ‘k’. Where ‘M’ is the set of all delivery points ‘m’ of market participant ‘k’. | Monthly | Due IESO | 13 | N/A | N/A | N/A | January 1. 2021 | December 31, 2021 | Ontario Regulation 429/04 |
| 9147 | Class A Global Adjustment Smoothing Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,6147 Where ‘K’ is the set of all market participants ‘k’. Where TDk,6147 is the settlement amount of charge type 6147 for the month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A | April 1, 2020 | December 31, 2021 | Ontario Regulation 429/04 |
| 9148 | Class B Global Adjustment Smoothing Balancing Amount |  | N/A |  | N/A |  | ΣK TDk,6148 Where ‘K’ is the set of all market participants ‘k’. Where TDk,6148 is the settlement amount of charge type 6148 for the current month for market participant ‘k’. | Monthly | Due IESO | 0 | N/A | N/A | N/A | April 1, 2020 | December 31, 2021 | Ontario Regulation 429/04 |
| 9920 | Adjustment Account Credit |  | AAC |  | 9.6.18.6 |  | AAD x H M,T [(AQEWk,hm,t + SQEWk,hi,t) / K,H M,T (AQEWk,hm,t + SQEWk,hi,t)] Where ‘H’ is the set of all settlement hours ‘h’ in the billing periods immediately preceding the current billing period, as determined by IESO Board. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. Where ‘M’ is the set of all delivery points ‘m’ and intertie metering points ‘i’ Where ‘K’ is the set of all market participants ‘k’. | Monthly (when applicable) | Due MP | 13 | N/A | 0 | 13 |  |  | The billing period is defined in Market Manual 5: Settlements Part 5.5: Physical Markets Settlement Statements, section 1.6.30 |
| 9980 | Smart Metering Charge |  | N/A |  | N/A |  | Manual entry based on the values submitted by the Smart Metering Entity. | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  | Subject to Ontario Regulation 453/06 and the applicable OEB rate order. |
| 9982 | Ontario Rebate for Electricity Consumers (8% Provincial Rebate) Settlement Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement form “Ontario Rebate for Electricity Consumers (OREC) – LDC and USMP”; and (2) 8 per cent of the base invoice amount for market participant consumers who have an eligible account with the IESO | Monthly | Due LDCs, Unit Sub-Meter Providers and eligible MPs | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 363/16 |
| 9983 | Ontario Electricity Rebate Settlement Amount |  | N/A |  | N/A |  | Manual entry based on: (1) the values submitted via on-line settlement forms “Ontario Electricity Rebate (OER) – LDC & USMP”; and (2) 33.2 per cent of the base invoice amount for market participant consumers who have an eligible account with the IESO | Monthly | Due LDCs, Unit Sub-Meter Providers and eligible MPs | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 363/16 and 364/16 |
| 9984 | COVID-19 Energy Assistance Program (CEAP) Balancing Amount |  | N/A |  | N/A |  | ΣKTDk,1477 Where ‘K’ is the set of all market participants ‘k’ Where TDk,1477 is the settlement amount of charge type 1477 for the month for market participant ‘k’. | Monthly | Due Ministry of Energy | 0 | N/A | N/A | N/A |  |  | Implementation details subject to OEB order EB-2020-0186 and EB-2020-0163 |
| 9990 | IESO Administration Charge |  | N/A |  | 9.4.5.1 |  | H M,T(AQEWk,hm,t + SQEWk,hi,t + EGEIk) x TP Where ‘H’ is the set of all settlement hours ‘h’ in the month. Where ‘T’ is the set of all metering intervals ‘t’ in the set of all settlement hours ‘H’. | Monthly | Due IESO | 13 | N/A | 0 | 13 |  |  | TP rate subject to OEB regulation. |
| 9992 | Ontario Clean Energy Benefit (-10%) Program Settlement Amount |  | N/A |  | N/A |  | Manual entry based on the values submitted by market participants via on-line settlement forms “Ontario Clean Energy Benefit (-10%) – LDC” and “Ontario Clean Energy Benefit (-10%) – Unit Sub-Meter Provider”. | Monthly | Due LDCs and Unit Sub-Meter Providers Either way | 0 | N/A | N/A | N/A |  |  | Implementation details subject to Ontario Regulation 495/10. |
| 9996 | Recovery of Costs |  | N/A |  | Ch. 2, Appendix 3.4 |  | Manual entry as per Chapter 2, Appendix 3.4 | Monthly | Due IESO | 13 | N/A | N/A | N/A |  |  |  |

## Rounding Conventions – by Settlement Variable

### Key to the Table of Rounding Conventions for Individual Settlement Variables

| Column Name | Description |
| --- | --- |
| Variable referenced in Section 2.1 | This column provides the name of the variable listed in Section 2.1. |
| Data Description | The short name of the variable in question. |
| Number of decimal places (values published by upstream systems) | If this variable is available to market participants via another system besides settlements, this number of significant digits to the right of the decimal place in the published value. NOTE: “published” does not necessarily mean a public report or a report available to all market participants. E.g. metering data from the metering database. |
| Number of significant digits to the right of the decimal (values received by CRS) | This column discloses the accuracy of a settlement variable received by the IESO settlements system via an upstream system OR manually entered as the case may be. |
| Number of significant digits to the right of the decimal (externally passed from CRS in settlement statements or data files) | This column discloses the accuracy of a settlement variable appearing on a settlement statement. NOTE: This should NOT be confused with the number of decimal places allowable in some columns on the settlement statements and data files as set out in, “Format Specification for Settlement Statements and Data Files.” |
| Comments | Any comments as to the availability of such variables. In some cases, variables are not made available to market participants via upstream systems and are noted as such. In other instances variables are not published in a report but are communicated in participant-specific messages (e.g. bid/offer confirmation). |

| Variable referenced in Section 2.1 | Data Description | Number of DECIMAL PLACES (values published by upstream systems) | MAXIMUM Number of SIGNIFICANT DIGITS to the right of the decimal (values received by CRS) | MAXIMUM Number of SIGNIFICANT DIGITS to the right of the decimal (externally passed from CRS in settlement statements or data files) | Comments |
| --- | --- | --- | --- | --- | --- |
| AAD | Adjustment Account Disbursement | N/A | 2 | 3 | - Not published in upstream IESO systems |
| AQEIk,hm,t | Allocated Quantity of Energy Injected | 2 | 3 | 3 | - RMS presentation is in units of KW to TWO decimal places. - Unit change to MW to 3 decimal places occurs prior to transfer to CRS. |
| AQEWk,hm,t | Allocated Quantity of Energy Withdrawn | 2 | 3 | 3 | - RMS presentation is in units of KW to TWO decimal places. - Unit change to MW to 3 decimal places occurs prior to transfer to CRS. |
| AQORr,k,hm,t | Allocated Quantity of Operating Reserve | 1 | 1 | 1 | - See SQROR. |
| BE | Energy Offers | N/A | 1 | 1 | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| BL | Energy Bids | N/A | 1 | 1 | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| BRr | Operating Reserve Offers | N/A | 1 | 1 | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| BCQs,k,hm,t | Physical Bilateral Contract Quantity of Energy bought | N/A | 1 or 3 | 1 or 3 | - Not published via upstream IESO systems. - Physical Bilateral Contract Data is provided to the IESO by the selling market participant. - Accuracy driven by the submission at the MIM interface and the method used (i.e. absolute quantities vs. 100% of PBC). |
| BCQk,b,hm,t | Physical Bilateral Contract Quantity of Energy sold | N/A | 1 or 3 | 1 or 3 | - Not published via upstream IESO systems. - Physical Bilateral Contract Data is provided to the IESO by the selling market participant. - Accuracy driven by the submission at the MIM interface and the method used (i.e. absolute quantities vs. 100% of PBC). |
| CACP | Capacity Auction Clearing Price | 2 | 2 | 2 | - Published in post-auction report. |
| CACPh | Hourly Capacity Auction Clearing Price | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| CAEOk | Capacity Auction Energy Offer | N/A | 1 | 1 | - Not published via upstream IESO system |
| CBOCk | Buy-Out Capacity | N/A | 3 | 3 | - Not published via upstream IESO systems. |
| CCOk | Capacity Obligation (MW) | 1 | 3 | 3 | - Published in private post-auction report. |
| CGC | Combined Guaranteed Costs | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| CNPFm | Capacity Auction Non-Performance Factor | N/A | 1 | 1 | - Not published via upstream IESO systems. |
| DA_BEk,hi,t | Energy Offer submitted into the schedule of record | N/A | N/A | N/A | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| DA_BEk,hm,t | Energy Offer submitted into the schedule of record at a delivery point | N/A | N/A | N/A | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| DA_BLk,hi,t | Energy Bids submitted into the schedule of record | N/A | N/A | N/A | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| DA_DQSIk,hi,t | Schedule of record dispatch quantity scheduled for injection at an intertie metering point | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_DQSIk,hm,t | Schedule of record dispatch quantity scheduled for injection at a delivery point | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_DQSWk,hi,t | Schedule of record dispatch quantity scheduled for withdrawal at an intertie metering point | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_ELMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the export zone | 2 | 2 | 2 | - MIM Publication. |
| DA_ILMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the import zone | 2 | 2 | 2 | - MIM Publication. |
| DA_SNLCk,hm | Speed-no-load costs submitted into the schedule of record | 1 | 2 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_SNLCk,hp | Speed-no-load costs for pseudo units submitted into the schedule of record | 1 | 2 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_SUCk,hm | Start-up costs submitted into the schedule of record | 1 | 2 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DA_SUCk,hp | Start-up costs for pseudo units submitted into the schedule of record | 1 | 2 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DIPCk,hm,t | Derived Interval Price Curve | 1 | 2 | 1 | - Derived price curve and therefore not published on settlement statements. |
| DIGQk,hm,t | Derived Interval Guaranteed Quantity | 1 | 1 | 1 | - Derived schedule quantity and therefore not published on settlement statements. |
| DQSIk,hm,t | Dispatch Quantity of Energy Scheduled for Injection | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DQSRr,k,hm,t | Dispatch Quantity Schedule of Operating Reserve | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DQSWk,hm,t | Dispatch Quantity of Energy Scheduled for Withdrawal | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| DRACP | Demand Response Auction Clearing Price | 2 | 2 | 2 | - Published in post-auction report. |
| DRACPh | Hourly Demand Response Auction Clearing Price | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| DRBOCk | Demand Response Buy-Out Capacity | N/A | 3 | 3 | - Not published via upstream IESO systems. |
| DRCOk | Demand Response Capacity Obligation (MW) | 1 | 3 | 3 | - Published in private post-auction report. |
| DREBQk | Demand Response Energy Bid Quantity | N/A | 1 | 1 | - Not published via upstream IESO systems. |
| DRNPF | Demand Response Non-Performance Factor | N/A | 1 | 1 | - Not published via upstream IESO systems. |
| DRSQty | Demand Response Scheduled Quantity | N/A | 1 | 1 | - Not published via upstream IESO systems. |
| EEQ | Excluded Energy Quantity | N/A | 3 | 3 | - Not published via upstream IESO systems. |
| EGEIk | Embedded Generator Energy Injection | N/A | 3 | 3 | - Not published via upstream IESO systems. |
| EIMk,h | Operating Profit Function for the IMPORT of Energy under the Intertie Offer/Bid Guarantee Settlement Credit | N/A See Section 2.4 | N/A See Section 2.4 | N/A See Section 2.4 | - This acronym is associated with the energy import component of the Intertie Offer/Bid Guarantee Settlement Credit. |
| EMPhi,t | 5-minute Energy Market Price at the Interties | 2 | 2 | 2 | - MIM Publication. |
| EMPhm,t | 5-minute Energy Market Price within Ontario | 2 | 2 | 2 | - MIM Publication. |
| EMPhREF,t | 5-minute Energy Market Reference Price | 2 | 2 | 2 | - MIM Publication. |
| ETS | Export Transmission Service Tariff Rate | N/A | 2 | 2 | - Not published via upstream IESO systems. - Subject to the OEB “Ontario Transmission Rate Order”. |
| FPhm | Fixed Energy Rate | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| FPChm | Rate for a designated group of charge types (see description of charge type 141)) | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| GRP | Generator Regulated Price | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| HDRBPh | HDR bid price | N/A | 1 | 1 | - Not published via upstream IESO systems. |
| HDRDC | Measured hourly demand response capacity | N/A | 3 | 3 | - Not published via upstream IESO systems. |
| HDRTAPR | Out of market test activation payment rate | N/A | N/A | N/A | - Not published via upstream IESO systems - Fixed rate as defined in this document |
| HOEPh | Hourly Ontario Energy Price | 2 | 2 | 2 | - MIM Publication. |
| LCDk,hm | Line Connection Demand (KW) | 2 and 3 | 3 | 3 | - RMS presentation is in units of KW to 2 decimal places. - Unit changes to MW to 3 decimal places prior to transfer to the Transmission Tariff Demand Calculator (TTDC). - Unit changes to KW to 3 decimal places prior to transfer to CRS. |
| MChm | Minimum Consumption | 1 | 1 | 1 |  |
| MI | Ordered matrix of and corresponding IOG settlement amounts | 1 and 2 | 2 | 2 | - Derived set of variables and therefore not published on settlement statements. |
| MLPk,hm,t | Minimum Loading Point | 1 | 1 | 1 | - Not published via upstream IESO systems. |
| MLP_CONSk,hm,t | Minimum Loading Point for a steam turbine resource or a combustion turbine resource associated to a pseudo unit | 1 | 1 | 1 | - Not published via upstream IESO systems. |
| MQSIk,hm,t | Market Quantity Scheduled for Injection | 1 | 1 | 1 |  |
| MQSI{adj}k,hm,t | Adjusted Market Quantity Scheduled for Injection | 1 | 1 | 1 | - Derived variable and therefore not published on settlement statements. |
| MQSWk,hm,t | Market Quantity Scheduled for Withdrawal | 1 | 1 | 1 |  |
| NSDk,hm | Network Service Demand (KW) | 2 and 3 | 3 | 3 | - RMS presentation is in units of KW to 2 decimal places. - Unit changes to MW to 3 decimal places prior to transfer to the Transmission Tariff Demand Calculator (TTDC). - Unit changes to KW to 3 decimal places prior to transfer to CRS. |
| OP | Operating Profit Function | N/A See Section 2.4 | N/A See Section 2.4 | N/A See Section 2.4 | - This acronym is associated with the operating profit equation used within the CMSC equation. |
| OPCAPk,hm,t | Operating Capacity | 1 | 1 | 1 | - Not published via upstream IESO systems. |
| PB_IMht | Price bias adjustment factor for import transactions | 2 | 2 | 2 | - Published on by the IESO on a periodic basis. |
| PB_EXht | Price bias adjustment factor for export transactions | 2 | 2 | 2 | - Published on by the IESO on a periodic basis. |
| PD_BEk,hi,t | Energy Offer submitted into the Pre-dispatch | N/A | 1 | 1 | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| PD_BLk,hi,t | Energy bids submitted into the Pre-dispatch | N/A | 1 | 1 | - Not published via upstream IESO systems. - Confirmations passed to market participants as bids/offers (“dispatch data”) are received. |
| PD_DQSIk,hi,t | Pre-dispatch quantity scheduled for injection at an intertie metering point | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| PD_DQSWk,hi,t | Pre-dispatch quantity scheduled for withdrawal at an intertie metering point | 1 | 1 | 1 | - Not published via upstream IESO systems. - Passed to market participants via dispatch messaging. |
| PD_ELMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the export zone | 2 | 2 | 2 | - MIM Publication. |
| PD_EMPhm,t | Pre-dispatch energy market price for Ontario | 2 | 2 | 2 | - MIM Publication. |
| PD_ILMPhm,t | Pre-dispatch constrained schedule price for an intertie metering point in the import zone | 2 | 2 | 2 | - MIM Publication. |
| PRORr,hm,t | 5-minute Operating Reserve Price | 2 | 2 | 5 | - MIM Publication. |
| PSTk,hp,t | Steam Turbine Portion from Daily Generator Data | 1 | 1 | 1 | - Not published via upstream IESO systems. |
| PTS-L | Provincial Transmission Service Line Connection Service Rate ($/KW) | N/A | 2 | 2 | - Not published via upstream IESO systems. - Subject to the OEB “Ontario Transmission Rate Order”. |
| PTS-N | Provincial Transmission Service Network Service Rate ($/KW) | N/A | 2 | 2 | - Not published via upstream IESO systems. - Subject to the OEB “Ontario Transmission Rate Order”. |
| PTS-T | Provincial Transmission Service Transformation Connection Service Rate ($/KW) | N/A | 2 | 2 | - Not published via upstream IESO systems. - Subject to the OEB “Ontario Transmission Rate Order”. |
| QTRk,hi,j | Quantity of Transmission Rights Owned | PENDING | 0 | 0 | - TR’s are in denominations to the nearest MW. - Upstream publication accuracy currently being resolved. |
| SQEIk,hi,t | Scheduled Quantity of Energy Injected at an intertie metering point | 1 | 1 | 1 |  |
| SQEWk,hi,t | Scheduled Quantity of Energy Withdrawn at an intertie metering point | 1 | 1 | 1 |  |
| SQRORr,k,hm,t | Scheduled Quantity of class r Operating Reserve | 1 | 1 | 1 |  |
| TCDk,hm | Transformation Connection Demand (KW) | 2 and 3 | 3 | 3 | - RMS presentation is in units of KW to 2 decimal places. - Unit changes to MW to 3 decimal places prior to transfer to the Transmission Tariff Demand Calculator (TTDC). - Unit changes to KW to 3 decimal places prior to transfer to CRS. |
| TDk,h,c | Total Market Settlement Amount | N/A | N/A | N/A | - N/A- notational description of an aggregated financial amount (reported to the nearest cent when applicable). |
| TPc | Tariff price | N/A | N/A | N/A | - N/A – notational description of tariff rate (reported to the nearest cent when applicable). |
| TRMP | TR Market Clearing Price | 2 | 2 | 2 |  |
| TRCAD | TR Clearing Account Disbursements | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| TRCADE | TR Clearing Account Disbursements for Exporters | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| TRCADL | TR Clearing Account Disbursements for Loads | N/A | 2 | 2 | - Not published via upstream IESO systems. |
| TRCAR | TR Shortfall Recovery Amount | N/A | 2 | 2 | - Not published via upstream IESO systems. |

## Rounding Conventions – by Charge Type

### General Notes

- The table below references significant digits to the right of the decimal place.  This should NOT be confused with the number of decimal places allowable in some columns on the settlement statements and data files as set out in, “Format Specification for Settlement Statements and Data Files.”

- All settlement amounts reported by the IESO settlements system are rounded to the nearest cent (i.e. to two decimal places) on settlement statements, although some settlement calculations may only yield 1 significant digit to the right of the decimal place.  In these instances, the financial amount is NOT further rounded to the nearest ten cents.  The table below does not include the final rounding step to the nearest cent, as this is done for ALL settlement amounts.  Rather, it describes any intermediate calculations (particularly, those involving division) that involve rounding prior to the final calculation of the settlement amount.

### Key to the Table of Rounding Conventions

| Column Name | Description |
| --- | --- |
| Charge Type Number | This table contains an entry for each charge type listed in Section 2.2 of this document (“IESO Charge Types and Equations”). |
| Charge Type Name | The name of each of the charge types. |
| INPUT VARIABLES Least number of significant digits to the right of the decimal | In terms of assessing the accuracy of the final settlement amount, this column is derived from the settlement variable received by the settlement system with the LEAST number of significant digits to the right of the decimal place. |
| INPUT VARIABLES Maximum number of significant digits to the right of the decimal | In terms of assessing the accuracy of the final settlement amount, this column is derived from the settlement variable received by the settlement system with the MAXIMUM number of significant digits to the right of the decimal place. |
| INTERMEDIATE Rounding done by Settlements | This column indicates whether or not any INTERMEDIATE rounding is done by the IESO settlement process. This does NOT include the final rounding of settlement amounts to 2 decimal places as the last step in the calculation of ALL charge types. |
| INTERMEDIATE CALCULATION 1 (where intermediate rounding occurs) | This column ONLY describes an intermediate calculation of the settlement amount in which rounding occurs PRIOR to the final rounding of the settlement amount to the nearest cent. |
| Disposition of INTERMEDIATE CALCULATION 1 | This column describes the disposition of the rounded value resulting from Intermediate Calculation 1. |
| INTERMEDIATE CALCULATION 2 (where intermediate rounding occurs) | This column ONLY describes an intermediate calculation of the settlement amount in which rounding occurs PRIOR to the final rounding of the settlement amount to the nearest cent. |
| Disposition of INTERMEDIATE CALCULATION 2 | This column describes the disposition of the rounded value resulting from Intermediate Calculation 2. |

| Charge Type Number | Charge Type Name | INPUT VARIABLES Least number of significant digits to the right of the decimal | INPUT VARIABLES Maximum number of significant digits to the right of the decimal | Intermediate Rounding done by Settlements? | INTERMEDIATE CALCULATION 1 (where intermediate rounding occurs) | DISPOSITION OF INTERMEDIATE CALCULATION 1 | INTERMEDIATE CALCULATION 2 (where intermediate rounding occurs) | DISPOSITION OF INTERMEDIATE CALCULATION 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52 | Transmission Rights Auction Settlement Debit | 0 | 2 | No |  |  |  |  |
| 100 | Net Energy Market Settlement for Generators and Dispatchable Load | 1 | 3 | Yes | Numerator: BCQ Denominator: 12 Resulting Decimals: 3 | BCQ quantities Multiplied by EMP when applicable. |  |  |
| 101 | Net Energy Market Settlement for Non-dispatchable Load | 1 | 3 | Yes | Numerator: BCQ Denominator: 12 Resulting Decimals: 3 | BCQ quantities Multiplied by EMP when applicable. |  |  |
| 102 | TR Clearing Account Credit | 1 | 3 | No |  |  |  |  |
| 103 | Transmission Charge Reduction Fund | 2 | 3 | Yes | Numerator: Difference between SQEW – SQEI by intertie zone Denominator: 12 Resulting Decimals: 3 | Resulting value included with the TCRF calculation at that particular zone for the metering interval in question. |  |  |
| 104 | Transmission Rights Settlement Credit | 0 | 2 | Yes | Numerator: Summation of the zonal price difference (EMPhj,t – EMP hi,t) Denominator: 12 Resulting Decimals: 5 | Multiplied by QTR for the settlement hour. |  |  |
| 105 | Congestion Management Settlement Credit for Energy | 1 | 3 | Yes | AQEI multiplied by 12 or AQEW multiplied by 12 Resulting Decimals: 3 | Used in the calculation of OP(EMP, AQEI, BE) or OP(EMP, AQEW, BL) as the case may be. | Numerators OP(EMP, MQSI, BE) OP(EMP, DQSI, BE) OP(EMP, AQEI, BE) OP(EMP, MQSW, BL) OP(EMP, DQSW, BL) OP(EMP, AQEW, BL) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |
| 106 | Congestion Management Settlement Credit for 10 Minute Spinning Reserve | 1 | 2 | Yes | Numerators OP(PROR, MQSR, BR) OP(PROR, DQSR, BR) OP(PROR, AQOR, BR) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |  |  |
| 107 | Congestion Management Settlement Credit for 10 Minute Non-spinning Reserve | 1 | 2 | Yes | Numerators OP(PROR, MQSR, BR) OP(PROR, DQSR, BR) OP(PROR, AQOR, BR) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |  |  |
| 108 | Congestion Management Settlement Credit for 30 Minute Operating Reserve | 1 | 2 | Yes | Numerators OP(PROR, MQSR, BR) OP(PROR, DQSR, BR) OP(PROR, AQOR, BR) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |  |  |
| 111 | Northern Pulp and Paper Mill Electricity Transition Program Settlement Amount | 1 | 3 | No |  |  |  |  |
| 112 | Ontario Power Generation Rebate | 2 | 3 | No |  |  |  |  |
| 113 | Additional Compensation for Administrative Pricing Credit | 1 | 3 | Yes | For the calculation outlined in 7.8.4A.16 only: for dispatchable facilities located within Ontario only AQEI multiplied by 12 or AQEW multiplied by 12 Resulting Decimals: 3 | (For the calculation outlined in 7.8.4A.16 only) For dispatchable facilities located within Ontario only: Used in the calculation of OP(EMP, AQEI, BE) or OP(EMP, AQEW, BL) as the case may be. | For the calculation outlined in 7.8.4A.16 only: Numerators: for dispatchable facilities located within Ontario: OP(EMP, AQEI, BE) OP(EMP, AQEW, BL) for Imports or Exports: OP(EMP, DQSI, BE) OP(EMP, DQSW, BL) Denominator: 12 Resulting Decimals: 2 | (For the calculation outlined in 7.8.4A.16 only) The results are used in the final calculation |
| 114 | Outage Cancellation/ Deferral Settlement Credit | 2 | 2 | No |  |  |  |  |
| 115 | Unrecoverable Testing Costs Credit | 2 | 2 | No |  |  |  |  |
| 116 | Tieline Maintenance Reliability Credit | 2 | 2 | No |  |  |  |  |
| 118 | Emergency Energy Rebate | 1 | 3 | No |  |  |  |  |
| 119 | Station Service Reimbursement Credit | 2 | 2 | No |  |  |  |  |
| 120 | Local Market Power Debit | 2 | 2 | No |  |  |  |  |
| 121 | Northern Industrial Electricity Rate Program Settlement Amount | 1 | 3 | No |  |  |  |  |
| 122 | Ramp Down Settlement Amount | 1 | 3 | Yes | AQEI multiplied by 12 or AQEW multiplied by 12 Resulting Decimals: 3 | Used in the calculation of OP(EMP, AQEI, BE) or OP(EMP, AQEW, BL) as the case may be. | Numerators OP(EMP, MQSI, BE) OP(EMP, DQSI, BE) OP(EMP, AQEI, BE) OP(EMP, MQSW, BL) OP(EMP, DQSW, BL) OP(EMP, AQEW, BL) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |
| 123 | MACD Enforcement Activity Amount | 2 | 2 | No |  |  |  |  |
| 124 | SEAL Congestion Management Settlement Credit Amount | 2 | 2 | No |  |  |  |  |
| 130 | Intertie Offer Settlement Credit – Energy | 1 | 3 | Yes | Numerators OP(EMP, MQSI, BE) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |  |  |
| 133 | Generator Cost Guarantee Payment | 1 | 3 | No |  |  |  |  |
| 134 | Demand Response Credit | 2 | 2 | No |  |  |  |  |
| 135 | Real-time Import Failure Charge | 1 | 3 | Yes | TERM 1 – Failure Charge Numerator: EMP + PB_IM – PD_EMP Denominator: 12 Resulting Decimals: 2 TERM 2 – Price Cap Numerator: MAX(0,EMP) * RT_ISD Denominator: 12 Resulting Decimals: 2 | TERM 1 and TERM 2 compared as applicable. |  |  |
| 136 | Real-time Export Failure Charge | 1 | 3 | Yes | TERM 1 – Failure Charge Numerator: PD_EMP – EMP – PB_EX Denominator: 12 Resulting Decimals: 2 TERM 2 – Price Cap Numerator: MAX(0,PD_EMP) * RT_ESD Denominator: 12 Resulting Decimals: 2 | TERM 1 and TERM 2 compared as applicable. |  |  |
| 137 | Generation Cost Guarantee - Output Based Pricing System Reimbursement Settlement Amount | 1 | 3 | No |  |  |  |  |
| 140 | Fixed Energy Rate Settlement Amount | 1 | 3 | No |  |  |  |  |
| 141 | Fixed Wholesale Charge Rate Settlement Amount | 1 | 3 | No |  |  |  |  |
| 142 | Regulated Price Plan Settlement Amount | 1 | 3 | No |  |  |  |  |
| 143 | NUG Contract Adjustment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 144 | Regulated Nuclear Generation Adjustment Amount | 1 | 3 | No |  |  |  |  |
| 145 | Regulated Hydroelectric Generation Adjustment Amount | 1 | 3 | No |  |  |  |  |
| 146 | Global Adjustment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 147 | Class A – Global Adjustment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 148 | Class B – Global Adjustment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 149 | Regulated Price Plan Retailer Settlement Amount | 1 | 3 | No |  |  |  |  |
| 150 | Net Energy Market Settlement Uplift | 1 | 3 | No |  |  |  |  |
| 155 | Congestion Management Settlement Uplift | 1 | 3 | No |  |  |  |  |
| 161` | Northern Pulp and Paper Mill Electricity Transition Program Balancing Amount | 1 | 3 | No |  |  |  |  |
| 162 | Ontario Power Generation Rebate Debit | 1 | 3 | No |  |  |  |  |
| 163 | Additional Compensation for Administrative Pricing Debit | 1 | 3 | No |  |  |  |  |
| 164 | Outage Cancellation/ Deferral Debit | 1 | 3 | No |  |  |  |  |
| 165 | Unrecoverable Testing Costs Debit | 1 | 3 | No |  |  |  |  |
| 166 | Tieline Reliability Maintenance Debit | 1 | 3 | No |  |  |  |  |
| 167 | Emergency Energy and EDRP Debit | 1 | 3 | No |  |  |  |  |
| 168 | TR Market Shortfall Debit | 1 | 3 | No |  |  |  |  |
| 169 | Station Service Reimbursement Debit | 1 | 3 | No |  |  |  |  |
| 170 | Local Market Power Rebate | 1 | 3 | No |  |  |  |  |
| 171 | Northern Industrial Electricity Rate Program Balancing Amount | 1 | 3 | No |  |  |  |  |
| 173 | MACD Enforcement Activity Balancing Amount | 2 | 2 | No |  |  |  |  |
| 183 | Generator Cost Guarantee Recovery Debit | 1 | 3 | No |  |  |  |  |
| 184 | Demand Response Debit | 2 | 2 | No |  |  |  |  |
| 186 | Intertie Failure Charge Rebate | 1 | 3 | No |  |  |  |  |
| 190 | Fixed Energy Rate Balancing Amount | 2 | 2 | No |  |  |  |  |
| 191 | Fixed Wholesale Charge Rate Balancing Amount | 2 | 2 | No |  |  |  |  |
| 192 | Regulated Price Plan Balancing Amount | 2 | 2 | No |  |  |  |  |
| 193 | NUG Contract Adjustment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 194 | Regulated Nuclear Generation Balancing Amount | 2 | 2 | No |  |  |  |  |
| 195 | Regulated Hydroelectric Generation Balancing Amount | 2 | 2 | No |  |  |  |  |
| 196 | Global Adjustment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 197 | Global Adjustment-Special Programs Balancing Amount | 2 | 2 | No |  |  |  |  |
| 198 | Renewable Generation Balancing Amount | 2 | 2 | No |  |  |  |  |
| 199 | Regulated Price Plan Retailer Balancing Amount | 2 | 2 | No |  |  |  |  |
| 200 | 10 Minute Spinning Reserve Market Settlement Credit. | 1 | 2 | No |  |  |  |  |
| 201 | 10 Minute Spinning Reserve Market Shortfall Rebate | 1 | 3 | No |  |  |  |  |
| 202 | 10 Minute Non-spinning Reserve Market Settlement Credit | 1 | 2 | No |  |  |  |  |
| 203 | 10 Minute Non-spinning Reserve Market Shortfall Rebate | 1 | 3 | No |  |  |  |  |
| 204 | 30 Minute Operating Reserve Market Settlement Credit | 1 | 2 | No |  |  |  |  |
| 205 | 30 Minute Operating Reserve Market Shortfall Rebate | 1 | 3 | No |  |  |  |  |
| 250 | 10 Minute Spinning Market Reserve Hourly Uplift | 1 | 3 | No |  |  |  |  |
| 251 | 10 Minute Spinning Market Reserve Shortfall Debit | 1 | 3 | No |  |  |  |  |
| 252 | 10 Minute Non-spinning Market Reserve Hourly Uplift | 1 | 3 | No |  |  |  |  |
| 253 | 10 Minute Non-spinning Market Reserve Shortfall Debit | 1 | 3 | No |  |  |  |  |
| 254 | 30 Minute Operating Reserve Market Hourly Uplift | 1 | 3 | No |  |  |  |  |
| 255 | 30 Minute Operating Reserve Market Shortfall Debit | 1 | 3 | No |  |  |  |  |
| 400 | Black Start Capability Settlement Credit | 2 | 2 | No |  |  |  |  |
| 404 | Regulation Service Settlement Credit | 2 | 2 | No |  |  |  |  |
| 406 | Emergency Demand Response Credit | 2 | 2 | No |  |  |  |  |
| 410 | IESO-Controlled Grid Special Operations Credit | 2 | 2 | No |  |  |  |  |
| 450 | Black Start Capability Settlement Debit | 1 | 3 | No |  |  |  |  |
| 451 | Hourly Reactive Support and Voltage Control Settlement Debit | 1 | 3 | No |  |  |  |  |
| 452 | Monthly Reactive Support and Voltage Control Settlement Debit | 1 | 3 | No |  |  |  |  |
| 454 | Regulation Service Settlement Debit | 1 | 3 | No |  |  |  |  |
| 460 | IESO-Controlled Grid Special Operations Debit | 2 | 2 | No |  |  |  |  |
| 500 | Must Run Contract Settlement Credit | 2 | 2 | No |  |  |  |  |
| 550 | Must Run Contract Settlement Debit | 1 | 3 | No |  |  |  |  |
| 600 | Network Service Credit | 2 | 3 | No |  |  |  |  |
| 601 | Line Connection Service Credit | 2 | 3 | No |  |  |  |  |
| 602 | Transformation Connection Service Credit | 2 | 3 | No |  |  |  |  |
| 603 | Export Transmission Service Credit | 1 | 2 | No |  |  |  |  |
| 650 | Network Service Charge | 2 | 3 | No |  |  |  |  |
| 651 | Line Connection Service Charge | 2 | 3 | No |  |  |  |  |
| 652 | Transformation Connection Service Charge | 2 | 3 | No |  |  |  |  |
| 653 | Export Transmission Service Charge | 1 | 2 | No |  |  |  |  |
| 700 | Dispute Resolution Settlement Credit | 2 | 2 | No |  |  |  |  |
| 702 | Debt Retirement Credit | 2 | 2 | No |  |  |  |  |
| 703 | Rural and Remote Settlement Credit | 2 | 2 | No |  |  |  |  |
| 704 | OPA Administration Credit | 2 | 2 | No |  |  |  |  |
| 705 | Ontario Fair Hydro Plan First Nations On-reserve Delivery Amount | 2 | 2 | No |  |  |  |  |
| 706 | Ontario Fair Hydro Plan Distribution Rate Protection Amount | 2 | 2 | No |  |  |  |  |
| 750 | Dispute Resolution Settlement Debit | 2 | 2 | No |  |  |  |  |
| 751 | Dispute Resolution Board Service Debit | 2 | 2 | No |  |  |  |  |
| 752 | Debt Retirement Charge | 2 | 3 | No |  |  |  |  |
| 753 | Rural and Remote Settlement Debit | 2 | 3 | No |  |  |  |  |
| 754 | OPA Administration Charge | 1 | 3 | No |  |  |  |  |
| 755 | MOE - Ontario Fair Hydro Plan First Nations On-reserve Delivery Balancing Amount | 2 | 2 | No |  |  |  |  |
| 756 | MOE - Ontario Fair Hydro Plan Distribution Rate Protection Balancing Amount | 2 | 2 | No |  |  |  |  |
| 850 | Market Participant Default Settlement Debit (recovery) | 2 | 2 | No |  |  |  |  |
| 851 | Market Participant Default Interest Debit | 2 | 2 | No |  |  |  |  |
| 900 | GST/HST Credit | 2 | 2 | No |  |  |  |  |
| 950 | GST/HST Debit | 2 | 2 | No |  |  |  |  |
| 1050 | Self-Induced Dispatchable Load CMSC Clawback | 1 | 3 | Yes | AQEW multiplied by 12 Resulting Decimals: 3 | Used in the calculation of OP(EMP, AQEW, BL) as the case may be. | Numerators OP(EMP, MQSW, BL) OP(EMP, DQSW, BL) OP(EMP, AQEW, BL) OP(EMP, MC, BL) Denominator: 12 Resulting Decimals: 2 | Profits compared as applicable. |
| 1051 | Ramp-Down CMSC Claw Back | 2 | 2 | No |  |  |  |  |
| 1130 | Day-Ahead Intertie Offer Guarantee Settlement Credit | 1 | 3 | Yes | FOR EACH 5-MINUTE METERING INTERVAL: Numerators OP[EMP, MIN(DQSI, PDR_DQSI), PDR_BE] Denominator: 12 Resulting Decimals: 2 | Results for each 5-minute metering interval are summed for the hour. Profits compared as applicable. |  |  |
| 1131 | Intertie Offer Guarantee Settlement Credit | 1 | 3 | Yes | For each 5 minute metering interval: RT-IOG – Real Time IOG Numerator OP(EMP,MQSI,BE) Denominator: 12 Resulting Decimal: 2 DA-IOG - Day-Ahead IOG Component 1 Numerator OP(EMP, Min(DA_DQSI,DQSI),DA_BE) Denominator: 12 Resulting Decimal: 2 Component 2 Numerator XDA_BE – MAX(0,XBE) Denominator: 12 Resulting Decimal: 2 Component 3 Numerator OP(EMP,MQSI,BE), OP(EMP,DA_DQSI,BE) OP(EMP,DQSI,BE) Denominator: 12 Resulting Decimal: 2 IOG Rate Resulting Decimal: 5 | For DA-IOG, Component 1, Component 2 and Component 3 are compared as applicable. Results of RT-IOG and DA-IOG are compared in IOG OFFSET component. |  |  |
| 1133 | Day-Ahead Generation Cost Guarantee Payment | 1 | 3 | No |  |  |  |  |
| 1134 | Day-Ahead Linked Wheel Failure Charge | 1 | 3 | Yes | RT_EFC_DALW and RT_IFC_DALW for each 5-minute metering interval are summed for the hour. Resulting Decimal: 2 | Results are compared as applicable. |  |  |
| 1135 | Day-Ahead Import Failure Charge | 1 | 3 | Yes | TERM 1 – Operating Profit („OP”) Function used to calculate Failure Charge OP(PD_EMP, DA_DQSI, DA_BE) OP(PD_EMP, PD_DQSI, DA_BE) Resulting Decimals: 2 TERM 2 – Operating Profit („OP”) Function used to calculate Failure Charge OP(PD_EMP, DA_DQSI, PD_BE) OP(PD_EMP, PD_DQSI, PD_BE) Resulting Decimals: 2 TERM 3 – Price cap Numerator Max(0,PD_EMP) x DA_ISD Denominator: 12 Resulting Decimals: 2 | TERM 1, TERM 2 and TERM 3 compared as applicable. |  |  |
| 1136 | Day-Ahead Export Failure Charge | 1 | 3 | Yes | TERM 1 – Operating Profit („OP”) Function used to calculate Failure Charge OP(PD_EMP, DA_DQSW, DA_BL) OP(PD_EMP, PD_DQSW, DA_BL) Resulting Decimals: 2 TERM 2 – Operating Profit („OP”) Function used to calculate Failure Charge OP(PD_EMP, DA_DQSW, PD_BL) OP(PD_EMP, PD_DQSW, PD_BL) Resulting Decimals: 2 | TERM 1, TERM 2 and TERM 3 compared as applicable. |  |  |
| 1137 | Intertie Offer Guarantee Reversal | 2 | 2 | No |  |  |  |  |
| 1138 | Day-Ahead Fuel Cost Compensation Credit | 2 | 2 | No |  |  |  |  |
| 1139 | Intertie Failure Charge Reversal | 2 | 2 | No |  |  |  |  |
| 1142 | Ontario Fair Hydro Plan Eligible RPP Consumer Discount Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1143 | Ontario Fair Hydro Plan Eligible Non-RPP Consumer Discount Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1144 | Ontario Fair Hydro Plan Financing Entity Amount | 2 | 2 | No |  |  |  |  |
| 1145 | Ontario Fair Hydro Plan Financing Entity Interest | 2 | 2 | No |  |  |  |  |
| 1148 | GA Energy Storage Injection Reimbursement | 2 | 2 | No |  |  |  |  |
| 1188 | Day-Ahead Fuel Cost Compensation Debit | 1 | 3 | No |  |  |  |  |
| 1192 | Ontario Fair Hydro Plan Eligible RPP Consumer Discount Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1193 | Ontario Fair Hydro Plan Eligible Non-RPP Consumer Discount Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1194 | Ontario Fair Hydro Plan Financing Entity Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1195 | Ontario Fair Hydro Plan Financing Entity Balancing Interest | 2 | 2 | No |  |  |  |  |
| 1300 | Capacity Based Demand Response Program Availability Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1301 | Capacity Based Demand Response Program Availability Over-Delivery Settlement Amt | 1 | 3 | No |  |  |  |  |
| 1302 | Capacity Based Demand Response Program Availability Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1303 | Capacity Based Demand Response Program Utilization Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1304 | Capacity Based Demand Response Program Utilization Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1305 | Capacity Based Demand Response Program Planned Non-Performance Event Set-Off Amt | 1 | 3 | No |  |  |  |  |
| 1306 | Capacity Based Demand Response Program Measurement Data Set-Off Settlement Amt | 1 | 3 | No |  |  |  |  |
| 1307 | Capacity Based Demand Response Program Buy-Down Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1308 | Capacity Based Demand Response Program Performance Breach Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1309 | Demand Response Pilot – Availability Payment | 1 | 3 | No |  |  |  |  |
| 1310 | Demand Response Pilot – Availability Clawback | 1 | 3 | No |  |  |  |  |
| 1311 | Demand Response Pilot – Availability Charge | 1 | 3 | No |  |  |  |  |
| 1312 | Demand Response Pilot – Availability Adjustment | 1 | 3 | No |  |  |  |  |
| 1313 | Demand Response Pilot – Demand Response Bid Guarantee | 1 | 3 | No |  |  |  |  |
| 1314 | Capacity Obligation – Availability Payment | 1 | 3 | No |  |  |  |  |
| 1315 | Capacity Obligation – Availability Charge | 1 | 3 | No |  |  |  |  |
| 1316 | Capacity Obligation – Administration Charge | 1 | 3 | No |  |  |  |  |
| 1317 | Capacity Obligation – Dispatch Charge | 1 | 3 | No |  |  |  |  |
| 1318 | Capacity Obligation – Capacity Charge | 1 | 3 | No |  |  |  |  |
| 1319 | Capacity Obligation – Buy-Out Charge | 1 | 3 | No |  |  |  |  |
| 1320 | Capacity Obligation – Out of Market Activation Payment | 1 | 3 | No |  |  |  |  |
| 1330 | On behalf of Former OPA for the DR2 Program – Availability Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1331 | On behalf of Former OPA for the DR2 Program – Availability Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1332 | On behalf of Former OPA for the DR2 Program – Utilization Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1333 | On behalf of Former OPA for the DR2 Program – Utilization Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1334 | On behalf of Former OPA for the DR2 Program – Meter Data Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1335 | On behalf of Former OPA for the DR2 Program – Buy-Down Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1336 | On behalf of Former OPA for the DR2 Program – Miscellaneous Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1340 | On behalf of Former OPA for the DR3 Program – Availability Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1341 | On behalf of Former OPA for the DR3 Program – Availability Over-Delivery Settlement Amt | 1 | 3 | No |  |  |  |  |
| 1342 | On behalf of Former OPA for the DR3 Program – Availability Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1343 | On behalf of Former OPA for the DR3 Program – Utilization Payment Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1344 | On behalf of Former OPA for the DR3 Program – Utilization Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1345 | On behalf of Former OPA for the DR3 Program – Planned Non-Performance Event Set-Off Amt | 1 | 3 | No |  |  |  |  |
| 1346 | On behalf of Former OPA for the DR3 Program – Meter Data Set-Off Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1347 | On behalf of Former OPA for the DR3 Program – Buy-Down Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1348 | On behalf of Former OPA for the DR3 Program – Miscellaneous Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1350 | Capacity Based Recovery Amount for Class A Loads | 1 | 3 | No |  |  |  |  |
| 1351 | Capacity Based Recovery Amount for Class B Loads | 1 | 3 | No |  |  |  |  |
| 1380 | Demand Response 2 Availability Payment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1381 | Demand Response 2 Availability Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1382 | Demand Response 2 Utilization Payment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1383 | Demand Response 2 Utilization Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1384 | Demand Response 2 Meter Data Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1385 | Demand Response 2 Buy-Down Balancing amount | 2 | 2 | No |  |  |  |  |
| 1386 | Demand Response 2 Miscellaneous Balancing amount | 2 | 2 | No |  |  |  |  |
| 1390 | Demand Response 3 Availability Payment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1391 | Demand Response 3 Availability Over-Delivery Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1392 | Demand Response 3 Availability Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1393 | Demand Response 3 Utilization Payment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1394 | Demand Response 3 Utilization Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1395 | Demand Response 3 Planned Non-Performance Event Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1396 | Demand Response 3 Meter Data Set-Off Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1397 | Demand Response 3 Buy-Down Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1398 | Demand Response 3 Miscellaneous Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1400 | OPA Contract Adjustment Settlement Amount | 1 | 2 | No |  |  |  |  |
| 1401 | Incremental Loss Settlement Credit | 1 | 6 | No |  |  |  |  |
| 1402 | Hourly Condense System Constraints Settlement Credit | 1 | 5 | No |  |  |  |  |
| 1403 | Speed-no-load Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1404 | Condense Unit Start-up and OM&A Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1405 | Hourly Condense Energy Costs Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1406 | Monthly Condense Energy Costs Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1407 | Condense Transmission Tariff Reimbursement Settlement Credit | 2 | 3 | No |  |  |  |  |
| 1408 | Condense Availability Cost Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1409 | Monthly Condense System Constraints Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1410 | Renewable Energy Standard Offer Program Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1411 | Clean Energy Standard Offer Program Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1412 | Feed-In Tariff Program Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1413 | Renewable Generation Connection – Monthly Compensation Settlement Credit | 1 | 3 | No |  |  |  |  |
| 1414 | Hydroelectric Contract Initiative Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1415 | Conservation Assessment Recovery | 1 | 3 | No |  |  |  |  |
| 1416 | Conservation and Demand Management - Compensation Settlement Credit | 1 | 3 | No |  |  |  |  |
| 1417 | Daily Condense Energy Costs Settlement Credit | 1 | 2 | No |  |  |  |  |
| 1418 | Biomass Non-Utility Generation Contracts Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1419 | Energy from Waste (EFW) Contracts Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1420 | Ontario Electricity Support Program Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1421 | Capacity Agreement Settlement Credit | 0 | 2 | No |  |  |  |  |
| 1422 | Capacity Agreement Penalty Settlement Amount | 0 | 2 | No |  |  |  |  |
| 1423 | Energy Sales Agreement Settlement Credit | 0 | 3 | No |  |  |  |  |
| 1424 | Energy Sales Agreement Penalty Settlement Amount | 0 | 2 | No |  |  |  |  |
| 1425 | Hydroelectric Standard Offer Program Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1427 | Non-Hydro Renewables Funding Amount | 2 | 2 | No |  |  |  |  |
| 1450 | OPA Contract Adjustment Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1451 | Incremental Loss Offset Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1457 | Ontario Electricity Rebate Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1460 | Renewable Energy Standard Offer Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1461 | Clean Energy Standard Offer Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1462 | Feed-In Tariff Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1463 | Renewable Generation Connection – Monthly Compensation Settlement Debit | 1 | 3 | No |  |  |  |  |
| 1464 | Hydroelectric Contract Initiative Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1465 | Ontario Clean Energy Benefit (-10%) Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1466 | Conservation and Demand Management - Compensation Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1467 | Ontario Rebate for Electricity Consumers (8% Provincial Rebate) Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1468 | Biomass Non-Utility Generation Contracts Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1469 | Energy from Waste (EFW) Contracts Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1470 | Ontario Electricity Support Program Balancing Amount | 2 | 3 | No |  |  |  |  |
| 1471 | Capacity Agreement Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1472 | Capacity Agreement Penalty Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1473 | Energy Sales Agreement Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1474 | Energy Sales Agreement Penalty Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1475 | Hydroelectric Standard Offer Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1477 | COVID-19 Energy Assistance Program (CEAP) Settlement Amount | 2 | 2 | No |  |  |  |  |
| 1487 | Non-Hydro Renewables Funding Balancing Amount | 2 | 2 | No |  |  |  |  |
| 1500 | Day-Ahead Production Cost Guarantee Payment – Component 1 and Component 1 Clawback | 1 | 3 | Yes | AQEI is multiplied by 12 Resulting decimal: 3 | Use in the calculation of OP(EMP,AQEI, DA_BE), | For each 5 minute metering interval: Numerator OP(EMP,AQEI, DA_BE), OP(EMP,DQSI, DA_BE), OP(EMP,DA_DQSI, DA_BE) Denominator: 12 Resulting Decimal: 2 Numerator DA_SNLC Denominator: 12 Resulting decimal: 2 Results for each 5-minute metering interval are summed for the hour. | Profits are compared as applicable. |
| 1501 | Day-Ahead Production Cost Guarantee Payment – Component 2 | 1 | 3 | Yes | AQEI is multiplied by 12 Resulting decimal: 3 | Use in the calculation of OP(EMP,AQEI, DA_BE), OP(EMP,AQEI, BE) | For each 5 minute metering interval: Numerator OP(EMP,AQEI, DA_BE), OP(EMP,DQSI, DA_BE), OP(EMP,DA_DQSI, DA_BE) OP(EMP,OPCAP, DA_BE) OP(EMP,AQEI, BE), OP(EMP,DQSI, BE), OP(EMP,DA_DQSI, BE) OP(EMP,OPCAP, BE) Resulting Decimal: 2 | Profits are compared as applicable. |
| 1502 | Day-Ahead Production Cost Guarantee Payment – Component 3 and Component 3 Clawback | 1 | 3 | Yes | AQEI is multiplied by 12 Resulting decimal: 3 | Use in the calculation of OP(EMP,AQEI, BE), | For each 5 minute metering interval: Numerator OP(EMP,AQEI, BE), OP(EMP,DQSI, BE), OP(EMP,DA_DQSI, BE) OP(EMP,MLP, BE) Results for each 5-minute metering interval are summed for the hour. Resulting Decimal: 2 | Profits are compared as applicable. |
| 1503 | Day-Ahead Production Cost Guarantee Payment – Component 4 | 1 | 3 | Yes | For each 5 minute metering interval: Numerators OP(PROR,30R_SQROR,BR), OP(PROR,10NS_SQROR,BR), OP(PROR,10S_SQROR,BR), Denominator: 12 Resulting Decimal: 2 | Profits are compared as applicable. |  |  |
| 1504 | Day-Ahead Production Cost Guarantee Payment – Component 5 | 1 | 3 | No |  |  |  |  |
| 1505 | Day-Ahead Production Cost Guarantee Reversal | 1 | 3 | No |  |  |  |  |
| 1510 | Day-Ahead Generator Withdrawal Charge | 1 | 3 | Yes | For each 5 minute metering interval: Numerators OP(EMP,MLP,DA_BE) or OP(PD_EMP,MLP,DA_BE) Denominator: 12 Resulting Decimal: 2 | Results for each 5-minute metering interval are summed for the hour. |  |  |
| 1550 | Day-Ahead Production Cost Guarantee Recovery Debit | 1 | 3 | No |  |  |  |  |
| 1560 | Day-Ahead Generator Withdrawal Rebate | 1 | 3 | No |  |  |  |  |
| 1600 | Forecasting Service Settlement Amount | 1 | 3 | No |  |  |  |  |
| 1650 | Forecasting Service Balancing Amount | 1 | 3 | No |  |  |  |  |
| 1750 | Dispute Resolution Balancing Amount (Market) | 2 | 2 | No |  |  |  |  |
| 1753 | MOE - Rural and Remote Settlement Debit | 2 | 2 | No |  |  |  |  |
| 2148 | Class B Global Adjustment Prior Period Correction Settlement Amount | 2 | 2 | No |  |  |  |  |
| 2470 | MOE - Ontario Electricity Support Program Balancing Amount | 2 | 2 | No |  |  |  |  |
| 6000 | Ontario Fair Hydro Plan - Regulatory Asset Transfer Amount | 2 | 2 | No |  |  |  |  |
| 6050 | Ontario Fair Hydro Plan - Regulatory Asset Transfer Balancing Amount | 2 | 2 | No |  |  |  |  |
| 6147 | Class A Global Adjustment Deferral Recovery Amount | 1 | 3 | No |  |  |  |  |
| 6148 | Class B Global Adjustment Deferral Recovery Amount | 1 | 3 | No |  |  |  |  |
| 9147 | Class A Global Adjustment Smoothing Balancing Amount | 1 | 3 | No |  |  |  |  |
| 9148 | Class B Global Adjustment Smoothing Balancing Amount | 1 | 3 | No |  |  |  |  |
| 9920 | Adjustment Account Credit | 1 | 1 | No |  |  |  |  |
| 9980 | Smart Metering Charge | 2 | 2 | No |  |  |  |  |
| 9982 | Ontario Rebate for Electricity Consumers (8% Provincial Rebate) Settlement Amount | 2 | 2 | No |  |  |  |  |
| 9983 | Ontario Electricity Rebate Settlement Amount | 2 | 2 | No |  |  |  |  |
| 9984 | COVID-19 Energy Assistance Program (CEAP) Balancing Amount | 2 | 2 | No |  |  |  |  |
| 9990 | IESO Administration Charge | 2 | 3 | No |  |  |  |  |
| 9992 | Ontario Clean Energy Benefit (-10%) Program Settlement Amount | 2 | 2 | No |  |  |  |  |
| 9996 | Recovery of Costs | 2 | 2 | No |  |  |  |  |

## Settlement of Physical Bilateral Contracts

### Governing Rules

Settlement of physical bilateral contracts is discussed in Section 2.1 of Chapter 8, of the IESO market rules.  In summary this particular market rules Section prescribes the prices to be applied to a Physical Bilateral Contract Quantity of Energy Sold (BCQk,b,hm,t) or a Physical Bilateral Contract Quantity of Energy Bought (BCQs,k,hm,t) at a delivery point or an intertie metering point.  This treatment is summarized in the table below with respect to each settlement variable defined in Section 2.1 and charge type described in Section 2.2 of this document.

| Location of Bilateral Contract | Settlement of Selling Market Participant | Settlement of Buying Market Participant | Charge Type |
| --- | --- | --- | --- |
| Non-dispatchable delivery point | - Debit the Physical Bilateral Contract Quantity of Energy Sold (BCQk,b,hm,t) at the 5-Minute Energy Market Price within Ontario (EMPhm,t). | - Credit the Physical Bilateral Contract Quantity of Energy Bought (BCQs,k,hm,t) at the Hourly Ontario Energy Price (HOEP). | 101 |
| Dispatchable delivery point | - Debit the Physical Bilateral Contract Quantity of Energy Sold (BCQk,b,hm,t) at the 5-Minute Energy Market Price within Ontario (EMPhm,t). | - Credit the Physical Bilateral Contract Quantity of Energy Bought (BCQs,k,hm,t) at the 5-Minute Energy Market Price within Ontario (EMPhm,t). | 100 |
| Intertie Metering Point | - Debit the Physical Bilateral Contract Quantity of Energy Sold (BCQk,b,hm,t) at the 5-minute Energy Market Price at the Interties (EMPhi,t). | - Credit the Physical Bilateral Contract Quantity of Energy Bought (BCQs,k,hm,t) at the 5-minute Energy Market Price at the Interties (EMPhi,t). | 100 |

These financial credits and debits are then included the overall settlement amounts calculated for charge types 100 and 101 as per the equations in Section 2.2.

### The Nature of the Bilateral Contract Quantity

| BCQs,k,hm,t | Physical Bilateral Contract Quantity of Energy bought. | Physical bilateral contract quantity of energy in MWh bought by buying market participant ‘k’ from selling market participant ‘s’ at RWM or intertie metering point ‘m’ for each metering interval ‘t’ in settlement hour ‘h’. |
| --- | --- | --- |
| BCQk,b,hm,t | Physical Bilateral Contract Quantity of Energy sold. | Physical bilateral contract quantity of energy in MWh sold by selling market participant ‘k’ to buying market participant ‘b’ at RWM or intertie metering point ‘m’ for each metering interval ‘t’ in settlement hour ‘h’. |

The submission of physical bilateral contract data is governed by Section 2.4 of Chapter 8 of the IESO market rules.  Furthermore, Section 2.3 of Chapter 8 describes 2 distinct “forms” of physical bilateral contract data that may be submitted by the selling market participant.  Specifically, the two forms of such data are as follows:

- Absolute quantities: specifying the absolute quantity of energy in MWh sold by the selling market participant to the buying market participant for each settlement hour at a particular delivery point or intertie metering point; and

- Derived quantities***: specifying that the physical bilateral contract quantity shall be 100% of the energy sold by the selling market participant to the buying market participant for each settlement hour as derived from a particular delivery point value (i.e. NOT an intertie metering point).

Where:

- The delivery point chosen by the selling market participant must belong to either the selling market participant or the buying market participant.

- If the delivery point is designated as a sub-type ‘I’ (injection) delivery point, 100% of all injected energy for each metering interval in each applicable settlement hour shall be used regardless of any physical allocation data.

- If the delivery point is designated as a sub-type ‘W’ (withdrawal) delivery point, 100% of all withdrawn energy for each metering interval in each applicable settlement hour shall be used regardless of any physical allocation data.

*** See derived quantities examples that follow.

| Derived Quantities Example 1: Delivery point belongs to the SELLING market participant and is a sub-type ‘I’ (injection) delivery point. (note parity with EXAMPLE 3) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| metering interval | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| ENERGY QUANTITY | 10 | 10 | 10 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 10 | 10 |
| ENERGY FLOW Injection (I) Withdrawal (W) | I | I | I | I | I | I | W | W | I | I | I | I |
| BCQ value used for settlement purposes (for both the buying and selling market participant) | 10 | 10 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 10 |
| Total Quantity for the hour | 50 (SEE SECTION 2.5.3 FOR THE DATA PRESENTATION OF THE BILATERAL CONTRACT QUANTITY) |  |  |  |  |  |  |  |  |  |  |  |

| Derived Quantities Example 2: Delivery point belongs to the SELLING market participant and is a sub-type ‘W’ (Withdrawal) delivery point. (note parity with EXAMPLE 4) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| metering interval | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| ENERGY QUANTITY | 10 | 10 | 10 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 10 | 10 |
| ENERGY FLOW Injection (I) Withdrawal (W) | I | I | I | W | W | W | W | W | W | W | I | I |
| BCQ value used for settlement purposes (for both the buying and selling market participant) | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 0 | 0 |
| Total Quantity for the hour | 20 (SEE SECTION 2.5.3 FOR THE DATA PRESENTATION OF THE BILATERAL CONTRACT QUANTITY) |  |  |  |  |  |  |  |  |  |  |  |

| Derived Quantities Example 3: Delivery point belongs to the BUYING market participant and is a sub-type ‘I’ (injection) delivery point. (note parity with EXAMPLE 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| metering interval | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| ENERGY QUANTITY | 10 | 10 | 10 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 10 | 10 |
| ENERGY FLOW Injection (I) Withdrawal (W) | I | I | I | I | I | I | W | W | I | I | I | I |
| BCQ value used for settlement purposes (for both the buying and selling market participant) | 10 | 10 | 10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 10 |
| Total Quantity for the hour | 50 (SEE SECTION 2.5.3 FOR THE DATA PRESENTATION OF THE BILATERAL CONTRACT QUANTITY) |  |  |  |  |  |  |  |  |  |  |  |

| Derived Quantities Example 4: Delivery point belongs to the BUYING market participant and is a sub-type ‘W’ (Withdrawal) delivery point. (note parity with EXAMPLE 2) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| metering interval | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| ENERGY QUANTITY | 10 | 10 | 10 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 10 | 10 |
| ENERGY FLOW Injection (I) Withdrawal (W) | I | I | I | W | W | W | W | W | W | W | I | I |
| BCQ value used for settlement purposes (for both the buying and selling market participant) | 0 | 0 | 0 | 0 | 0 | 0 | 10 | 10 | 0 | 0 | 0 | 0 |
| Total Quantity for the hour | 20 (SEE SECTION 2.5.3 FOR THE DATA PRESENTATION OF THE BILATERAL CONTRACT QUANTITY) |  |  |  |  |  |  |  |  |  |  |  |

### Time Resolution of Bilateral Contract Quantities and Rounding

Where a physical bilateral contract takes place at a non-dispatchable delivery point, the Physical Bilateral Contract Quantity of Energy Bought is reported by settlement hour as per the market rules (because the Hourly Ontario Energy Price is applied to this quantity – see Chapter 9, Section 3.3).  At the same location however, the ‘Physical Bilateral Contract Quantity of Energy Sold’ is debited at the 5-minute energy market price.  This latter, sold quantity must therefore be divided into 12, equal metering intervals (see Chapter 9, Section 3.1.6 of the market rules) and rounded to the appropriate number of significant digits (see Section 2.4 of this document).  As a result, the summation of these 12, equal quantities may not equal the original, hourly value submitted in some circumstances due to this intermediate rounding.  The table below summarizes this phenomenon in terms of the location sub-type and the applicable charge type used.  The reader is directed to Section 2.4 of this document for further details.

|  |  | Location Type | Charge Type | Time Resolution used for Settlements Purposes | Intermediate Rounding Applied within Settlements System? |
| --- | --- | --- | --- | --- | --- |
| BCQs,k,hm,t | Physical Bilateral Contract Quantity of Energy bought. | Dispatchable Delivery Point (injection or withdrawal sub-type) | 100 | by metering interval | Yes – See Section 2.4 |
|  |  | Non-Dispatchable Delivery Point (injection or withdrawal sub-type) | 101 | by settlement hour | No |
|  |  | Intertie metering point | 100 | by metering interval | Yes – See Section 2.4 |
| BCQk,b,hm,t | Physical Bilateral Contract Quantity of Energy sold. | Dispatchable Delivery Point (injection or withdrawal sub-type) | 100 | by metering interval | Yes – See Section 2.4 |
|  |  | Non-Dispatchable Delivery Point (injection or withdrawal sub-type) | 101 | by metering interval | Yes – See Section 2.4 |
|  |  | Intertie metering point | 100 | by metering interval | Yes – See Section 2.4 |

### Allocation of Hourly Uplift Components Between Buying and Selling Market Participants

Hourly uplift is defined in Section 3.9.1 of Chapter 9 of the IESO market rules and may be “disaggregated” (sic) on settlement statements into its component parts as per Section 3.9.2.  The following components hourly uplift charges may be allocated from the buying market participant to the selling market participant as per the physical bilateral contract data submitted by the selling market participant (see also, IESO market rules, Chapter 8, Section 2.2.2).

| Hourly Uplift Component Group | Associated Charge Types | Comments |
| --- | --- | --- |
| Net Energy Market Settlement Credit (NEMSC) Hourly Uplift Component (also known as the “Losses” component) | 150 | - This hourly uplift component is an aggregation of charge types 100 (NEMSC), 101 (NEMSC), 104 (TRSC), and 103 (TCRF),. The aggregation of these charge types mathematically resolves down to the value of the difference between AQEI, AQEW, SQEW and SQEI quantities valued at the 5-minute Energy Market Reference Price (EMPhREF,t) for each metering interval in the settlement hour . |
| Operating Reserve Settlement Credit (ORSC) Hourly Uplift Component | 250 252 254 | - Separate charge types for recovery of ORSC settlement amounts paid to market participants for each class of operating reserve. |
| Intertie Failure Charge Rebate (IFCR) Hourly Uplift Component | 186 | Two components as follows: - Charge type 186: an aggregation of charge types 135 (Real-time Import Failure Charge), 136 (Real-time Export Failure Charge), 1134 (Day-Ahead Linked Wheel Failure Charge, 1135 (Day-Ahead Import Failure Charge) and 1136 (Day-Ahead Export Failure Charge). These charge types are primarily rebates back to market participants for amounts collected under these charges. |
| Congestion Management Settlement Credit (CMSC) Hourly Uplift Component | 155 | - Includes recovery of CMSC payments for energy and each class of operating reserve. |
| Transmission Rights Settlement Credit (TRSC) Hourly Uplift Component | NOT USED | - INCLUDED WITH THE “NET ENERGY MARKET SETTLEMENT CREDIT (NEMSC) Hourly Uplift COMPONENT”. - SEE NOTE ABOVE. |
| Transmission Charge Reduction Fund (TCRF) Hourly Uplift Component | NOT USED | - INCLUDED WITH THE “NET ENERGY MARKET SETTLEMENT CREDIT (NEMSC) Hourly Uplift COMPONENT”. - SEE NOTE ABOVE. |
| Operating Reserve Shortfall Settlement Debit (ORSSD) Hourly Uplift Component | 201 203 205 | - Separate charge types for distribution of ORSSD settlement amounts received from market participants for shortfalls in the provision of each class of operating reserve. |

Each hourly uplift component group (i.e. not the individual charge types themselves) may be selected in any combination when the physical bilateral contract data is submitted by the selling market participant.  Confirmation of this selection is included within the settlement statement supporting data files (type “B” records).  A schematic overview of the format of type “B” records may be found within Table 3-2 of the IESO’s Technical Interface Document entitled, “Format Specification for Settlement Statement Files and Data Files”.

The effect of selecting an hourly uplift component group within physical bilateral contract data, is the creation of a “Reallocate Quantity (RQ)”.

The RQ specific to a single physical bilateral contract is exactly equal to the quantity of energy involved in the contract itself.

The RQ specific to a single market participant is equal to the sum of all RQ quantities for which the market participant is the selling market participant, minus the sum of all RQ quantities for which the market participant is the buying market participant.

The RQ specific to a single market participant for a particular hourly uplift component group is equal to the sum of all RQ quantities designated to for that particular hourly uplift component group within physical bilateral contract data for which the market participant is the selling market participant, minus the sum of all RQ quantities for which the market participant is the buying market participant.

This RQ quantity is then applied to the calculation of the settlement amounts for each charge type associated with the hourly uplift component group as per the table above.

Therefore, when calculating the RQ quantity for a particular hourly uplift charge type for market participant ‘k’ at a particular location ‘m’ during a particular metering interval ‘t’, the quantity may be expressed as follows:

RQ k,hm,t = s,b [BCQk,b,hm,t  - BCQs,k,hm,t]

Where all variables are defined as per Section 2.1.

The RQ quantity is then used to either augment or decrease the settlement amount for the hourly uplift charge type “c” as follows:

cM,T TDk,h,c x [(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) / kM,T (AQEWk,hm,t + SQEWk,hi,t)]

Where all variables are defined as per Section 2.1.

In the event that the term,

(AQEWk,hm,t + SQEW k,hi,t + RQk,hm,t) < 0

Where:

RQk,hm,t < 0 and |RQk,hm,t| > |(AQEWk,hm,t + SQEW k,hi,t)| and TDk,h,c > 0

The calculation of the applicable hourly uplift charge type “c” will yield a net credit to the buying market participant as a result of the reallocated quantity exceeding their actual/scheduled withdrawals of energy for the metering interval ‘t’ in question.

The above mechanism applies to those “associated charge types” that are enumerated in the table at the beginning of this Section.  See Section 2.2 for specific listings of charge types and their respective equations.

## Exemptions from the Day-Ahead Import Failure Charge, Day-Ahead Export Failure Charge, and Day-Ahead Linked Wheel Failure Charge

### Purpose of this Section

This section describes how Day-Ahead Import transactions are subject to an “Offer Price Test” in order to determine if they are exempt from the Day-Ahead Import Failure Charge (charge type 1135), Day-Ahead Export Failure Charge (charge type 1136) and Day-Ahead Linked Wheel Failure Charge (charge type 1134).

Generally speaking the applicability of the five Intertie Failure charges is affected by the “Reason Codes” attached to the applicable interchange schedule received by the Settlement Process.  The impact of these Reason Codes is outlined in Table 3-5 of the IESO Technical Interface document entitled, “Format Specifications for Settlement Statement Files and Data Files” (IMP_SPEC_0005).  As noted in that table however, day-ahead import transactions arranged in the pre-dispatch-of-record that include the ‘AUTO’‘NY90’ or ‘ADQh’, or ‘ORA’ Reason Codes in the resulting real-time dispatch will be further subject to an “Offer Price Test” which determines whether or not the transaction in question is in fact exempt from the Day-Ahead Failure Charges.

### Objective of the “Offer Price Test”

The main objective of the Offer Price Test is to grant an exemption from the DA-IFC, DA-EFC and DA-LWFC for those import and export transactions that make a best effort to ensure that they are scheduled in the real-time market.  The Offer Price Test assesses “best effort” on the basis of the offer price of the transaction itself.

### How the Offer Price Test Works

The Offer Price Test is a simple test that is performed on the first lamination of the real-time market import offer/or export bid.  The “first lamination” is defined by the first two price-quantity (“p-q”) pairs in the real-time market offer curve, where:

- The first price-quantity pair contains an offer or bid price and a quantity of zero; and

- The second price-quantity pair contains the same offer or bid price as the first price-quantity pair and a non-zero quantity.

The Offer Price Test applies to any situation in which a day-ahead import or export transaction has a Reason Code, ‘AUTO’, ‘NY90’ ‘ADQh’, or ‘ORA’ assigned to the corresponding real-time import or export transaction at the same location.  It is applicable to any intertie metering point where the underlying constrained scheduling point (CSP) is a “source” (i.e. applicable to imports only) or a “sink” (i.e. applicable to exports only).

If the transaction fails this test; it will not receive exemption status from the DA-IFC or DA-EFC.  If the transaction passes this test, then it will be exempted from the DA-IFC or DA-EFC – without actually changing the Reason Code itself.

### Input Data:

| DA_DQSIk,hi,t | = | Day-ahead constrained quantity scheduled for injection by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ |
| --- | --- | --- |
| PD_DQSIk,hi,t | = | Pre- dispatch constrained quantity scheduled for injection by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. |
| PD_BEk,hi,t | = | Energy offers submitted in Pre-dispatch, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2 |
| - MMCP | = | The Minimum Market Clearing Price. |
| DA_DQSWk,h i,t |  | Day-ahead constrained quantity scheduled for withdrawal by market participant 'k' at intertie metering point 'i' during metering interval 't' of settlement hour 'h' |
| PD_DQSWk,hi,t |  | Pre- dispatch constrained quantity scheduled for withdrawal by market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’. |
| PD_BLk,hi,t |  | Energy bids submitted in pre-dispatch, represented as an N by 2 matrix of price-quantity pairs for each market participant ‘k’ at intertie metering point ‘i’ during metering interval ‘t’ of settlement hour ‘h’ arranged in ascending order by the offered price in each price quantity pair where offered prices ‘P’ are in column 1 and offered quantities ‘Q’ are in column 2 |
| +MMCP | = | The Maximum Market Clearing Price. |

### Decision Logic Applied During the Offer Price Test for Import Transactions:

PART 1:

The first part of the test ensures that the original schedule-of-record schedule (DA_DQSIk,hi,t) for the import transaction is indeed GREATER THAN the resulting Pre-dispatch schedule (PD_DQSIk,hi,t) over the course of settlement hour ‘h’.

IF  ∑T DA_DQSIk,hi,t  >  ∑T PD_DQSIk,hi,t

THEN

Proceed to PART 2

ELSE

END of the test for this transaction.

PART 2:

The second part of the test ensures that the first lamination (i.e. as defined by the first 2 price-quantity pairs) of the offer curve submitted into the pre-dispatch scheduling process:

- Was large enough to cover the entire quantity of the transaction originally scheduled by the schedule-of-record at the same market participant/intertie metering point combination (commonly referred to as a “MP/MSP/CSP triplet”); and,

- Was offered at the Minimum Market Clearing Price (-MMCP).

The test is as follows:

For each metering interval ‘t’ at intertie metering point ‘i’ where the transaction passed PART 1 for settlement hour ‘h’:

Let ‘B’ be matrix PD_BEk,hi,t  (see above for definition).

IF B[2,2] ≥  DA_DQSIk,hi,t  AND B[2,1] = -MMCP

THEN

Allow Reason Code to remain as-is, but exempt the transaction from the DA-IFC.

ELSE

Allow Reason Code to remain as-is, and do NOT exempt the transaction from the DA-IFC.

Implications:

- A day-ahead import transaction must be constrained down to a level lower than its original schedule-of-record schedule in order to receive exemption status;

- The entire amount of the constrained portion of the transaction must have been offered into the Pre-dispatch at –MMCP in order to receive exemption status (compare Figures 2-1 and 2-2 to see examples where this condition is met and not met respectively); and

- Only the first lamination (i.e. the first 2 p-q pairs) of the Pre-dispatch offer curve for each import transaction are relevant in performing this test (due to the existing market rule requirement that offer prices must be monotonically increasing).

### Decision Logic Applied During the Offer Price Test for Export Transactions:

PART 1:

The first part of the test ensures that the original schedule-of-record (DA_DQSWk,hi,t) for the export transaction is indeed GREATER THAN the resulting Pre-dispatch schedule (PD_DQSWk,hi,t) over the course of settlement hour ‘h’.

IF  ∑T DA_DQSWk,hi,t  >  ∑T PD_DQSWk,hi,t

THEN

Proceed to PART 2

ELSE

END of the test for this transaction.

PART 2:

The second part of the test ensures that the first lamination (i.e. as defined by the first 2 price-quantity pairs) of the offer curve submitted into the Pre-dispatch scheduling process:

- Was large enough to cover the entire quantity of the transaction originally scheduled by the schedule-of-record at the same market participant/intertie metering point combination (commonly referred to as a, “MP/MSP/CSP triplet”); and,

- Was offered at the Maximum Market Clearing Price (+MMCP).

The test is as follows:

For each metering interval ‘t’ at intertie metering point ‘i’ where the transaction passed PART 1 for settlement hour ‘h’:

Let ‘B’ be matrix BLk,hi,t  (see above for definition).

IF B[2,2] ≥  DA_DQSWk,hi,t  AND B[2,1] = +MMCP

THEN

Allow Reason Code to remain as-is, but exempt the transaction from the DA-EFC.

ELSE

Allow Reason Code to remain as-is, and do NOT exempt the transaction from the DA-EFC.

Implications:

- A day-ahead export transaction must be constrained down to a level lower than its original schedule-of-record in order to receive exemption status;

- The entire amount of the constrained portion of the transaction must have been offered into the Pre-dispatch at +MMCP in order to receive exemption status (compare Figures 2-1 and 2-

- 2 to see examples where this condition is met and not met respectively); and

- Only the first lamination (i.e. the first 2 p-q pairs) of the Pre-dispatch offer curve for each export transaction are relevant in performing this test (due to the existing market rule requirement that offer prices must be monotonically decreasing).

### Decision Logic Applied During the Offer Price Test for Linked Wheel Transactions:

The test seeks to demonstrate a best efforts attempt to schedule both the import and export legs of a day-ahead linked wheel (DALW) transaction through both:

• A Pre-dispatch bid at positive maximum market clearing price (+MMCP) for a quantity at least equal to the day-ahead export quantity, and

• A Pre-dispatch offer at negative maximum market clearing price (–MMCP) for a quantity at least equal to the day-ahead import quantity.

For import leg of the linked wheel, the decision logic for the price test is described in Section 2.6.5 with the following amendment:

For each metering interval ‘t’ at intertie metering point ‘i’ where the transaction passed PART 1 for settlement hour ‘h’:

Let ‘B’ be matrix PD_BEk,hi,t  (see above for definition).

IF B[2,2] ≥  DA_DQSIk,hi,t  AND B[2,1] = -MMCP

THEN

Allow Reason Code to remain as-is, but exempt the transaction from the RT-IFC-DALW.

ELSE

Allow Reason Code to remain as-is, and do NOT exempt the transaction from the RT-IFC-DALW.

For export leg of the linked wheel, the decision logic for the price test is described in Section 2.6.6 with the following amendment:

For each metering interval ‘t’ at intertie metering point ‘i’ where the transaction passed PART 1 for settlement hour ‘h’:

Let ‘B’ be matrix BLk,hi,t  (see above for definition).

IF B[2,2] ≥  DA_DQSWk,hi,t  AND B[2,1] = +MMCP

THEN

Allow Reason Code to remain as-is, but exempt the transaction from the RT-EFC-DALW.

ELSE

Allow Reason Code to remain as-is, and do NOT exempt the transaction from the RT-EFC-DALW.

Figure 2-1 – Example of an Import Transaction that PASSES the “Offer Price Test”

Figure 2-2 – Example of an Import Transaction that FAILS the “Offer Price Test”

Figure 2-3 – Example of an Export Transaction that PASSES the “Offer Price Test”

Figure 2-4 – Example of an Export Transaction that PASSES the “Offer Price Test”

- End of Section -

References

| Document Name | Doc ID |
| --- | --- |
| Market Rules | MDP_RUL_0002 |
| Format Specification for Settlement Statement Files and Data Files | IMP_SPEC_0005 |
| Ontario Energy Board: Ontario Transmission Rate Schedules EB-2007-0759 | EB-2007-0759 |
| Order-in-Council 137/2008 Ontario Power Generation Rebate | OIC 137/2008 |
| Ontario Regulation 442/01 “Rural or Remote Electricity Rate Protection | 442/01 |
| Ontario Regulation 493/01 “Debt Retirement Charge – Rates and Exemptions” | 493/01 |
| Ontario Regulation 494/01 “Debt Retirement Charge Administration” | 494/01 |
| Legislative Assembly of Ontario S.O. 2003, Chapter 8 “Bill 4, An Act to amend the Ontario Energy Board Act, 1998 with respect to electricity pricing.” Royal Assent: December 18, 2003 | Bill 4 |
| Regulations made pursuant to Bill 4 Ontario Regulation 42/04 made under the Ontario Energy Board Act, 1998. Ontario Regulation 43/04 made under the Ontario Energy Board Act, 1998. | 42/04 43/04 |
| Legislative Assembly of Ontario, Bill 210 – “Electricity Pricing, Conservation and Supply Act, 2002.” S.O. 2002, Chapter 23 Formal Title: “An Act to amend various Acts in respect of pricing, conservation and supply of electricity an in respect of other matters related to electricity.” Royal Assent: December 9, 2002 | Bill 210 |
| Regulations made pursuant to BILL 210 “Electricity Pricing, Conservation and Supply Act, 2002.” Regulation 339/02 (Under the Ontario Energy Board Act, 1998) “Electricity Pricing” – amended by regulation 433/02 Regulation 341/02 (Under the Ontario Energy Board Act, 1998) “Compensation and Set-Offs Under Part V of the Act” – amended by regulation 434/02 Regulation 342/02 (Under the Ontario Energy Board Act, 1998) “Payments to the IMO” – revoked by regulation 432/02 Regulation 432/02 (Under the Ontario Energy Board Act, 1998) “Revoking Ontario Regulation 342/02 (Payments to the IMO)” Regulation 433/02 (Under the Ontario Energy Board Act, 1998) “Amending Ontario Regulation 339/02 (Electricity Pricing)” Regulation 434/02 (Under the Ontario Energy Board Act, 1998) “Amending Ontario Regulation 341/02 (Compensation and Set-Offs Under Part V of the Act)” Regulation 435/02 (Under the Ontario Energy Board Act, 1998) “Payments re Section 79.4 of the Act” Regulation 436/02 (Under the Ontario Energy Board Act, 1998) “Payments re Various Electricity-Related Charges” Regulation 330/09 (Under the Ontario Energy Board Act, 1998) “Cost recovery re section 79.1 of the Act” | 339/02 (amended by 433/02) 341/02 (amended by 434/02) 342/02 (revoked by 432/02) 433/02 434/02 435/02 436/02 330/09 |
| Ontario Energy Board, Independent Electricity Market Operator Licence EI-2003-0088, issued on July 30, 2003 | EI-2003-0088 |
| Legislative Assembly of Ontario, Bill 100 – “Electricity Restructuring Act, 2004” Royal Assent: December 9, 2004 Subject to regulations made pursuant to the “Electricity Restructuring Act, 2004” once proclaimed into force: Ontario regulation 427/04 “Payments to the Financial Corp. re Section 78.2 of the Act” Ontario regulation 428/04 “Payments re Section 79.4 of the Act” Ontario regulation 398/10 Amending Ontario regulation 429/04 “Adjustments Under Section 25.33 of the Act” Ontario regulation 430/04 “Payments re Section 25.33 of the Act” Ontario regulation 431/04 “Payments re Section 25.34 of the Act” Section 78.3 of the (Ontario Energy Board) Act Section 78.4 of the (Ontario Energy Board) Act Section 78.5 of the (Ontario Energy Board) Act | BILL 100 See also, Ontario e-laws website for official Ontario Government Regulation ID numbers at: http://www.e-laws.gov.on.ca |
| Ontario regulation 53/05 made under OEB Act, 1998 re “Payments under Section 78.1 of the Act” Ontario regulation 98/05 made under OEB Act, 1998 re “Payments re Various Electricity-Related Charges” Ontario Regulation 66/10 made under OEB Act, 1998 re “Assessments for Ministry of Energy and Infrastructure Conservation and Renewable Energy Program Costs” | BILL 100 See also, Ontario e-laws website for official Ontario Government Regulation ID numbers at: http://www.e-laws.gov.on.ca/ |
| Ontario Clean Energy Benefit Act, 2010, Ontario Regulation 495/10. |  |
| Ontario Regulation 314/15 “Ontario Electricity Support Program” | 314/15 |
| Ontario Regulation 363/16 made under “Ontario Rebate for Electricity Consumers Act, 2016”. | 363/16 |
| Ontario Regulation 364/16 made under “Ontario Rebate for Electricity Consumers Act, 2016”. | 364/16 |

– End of Document –

Market Rules ref.:  Section 3.6.2 of Chapter 9.

The price test for the Day-Ahead Linked Wheel Failure Charge (1134) is used to determine exemption from the RT-EFC-DALW and RT-IFC-DALW portions only.

Specifically, the Real-time Import Failure Charge (charge type 135), the Real-time Export Failure Charge (charge type 136), the Day-Ahead Import Failure Charge (charge type 1135), the Day-Ahead Export Failure Charge (charge type 1136) and the Day-Ahead Linked Wheel Failure Charge (charge type 1134).