PUBLIC

Market Manual

Market Manual 14: Market Power Mitigation

# Part 14.1: Market Power Mitigation Procedures

Issue 2.1

December 3, 2025

MAN-126

This procedure describes the activities to be undertaken by the IESO and market participants to complete the market power mitigation procedures required to participate in the day-ahead market and the real-time market.

Document Change History

| Issue | Reason for Issue | Date |
| --- | --- | --- |
| 1.0 | Market Transition | November 11, 2024 |
| 2.0 | Issued in advance of MRP Go Live – May 1, 2025 | April 25, 2025 |
| 2.1 | Updated in Baseline 54.1 | December 3, 2025 |

Related Documents

| Document ID | Document Title |
| --- | --- |
| MAN-127 | Market Manual 14.2: Reference Level and Reference Quantity Procedures |

Table of Contents

Table of Contents	i

List of Figures	iv

List of Tables	iv

Table of Changes	v

Market Manual Conventions	vii

1.	Introduction	1

1.1.	Purpose	1

1.2.	Scope	1

1.3.	Overview	2

1.4.	Contact Information	3

2.	Designation of Constrained Areas	4

2.1.	Potential Constrained Area Designations	4

2.2.	Narrow Constrained Area Designation	7

2.3.	Dynamic Constrained Area Designation	9

3.	Designation and Removal of Designation for Uncompetitive Intertie Zones	11

3.1.	Conditions Restricting Competition in an Intertie Zone	11

3.2.	Publication	12

4.	Determination of Global Market Power Reference Intertie Zones	13

4.1.	Evaluating Designations of Global Market Power Reference Intertie Zones	13

4.2.	Publication	13

5.	Ex-Post Mitigation for Physical Withholding	14

5.1.	Physical Withholding Timeline	14

5.2.	Using Reference Quantities	14

5.3.	Determining Which Dispatchable Resources Meet the Conditions to Test for Physical Withholding	15

5.4.	Conduct Test for Energy: Example	15

5.5.	Conduct Test for Operating Reserve: Example	18

5.6.	Impact Test Simulation Methodology	19

5.7.	Determining the Settlement Charge	22

5.8.	Supporting Documentation for Alternative Reference Quantity Value Requests	24

5.9.	Second Notice of Physical Withholding	25

5.10.	Settlement Charges	25

5.11.	Reporting on Physical Withholding	25

6.	Ex-Post Mitigation for Intertie Economic Withholding on an Uncompetitive Intertie Zone	26

6.1.	Sample Intertie Economic Withholding Timeline	26

6.2.	Impact Test Simulation Methodology	27

6.3.	Determining the Settlement Charge	27

6.4.	Supporting Documentation for Requests for Alternative Intertie Reference Level Value	27

6.5.	Applying Settlement Charge	28

6.6.	Reporting on Intertie Economic Withholding	28

List of Acronyms	29

References	30

List of Figures

Figure 2-1: Sample NCA	7

Figure 5-1: Sample Physical Withholding Assessment Timeline	14

Figure 5-2: Scenario 1 with One Instance of Physical Withholding	23

Figure 5-3: Scenario 2 with Two Instances of Physical Withholding	23

Figure 5-4: Scenario 3 with One Instance of Physical Withholding	24

Figure 6-1: Timeline of Ex-Post Mitigation for Intertie Economic Withholding on Uncompetitive Intertie Zones	26

List of Tables

Table 2-1: Designation of DCAs in DAM Based on the Accumulated Hours	9

Table 5-1: Resource Conduct Test (Energy)	15

Table 5-2: Market Control Entity Conduct Test (Energy)	17

Table 5-3: Adjusted Operating Reserve Offer	18

Table 5-4: Adjusted Operating Reserve Offer Example	19

Table of Changes

| Reference | Description of Change |
| --- | --- |
| Section A | Removed Market Transition section |
| Throughout document | Removed zero series references used for market transition |
| 2.1.3 | - Added new section to describe publication details of potential contrained areas |

Market Manual Conventions

This market manual uses the following standard conventions:

The word 'shall' denotes a mandatory requirement;

References to market rule sections and sub-sections may be abbreviated in accordance with the following representative format: ‘MR Ch.1 ss.1.1-1.2’  (i.e. market rules, Chapter 1, sections 1.1 to 1.2).

References to market manual sections and sub-sections may be abbreviated in accordance with the following representative format: ‘MM 1.5 ss.1.1-1.2’  (i.e. market manual 1.5, sections 1.1 to 1.2).

Internal references to sections and sub-sections within this manual take the representative format: ‘sections 1.1 – 1.2’

Terms and acronyms used in this market manual in its appended documents that are italicized have the meanings ascribed thereto in MR Ch.11;

All user interface labels and options that appear on the IESO gateway and tools are formatted with the bold font style; and

Data fields are identified in all capitals.

## Introduction

This market manual describes the market power mitigation framework and the processes by which the IESO shall assess the exercise of global market power and local market power, and specifically the:

designation of constrained areas;

designation of uncompetitive intertie zones;

determination of global market power reference intertie zones;

ex-post mitigation for physical withholding; and

ex-post mitigation of intertie economic withholding on an uncompetitive intertie zone.

The IESO’s assessment and mitigation of the exercise of market power, including testing and any related step by the IESO, shall not constitute a review for compliance with any market rule, including MR Ch.1 s.10A or s.11.

### Purpose

This market manual provides administrative and procedural details to the market rules governing market power mitigation processes, including supplementary information relevant to understanding the rights and obligations of the IESO and market participants.

Market manuals must be read in conjunction with the applicable market rules. Where there is a conflict between a market manual and the market rules, the market rules shall prevail.

### Scope

This market manual supplements the following market rules:

MR Ch.7 s.22.1: Reference Levels - General

MR Ch.7 s.22.10: Designation of Constrained Areas

MR Ch.7 s.22.11: Global Market Power Reference Intertie Zones

MR Ch.7 s.22.12: Uncompetitive Intertie Zones

MR Ch.7 s.22.14: Ex-Ante Mitigation of Economic Withholding

MR Ch.7 s.22.15: Ex-Post Mitigation of Physical Withholding

MR Ch.7 App.7.5: The Day-Ahead Market Calculation Engine Process

MR Ch.7 App 7.5A: The Pre-Dispatch Calculation Engine Process

MR Ch.7 App 7.6: The Real-Time Calculation Engine Process

MR Ch.9 s.5.4: Ex-Post Mitigation for Physical Withholding

MR Ch.9 s.5.5: Ex-Post Mitigation for Economic Withholding on Uncompetitive Interties

### Overview

Reference Levels and Reference Quantities

This manual describes how reference levels and reference quantities are used in some of the IESO’s ex-ante and ex-post market power mitigation processes. For a detailed description of the processes used to establish and calculate reference levels and reference quantities, refer to MM 14.2.

Designation of Constrained Areas and Global Market Power Reference Intertie Zones

This manual describes the processes the IESO uses to designate potential constrained areas, narrow constrained areas, dynamic constrained areas and global market power reference intertie zones used in ex-ante market power mitigation. For the market rules that apply to the ex-ante market power mitigation processes, refer to MR Ch.7 s.22.14, Appendix 7.5 – Appendix 7.5A.

Section 2 describes the designation of constrained areas, which affect when offers are tested for ex-ante mitigation and which conduct and impact thresholds are used in these tests.

Section 4 describes the designation of global market power reference interties, which affect when offers are tested for ex-ante mitigation for global market power.

Ex-Post Mitigation

This manual describes processes the IESO uses to assess physical withholding and intertie economic withholding.

Section 5 describes how ex-post mitigation for physical withholding is carried out by the IESO including conditions for testing, conduct and impact thresholds, opportunities for market participant input and potential outcomes of a finding of physical withholding.

Section 6 describes how ex-post mitigation for intertie economic withholding is carried out by the IESO, including conditions for testing, conduct and impact thresholds, opportunities for market participant input and potential outcomes of a finding of intertie economic withholding.

Section 3 describes the designation of uncompetitive intertie zones, which affect when offers or bids from intertie traders are tested for ex-post mitigation for intertie economic withholding.

Settlement Mitigation

This manual describes the conduct and impact thresholds used in make-whole payment mitigation. Refer to MM 5.5 for more details on make-whole payments that are subject to settlement mitigation.

### Contact Information

Changes to this market manual are managed via the IESO Change Management process. Stakeholders are encouraged to participate in the evolution of this market manual via this process.

As part of the authorization and registration process, market participants are required to identify a Market Power Mitigation Contact. If a market participant has not identified a specific contact, the IESO will seek to contact the Primary Contact for activities within this procedure, unless alternative arrangements have been established between the IESO and the market participant.

To contact the IESO, market participants can email IESO Customer Relations at customer.relations@ieso.ca or use telephone or mail. Telephone numbers and the mailing address can be found on the IESO website. The IESO Customer Relations staff will respond as soon as possible.

– End of Section –

## Designation of Constrained Areas

The IESO identifies circumstances when competition may be restricted in localized areas and designates these areas as potential constrained areas. The IESO identifies potential constrained areas that are regularly impacted by binding transmission constraints. Depending on how frequently the transmission constraints bind in an area, a potential constrained area may be subsequently designated as one of the following:

a narrow constrained area (NCA); or

a dynamic constrained area (DCA).

### Potential Constrained Area Designations

(MR Ch.7 s.22.10.1)

When identifying and revising potential constrained area designations, the IESO will consider relevant configuration changes to the IESO-controlled grid, which can include, but are not limited to:

network model build updates, such as the addition or removal of a transmission facility or a resource;

system configuration changes that can affect a potential constrained area, such as new or removed transmission facilities and changed operating security limits (OSLs);

the need to add or remove a dispatchable resource to a potential constrained area;

a long-term outage that could affect a potential constrained area, such as a transmission facility outage or a generation facility outage; and

system element, transmission line, resource, or OSL name changes that may impact corresponding element names used in potential constrained areas.

Input Data

The data that the IESO will consult when identifying and revising potential constrained area designations may include, but is not limited to:

the real-time locational marginal price (LMP) congestion component (based on five-minute intervals) for the previous 365 days;

the sensitivity factors or generation shift factors (GSFs) of different resources on different transmission line constraints and OSLs;

the Zone ID for each resource, which represents the zone the resource belongs to among the 10 zones in Ontario (e.g. Toronto, East, Northwest, etc.);

the list of exisiting transmission facilities, branch groups, OSLs and previously identified potential constrained areas;

the real-time five-minute historical binding data including shadow prices for transmission facilities and OSLs, outages, and the GSFs for the previous 365 days; and

the impact of actual or expected material configuration changes to the IESO-controlled grid in the next 365 days on the congestion component of LMPs, sensitivity factors or GSFs and OSLs.

Methodology

The process by which the IESO identifies and revises potential constrained area designations consists of two activities:

- grouping resources whose real-time market LMP congestion components are closely correlated into a potential constrained area; and

- identifying the transmission facilities, branch groups and/or related OSLs for that potential constrained area, where the resources identified in the first activity can resolve import congestion on those constraints.

Inputs:

The IESO relies on various inputs in order to carry out the activities described above. These inputs include, but are not limited to, the following:

- analysis of the historical and prospective resources’ annual real-time congestion LMP component; specifically:

the frequency that real-time congestion components are greater than zero; and

in areas with negative congestion components, the difference between real-time congestion components.

- temporal correlations between the real-time congestion LMP components; and

- other information that identifies relative electrical proximity of resources.

Activity 1: Grouping Resources into Potential Constrained Areas

The IESO may group the resources according to potential constrained area by:

- determining the electrical zone that each resource is located within;

- comparing annual average of the congestion LMPs;

- calculating the mean square error of the congestion LMP probability density functions;

- calculating the temporal correlation coefficient for all resources against other resources to identify occasions when congestion at one resource moves similarly to congestion at other resources;

- comparing sensitivity factors of resources on the same transmission facilities, branch groups or OSLs to determine the electrical proximity of resources to other resources and the direction of their power injection.

The above analysis will be jointly considered to identify which resources should be grouped into each potential constrained area.

Activity 2: Identifying Transmission Facilities and Branch Groups for each Potential Constrained Area

The IESO will determine the transmission facilities and branch groups for each potential constrained area by first calculating the temporal correlation between the congestion LMP component and the real-time five-minute historical and prospective shadow prices for transmission facilities and OSLs.

This will identify a list of prospective transmission facilities and branch groups that may be added to a particular potential constrained area.

The IESO will then identify the sensitivity factors that apply for each resource in each group for each transmission facility and OSL related to a relevant branch group.

Where the group of resources in a potential constrained area have a significantly high sensitivity factor against a particular transmission facility or OSL related to a relevant branch group, that constraint will be added to the potential constrained area.

To supplement this analysis, the IESO may confirm the relationship between a particular OSL or transmission facility and a resource through historical analysis. This involves comparing the sensitivity factor of a resource against the transmission facility or OSL to the historical congestion component that occurred at a resource. Where this historical analysis shows that the transmission facility or OSL is not strongly related to the congestion component at a resource, that transmission facility or OSL will not be included in the potential constrained area.

Publication

(MR Ch.7 ss.22.10.1.3 and 22.10.1.4 )

The IESO publishes to the IESO website a list of potential constrained areas that includes the following information:

publication date and the dates upon which a potential constrained area designation takes effect;

the dispatchable generation resources and dispatchable electricity storage resources within each potential constrained area; and

a list of the transmission facilities and branch groups that make up each potential constrained area.

### Narrow Constrained Area Designation

Applying Designation Criteria

(MR Ch.7 s.22.10.2.2)

When designating a potential constrained area as a narrow constrained area, the IESO considers a potential constrained area import constrained if at least one transmission facility or OSL related to a relevant branch group is binding in either the day-ahead market (DAM) or the real-time market (RTM). A transmission facility or OSL is considered to be binding when the shadow price on the relevant constraint is non-zero.

When multiple transmission facilities or OSLs related to a relevant branch group in a potential constrained area are binding in the same hour, a single hour will be counted toward the 4% condition for narrow constrained area designation (see Figure 2-1).

The IESO assesses whether a potential constrained area is import constrained in the day-ahead market on an hourly basis. For the real-time market, if the potential constrained area was import constrained for one interval within an hour, the entire hour will be considered to have been import constrained. If the full data set is not available for the IESO-determined study period, the IESO will use the data that is available in the study period to make the designation and whether the potential constrained area was import constrained in more than 4% of the hours will be assessed against that reduced data set.

NCA

Line 1

Line 2

Line 3

In this hypothetical narrow constrained area, Line 1 and Line 2 were binding at the same hour.  This would be counted as 1 hour toward satisfying the requirement that 4% of the transmission facilities or OSLs were binding in the previous 365 days.

Figure 2-1: Sample NCA

Publication

(MR Ch.7 s.22.10.2.1 and 22.10.2.3)

Although the IESO designates NCAs once a year, it publishes a daily report pursuant to MR Ch.7 s.22.10.2.1 that includes the following information:

version number;

publication date and the dates upon which an NCA designation or removal takes effect;

the dispatchable generation resources and dispatchable electricity storage resources within each NCA;

a list of the transmission facilities and branch groups that make up each NCA; and

the congestion frequency data that the IESO used to determine such designation.

The IESO publishes a daily NCA report because it may update the NCA designations pursuant to MR Ch.7 s.22.10.2.3 at any time to:

remove a resource from an existing NCA if that resource no longer requires reference levels in accordance with MR Ch.7 s.22.1.1; and

update the name of for any system element (transmission facility, resource, or branch group) used in existing NCAs.

Transmission facilities and OSLs will not be added or removed from an NCA, nor will NCA designations be changed, in updates made pursuant to MR Ch.7 s.22.10.2.3.

Effective Date

(MR Ch.7 s.22.10.2.4)

- NCA designations made pursuant to MR Ch.7 s.22.10.2.1 will come into effect no sooner than thirty business days following the publication of the report. Any changes made pursuant to MR Ch.7 s.22.10.2.3 will come into effect no sooner than two days following the update.

### Dynamic Constrained Area Designation

Applying Designation Criteria

(MR Ch.7 s.22.10.3)

When designating a potential constrained area as a dynamic constrained area pursuant to MR Ch.7 s.22.10.3.1, the IESO considers a potential constrained area import constrained if at least one transmission facility or OSL related to a relevant branch group is binding for a dispatch hour. A transmission facility or OSL is considered to be binding when the shadow price on the relevant constraint is non-zero.

When multiple transmission facilities or OSLs related to a relevant branch group in a potential constrained area are binding in the same hour, a single hour will be counted toward the 15% condition for a dynamic constrained area designation made pursuant to MR Ch.7 s.22.10.3.2.

For the real-time market, if the potential constrained area was import constrained for one interval within an hour, the entire hour will be considered to have been import constrained.

For example, Table 2-1 displays the designation for a hypothetical DCA in the day-ahead market based on the accumulated hours for a period of 12 days. The table shows that the area was binding for four hours for five days in a row (Day-1 to Day-5).

At 06:00 on Day-6, 20 hours were binding in the previous 120 hours (Day-1 to Day-5). As this is more than 15% of the previous 120 hours (i.e. 18 hours), the criterion for designating the DCA is satisfied.

The DCA was designated from Day-6 onward from Day-6 to Day-10 (i.e., for five days), regardless of the number of binding hours in those days because 120 hours must pass before the designation will be reassessed.

After the first 120 hours following a DCA designation, the status of the designation is reassessed every day on a rolling basis. For the day-ahead market, the status is assessed at 06:00 every day for the next dispatch day.

At 06:00 on Day-10, the status of the designation is determined for Day-11. At that time the number of binding hours is calculated for the preceding 120 hours, which in this case was 21 hours (higher than 18 hours). Therefore, the DCA designation is extended for Day-11 in the day-ahead market.

At 06:00 on Day-11, the status of the designation is determined for Day-12. At that time, the number of binding hours for the last 120 hours was only 14 hours (lower than 18 hours), so the DCA designation is removed for Day-12 in the day-ahead market.

Table 2-1: Designation of DCAs in DAM Based on the Accumulated Hours

| Day | Day-1 | Day-2 | Day-3 | Day-4 | Day-5 | Day-6 | Day-7 | Day-8 | Day-9 | Day-10 | Day-11 | Day-12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Binding Hours | 4 | 4 | 4 | 4 | 4 | 7 | 0 | 4 | 5 | 5 | 0 | 7 |
| Accumulated Binding Hours (for the last 120 hours) | 0 | 4 | 8 | 12 | 16 | 20 | 23 | 19 | 19 | 20 | 21 | 14 |
| DCA Active | No | No | No | No | No | Yes | Yes | Yes | Yes | Yes | Yes | No |

Publication

(MR Ch.7 s.22.10.3.4)

The IESO publishes the DAM DCA Designation Report daily and the RTM DCA Designation Report hourly.

Both the DAM DCA Designation Report and the RTM DCA Designation Report include the following information:

version number;

publication and effective dates; publication date and the date and time when the DCA designation or removal of designation takes effect;

information that indicates whether the DCA designations in that report apply to the day-ahead market or the real-time market;

information that indicates that a potential constrained area is designated as a DCA;

the dispatchable generation resources within each DCA;

a list of the transmission facilities and branch groups that make up the DCA including the number of binding hours for each transmission facility; and

the congestion frequency data that the IESO used to determine such designation.

– End of Section –

## Designation and Removal of Designation for Uncompetitive Intertie Zones

(MR Ch.7 s.22.12)

This section provides additional details with respect to the processes the IESO uses to designate and remove designations for uncompetitive intertie zones in accordance with MR Ch.7 s.22.12.

The process that the IESO uses to assess intertie economic withholding on an uncompetitive intertie zone is further detailed in section 6 of this manual.

### Conditions Restricting Competition in an Intertie Zone

(MR Ch.7 s.22.12.1)

The conditions for designating an intertie zone as uncompetitive are provided in MR Ch.7 s.22.12.1. If the full data set is not available for a calendar quarter, the IESO will use the data that is available to make the designation. The assessment of whether a market participant received ninety percent of the day-ahead market scheduled energy withdrawals or injections will use that reduced data set.

The IESO considers the following conditions as restricting competition when determining whether effective competition in an intertie zone is or will be restricted as described in MR Ch.7 s.22.12.1.2:

lack of a market for supply of imports or demand for exports with open access to transmission in the neighbouring control area;

the existence of institutional or regulatory barriers to trading in the neighbouring control area;

the existence of physical barriers to trading in the neighbouring control area, such as limited transmission controlled by one party or captive load at the intertie zone; and

the existence of economic barriers to trading in the neighbouring control area, such as substantial transmission access fees.

- If, following the IESO’s assessment, an intertie zone that is designated as uncompetitive no longer meets the criteria that resulted in the designation, but the IESO reasonably expects that the criteria will be met following a transitory period, the designation will not be removed.

- If the IESO designates an intertie zone as uncompetitive under MR Ch.7 s.22.12.1.2 due to an expected future restriction to competition on that intertie zone, then the effective date for the designation will be no sooner than the date when competition is expected to be restricted.

### Publication

(MR Ch.7, s.22.12.4)

The IESO publishes the following information regarding a change to an intertie zone’s designation status:

the relevant intertie zone;

whether the intertie zone was designated as uncompetitive or had its designation removed;

the publication date of the change;

the effective date of the change;

the criteria the IESO used in its decision to designate the intertie zone as uncompetitive or to remove such designation, as the case may be.

– End of Section –

## Determination of Global Market Power Reference Intertie Zones

### Evaluating Designations of Global Market Power Reference Intertie Zones

(MR Ch.7 s.22.11.1)

The IESO considers intertie zones that have at least 500 MW of total transfer capacity absent de-rates, outages or effects of ambient conditions, to be of sufficient size relative to the IESO-administered markets to be able to provide effective competitive discipline.

The IESO may modify and evaluate the designation of global market power reference intertie zones when:

  - a new intertie zone is added;

  - there is a material change in the amount of electricity trade that an existing intertie zone can accommodate; or

  - there is a material change in market structure or regulation in a neighbouring control area.

### Publication

(MR Ch.7 s.22.11.3)

The IESO publishes the following information regarding a change to a global market power reference intertie zone designation status:

  - the relevant global market power reference intertie zone;

  - the criteria that resulted in a change to a designation;

  - the publication date of the change; and

  - the effective date of the change.

Designations remain in effect until a new designation takes effect.

– End of Section –

## Ex-Post Mitigation for Physical Withholding

(MR Ch.7 s.22.15)

The IESO may assess physical withholding by a resource in both or either of the day-ahead market and the real-time market for a dispatch day. If the IESO selects a single market, the IESO will deem the MWs withheld in the other market to be 0 for all relevant dispatch hours when calculating the ex-post mitigation for physical withholding settlement amount.

### Physical Withholding Timeline

Figure 5-1 illustrates the timeline associated with physical withholding assessment activities described above.

Figure 5-1: Sample Physical Withholding Assessment Timeline

The IESO will only issue one first notice per dispatch day to a resource. If the IESO discontinues an assessment after sending a first notice to the market participant, the IESO will notify the market participant of the discontinuation.

### Using Reference Quantities

A resource’s day-ahead reference quantity value is used to assess physical withholding by that resource in the day-ahead market.

A resource’s real-time reference quantity value is used to assess physical withholding by that resource in the real-time market.

### Determining Which Dispatchable Resources Meet the Conditions to Test for Physical Withholding

(MR Ch.7 ss.22.15.4 and 22.15.11)

The IESO considers the conditions in the day-ahead market calculation engine and the hour-ahead run of the pre-dispatch calculation engine when determining which dispatchable resources meet the conditions for testing for physical withholding of energy (MR Ch.7 s.22.15.4) or operating reserve (MR Ch.7 s.22.15.11).

In addition, to assess physical withholding that can impact a commitment decision for a GOG-eligible resource, the IESO considers the conditions in the hour-ahead run of the pre-dispatch calculation engine that was the last opportunity to operationally commit that GOG-eligible resource for a given dispatch hour. In these cases, a GOG-eligible resource must meet conditions for a given dispatch hour in both of these hour-ahead runs of the pre-dispatch calculation engine.

### Conduct Test for Energy: Example

(MR Ch.7 ss.22.15.5, 22.15.6, and 22.15.7)

The following examples illustrate the conduct test for a set of hypothetical resources that share a market control entity for physical withholding:

Table 5-1 represents the hypothetical resources assessed in accordance with the ‘resource conduct test’, related to MR Ch.7 ss.22.15.5.1.1 and 22.15.5.2.1.

Table 5-2 represents the same hypothetical resources assessed in accordance with the ‘market control entity conduct test’, related to MR Ch.7 ss.22.15.5.1.2 and 22.15.5.2.2.

Note that the same resources may be assessed with respect to both the resource conduct test and the market control entity conduct test.

Table 5-1: Resource Conduct Test (Energy)

| Resource Name | Constrained Area Condition | Eligible to be Tested for Resource Conduct Test? | Resource’s Offered Energy Quantity | Resource’s Reference Quantity | Resource Conduct Test Outcome |
| --- | --- | --- | --- | --- | --- |
| GENERATOR A | MR Ch.7 s.22.15.4.5 | Yes | 999 MW | 1000 MW | Pass |
| GENERATOR B | None | No | N/A | N/A | N/A |
| GENERATOR C | MR Ch.7 s.22.15.4.3 | Yes | 0 MW | 100 MW | Fail |
| GENERATOR D | MR Ch.7 s.22.15.4.3 | Yes | 198 MW | 200 MW | Pass |
| GENERATOR E | MR Ch.7 s.22.15.4.4 | Yes | 198 MW | 200 MW | Pass |
| GENERATOR F | MR Ch.7 s.22.15.4.4 | Yes | 198 MW | 200 MW | Pass |
| GENERATOR G | MR Ch.7 s.22.15.4.4 | Yes | 198 MW | 200 MW | Pass |
| GENERATOR H | MR Ch.7 s.22.15.4.4 | Yes | 0 MW | 1000 MW | Fail |
| GENERATOR I | MR Ch.7 s.22.15.4.4 | Yes | 198 MW | 200 MW | Pass |
| GENERATOR J | MR Ch.7 s.22.15.4.4 | Yes | 198 MW | 200 MW | Pass |

All of the dispatchable resources listed above (Generators A through J) are registered under the same market control entity for physical withholding.

GENERATOR B is not eligible to be tested for physical withholding as it has not met any constrained area condition.

GENERATOR C and GENERATOR H failed the resource conduct test and therefore will be tested under the impact test regardless of the outcome of the market control entity conduct test.

GENERATOR A and GENERATOR D have passed the resource conduct test with respect to the constrained area conditions that they have met but are still subject to be tested under the market control entity conduct test.

Seven dispatchable resources passed the resource conduct test and are, therefore, subject to be tested under the market control entity test. Of these seven, five (E, F, G, I and J) are tested under the market control entity test for MR Ch.7 s.22.15.4.4, one is tested under the market control entity test for MR Ch.7 s.22.15.4.5 and one is tested under the market control entity test for MR Ch.7 s.22.15.4.3.

Table 5-2: Market Control Entity Conduct Test (Energy)

| Resource Name | Constrained Area Condition | Eligible to be Tested for MCE Conduct Test? | Resources’ Aggregate Offered Energy Quantity | Resources’ Aggregate Reference Quantity | MCE Conduct Test Outcome |
| --- | --- | --- | --- | --- | --- |
| GENERATOR E, GENERATOR F, GENERATOR G, GENERATOR I, GENERATOR J | MR Ch.7 s.22.15.4.4 | Yes | 990 MW | 1000 MW | Fail |
| GENERATOR A | MR Ch.7 s.22.15.4.5 | Yes | 999 | 1000 | Pass |
| GENERATOR D | MR Ch.7 s.22.15.4.3 | Yes | 198 | 200 | Pass |

Each of the five resources (E, F, G, I and J) have failed the market control entity conduct test for the MR Ch.7 s.22.15.4.4 constrained area condition. This is because these resources’ aggregated energy offer quantities were less than the applicable conduct threshold.

Resource A passes the market control entity conduct test for the MR Ch.7 s.22.15.4.5 constrained area condition as resource (A)’s energy offer quantity was equal to or greater than the applicable conduct threshold.

Similarly, resource D passes the market control entity conduct test for the MR Ch.7 s.22.15.4.3 constrained area condition as resource (D)’s energy offer quantity was equal to or greater than the applicable conduct threshold.

### Conduct Test for Operating Reserve: Example

(MR Ch.7 s.22.15.12)

Table 5-3 shows how the conduct test for operating reserve treats offers of different classes of operating reserve.

Note that the resource conduct test for offers for operating reserve is applied for each class of operating reserve. With respect to the classes of operating reserve:

10S operating reserve is counted as 10S, 10N and 30R for the purposes of the conduct test;

10N operating reserve is counted as 10N and 30R operating reserve for the purposes of the conduct test; and

30R operating reserve is counted as only 30R for the purposes of the conduct test.

Table 5-3: Adjusted Operating Reserve Offer

| Reserve Class | Adjusted Operating Reserve Offer for Physical Withholding Conduct Test |
| --- | --- |
| 10-minute synchronized (10S) | = 10S OR Offer |
| 10-minute non-synchronized (10NS) | For NQS Resources = MIN (10S OR Offer + 10NS OR Offer, Maximum Generator Resource Active Power Capability - Min Loading Point) For QS Resources = MIN (10S OR Offer + 10NS OR Offer, Maximum Generator Resource Active Power Capability) For Dispatchable Load Resources = MIN (10S OR Offer + 10NS OR Offer, Maximum Load Active Power) |
| 30-minute synchronized (30R) | For NQS Resources = MIN (10S OR Offer + 10NS OR Offer + 30R OR Offer, Maximum Generator Resource Active Power Capability - Min Loading Point) For QS Resources = MIN (10S OR Offer + 10NS OR Offer + 30R OR Offer, Maximum Generator Resource Active Power Capability) For Dispatchable Load Resources = MIN (10S OR Offer + 10NS OR Offer + 30R OR Offer, Maximum Load Active Power) |

Table 5-4 illustrates an example of offers of operating reserve and how these offers would be reflected as inputs to the conduct test for physical withholding for operating reserve.

Table 5-4: Adjusted Operating Reserve Offer Example

| Market Participant Operating Reserve Offers | Adjusted Operating Reserve Offer for Physical Withholding Conduct Test |
| --- | --- |
| 40 MW of 10S | = 10S OR Offer = 40 MW |
| 50 MW of 10NS | = 10S OR Offer + 10NS OR Offer = 40 MW + 50 MW = 90 MW |
| 60 MW of 30R | = 10S OR Offer + 10NS OR Offer + 30R OR Offer = 40 MW + 50 MW + 60 MW = 150 MW |

### Impact Test Simulation Methodology

(MR Ch.7 ss.22.15.8-22.15.10 and ss.22.15.15-22.15.17)

Resource Grouping

(MR Ch.7 ss.22.15.10 and 22.15.17)

When the IESO applies an impact test for physical withholding pursuant to MR Ch.7 s.22.15.10 or s.22.15.17 the specific resource offers that will be modified in a particular simulation for a dispatch day will be determined based on the constrained area conditions met by each relevant resource and which of those resources share a market control entity for physical withholding.

Because a resource can appear in more than one narrow constrained area, dynamic constrained area or local operating reserve area, it could have its offers modified in more than one simulation for physical withholding. The IESO will issue a first notice of physical withholding based on the simulation that produces the lowest simulated reference quantity locational marginal price.

##### No Grouping

A resource will be placed in the “no grouping” category if it fails a conduct test for physical withholding for a dispatch hour in a particular dispatch day and does not share a market control entity for physical withholding with another resource that fails a conduct test for physical withholding for a dispatch hour in the same dispatch day.

The IESO will determine the simulated reference quantity locational marginal prices for each resource in the “no grouping” category by modifying offers for each of these resources in isolation (no other resource offers will be modified in that simulation).

##### Grouping

Resources that met one or more of the conditions in MR Ch.7 s.22.15.4.5, 22.15.4.6 or 22.15.11.3, failed a conduct test for physical withholding for a dispatch hour in a dispatch day and share a market control entity for physical withholding will be grouped according to the condition that they met to determine simulated reference quantity locational market prices and simulated as-offered locational market prices.

Resources that met one or more of the conditions in MR Ch.7 s.22.15.4.3, 22.15.4.4 or 22.15.11.4, failed a conduct test for physical withholding for a dispatch hour in a dispatch day and share a market control entity for physical withholding will be grouped according to the particular narrow constrained area, dynamic constrained area or operating reserve area they belong to in order to determine simulated reference quantity locational market prices and simulated as-offered locational market prices.

Inputs for Simulated Reference Quantity Locational Marginal Price

(MR Ch.7 ss.22.15.10 and 22.15.18)

For resources that did not submit offers:

If a market participant did not submit an energy offer or an offer for operating reserve for a resource, the IESO shall calculate the simulated reference quantity energy locational marginal price and the simulated reference quantity operating reserve locational marginal price using the resource’s reference level values up to the relevant reference quantity value.

For resources that submitted offers:

If a market participant submitted an energy offer or an offer for operating reserve with a maximum quantity lower than the resource’s energy or operating reserve reference quantity, the IESO determines the simulated reference quantity energy locational marginal price and the simulated reference quantity operating reserve locational marginal price by creating a combined offer-reference level curve.

This combined offer-reference level curve will be identical to the offer used to determine schedules and LMPs up to the maximum quantity of the submitted offer. For the MWs of the combined offer-reference level curve between the maximum quantity in the submitted offer and the maximum quantity in the reference quantity value, the prices and quantities in the combined offer-reference level curve will match the reference level values as long as these laminations will not result in the combined offer-reference level curve violating price monotonicity.

If following this approach results in combined offer-reference level curve laminations that violate price monotonicity, then the offer prices for the laminations in the reference quantity values above the maximum offer lamination will be set to the maximum price in the offer used to determine schedules and LMPs.

Electricity Storage Resource Impact Test

(MR Ch.7 ss.22.15.8)

If an electricity storage resource fails a conduct test for physical withholding in the day-ahead market or the real-time market, the IESO selects a single dispatch hour to carry out the impact test to assess physical withholding pursuant to MR Ch.7 s.22.15.8 as follows:

- The IESO reviews dispatch hours when the resource failed the conduct test and selects any dispatch hour(s) when the resource met the conditions below as follows:

For energy offers:

The IESO selects any dispatch hour(s) when the resource met the conditions in MR Ch.7 s.22.15.8.1.

If the resource did not meet the conditions above in any of the dispatch hours reviewed, the IESO selects any dispatch hour(s) that met the conditions in MR Ch.7 s.22.15.8.2.

If the resource did not meet the conditions above in any of the dispatch hours reviewed, the IESO selects any dispatch hour(s) that met the conditions in MR Ch.7 s.22.15.8.3.

If the resource did not meet the conditions above in any of the dispatch hours reviewed, the IESO selects any dispatch hour(s) that met the conditions in MR Ch.7 s.22.15.8.4.

For offers for operating reserve:

- If the resource did not meet the conditions above in any of the dispatch hours reviewed, the IESO will select the dispatch hour(s) that met the conditions in MR Ch.7 s.22.15.15.1.

- If the resource did not meet the conditions above in any of the dispatch hours reviewed, the IESO will select the dispatch hour(s) when the resource met the conditions in MR Ch.7 s.22.15.15.2.

- If two or more dispatch hours were selected following the review in (1), the IESO selects the dispatch hour(s) with the highest LMP in the day-ahead market for the resource, when assessing physical withholding in the day-ahead market, or the hour-ahead run of the pre-dispatch calculation engine, when assessing physical withholding in the real-time market.

- If two or more dispatch hours were selected pursuant to (2), the IESO selects the dispatch hour(s) with the highest hourly market demand based on the total energy data from the “Day-Ahead Totals Report”, when assessing physical withholding in the day-ahead market, or based on the total energy data from hour-ahead version of the “Pre-Dispatch Totals Report”, when assessing physical withholding in the real-time market.

- If two or more dispatch hours were selected pursuant to (3), the IESO selects the dispatch hour that occurred first.

### Determining the Settlement Charge

(MR Ch.9 s.5.4)

The IESO determines a settlement charge for energy and operating reserve for each dispatch day containing a settlement hour where the impact test was failed, as set out in MR Ch.9 s.5.4.1.

Persistence Multipliers

The persistence multiplier for physical withholding in MR Ch.9 s.5.4.1.1(b) (for energy) and MR Ch.9 s.5.4.1.2(b) (for operating reserve) is determined based on repeat failures of the impact test for physical withholding by a market control entity for physical withholding.

A persistence multiplier is used when determining a settlement charge in a first and second notice of physical withholding and also when determining the calculation of the settlement amount, as set out in MR Ch.9 s.5.4.1. The persistence multiplier starts at a value of one and increases by one for each second notice issued to any resources that share a market control entity for physical withholding in the 18-month period prior to the instance of physical withholding being assessed. The maximum value for the persistence multiplier is three. Calculation of the persistence multiplier excludes instances when an ex-post mitigation for physical withholding settlement amount is reversed as a result of a notice of disagreement, as of the time of the calculation of the persistence multiplier.

The following examples outline several scenarios.

Scenario 1: One Instance

Figure 5-2: Scenario 1 with One Instance of Physical Withholding

Because there were no previously issued second notices in the 18 months prior to the current second notice, the persistence multiplier is equal to one.

Scenario 2: Two Instances

Figure 5-3: Scenario 2 with Two Instances of Physical Withholding

Because there was a second notice issued in the 18-month period prior to the current second notice, the persistence multiplier is equal to two.

Scenario 3: One Instance

Figure 5-4: Scenario 3 with One Instance of Physical Withholding

There was no previously issued second notice within the 18-month period, though there was a prior instance that had only progressed as far as the issuance of a first notice. The persistence multiplier is, therefore, equal to one.

### Supporting Documentation for Alternative Reference Quantity Value Requests

(MR Ch.7 s.22.15.20.1)

Market participants submitting requests that the IESO use an alternative reference quantity value pursuant to MR Ch.7 s.22.15.20 must include documentation with their request to support any resource-specific considerations that were not accounted for in the resource’s reference quantities in use during the instance of physical withholding. This supporting documentation may include, but is not limited to, data regarding:

ambient temperature;

relative humidity;

water conditions (water flow, water level etc.);

actions taken to ensure the safety of any person, prevent the damage of equipment, prevent the violation of any applicable law, or to maintain the reliability of the IESO-controlled grid;

energy offers submitted by other resources located at the same facility, if the available supply of the facility is less than the sum of the registered maximum generator active power capability of all resources at that facility;

other resource-specific considerations that were not accounted for in the registered energy or operating reserve reference quantity formula;

IESO staff requesting that a resource reduces offer quantities or removes offers;

planned outages and equipment de-ratings; and

forced outages and equipment de-ratings.

### Second Notice of Physical Withholding

(MR Ch.7 ss.22.15.23-22.15.25)

If the conduct test and impact test are failed using an alternative reference quantity value, then the IESO will send a second notice of physical withholding that will contain updates to the information that was provided in the first notice.

### Settlement Charges

(MR Ch.7 s.22.15.25.1)

Settlement charges related to physical withholding are applied after the IESO issues the second notice of physical withholding to the market participant.

### Reporting on Physical Withholding

The IESO publishes a report each month with the following information:

monthly total and number of settlement amounts issued for physical withholding of energy and operating reserve;

timestamp of the report creation; and

the relevant month.

– End of Section –

## Ex-Post Mitigation for Intertie Economic Withholding on an Uncompetitive Intertie Zone

(MR Ch.7 ss.22.17-22.19)

This section provides details on the process for assessing intertie economic withholding on uncompetitive intertie zones using the relevant conduct tests and impact tests in MR Ch.7 ss.22.17-22.19.

The IESO may assess intertie economic withholding by a boundary entity resource in both or either of the day-ahead market and the real-time market for a dispatch day. If the IESO selects a single market, the IESO will deem the MWhs withheld in the other market to be 0 for all relevant dispatch hours when calculating the intertie economic withholding settlement amount.

### Sample Intertie Economic Withholding Timeline

Figure 6-1 illustrates the activities associated with intertie economic withholding on uncompetitive intertie zones:

Figure 6-1: Timeline of Ex-Post Mitigation for Intertie Economic Withholding on Uncompetitive Intertie Zones

### Impact Test Simulation Methodology

(MR Ch.7 ss.22.17.8 and 22.17.14)

Inputs for Simulated Intertie Reference Level Locational Marginal Price

The simulated intertie reference level energy locational marginal price calculated pursuant to MR Ch.7 s.22.17.8 or 22.17.14 will be determined using a combined offer/bid-intertie reference level curve for a boundary entity resource that failed the conduct test. The IESO determines the combined offer/bid-intertie reference level by replacing all price components of each offer lamination that failed the conduct test with the intertie reference level and then reordering the laminations to respect price monotonicity.

Determining the Simulated Intertie Reference Level Locational Marginal Price Based on Uncompetitive Intertie Zones

All submitted boundary entity resource import offer and export bid curves that failed the conduct test for the same dispatch hour of the same dispatch day for a market participant at an uncompetitive intertie zone for a given dispatch hour will be replaced by their combined offer/bid-intertie reference level curves simultaneously to determine the simulated intertie reference level locational marginal price. For a market participant that has boundary entity resource offer or bid curves at multiple uncompetitive intertie zones for a given dispatch hour that have failed the conduct test, the offers or bids on each uncompetitive intertie zone will be replaced by their applicable combined offer/bid-intertie reference level curves simultaneously to determine the simulated intertie reference level locational marginal price.

### Determining the Settlement Charge

(MR Ch.7 ss.22.19.7, 22.19.8 and MR Ch.9 s.5.5)

The IESO determines an ex-post mitigation for economic withholding on uncompetitive interties settlement amount for each dispatch day that contains a settlement hour where the impact test was failed, as set out in MR Ch.9 s.5.5.

### Supporting Documentation for Requests for Alternative Intertie Reference Level Value

(MR Ch.7 s.22.19.3)

The IESO evaluates the supporting documentation provided to determine whether it is consistent with the alternative intertie reference level value requested.

Alternative intertie reference level values are based on short-run marginal costs for importers and short-run marginal benefits for exporters.

With respect to importers, the short-run marginal cost is the cost of the power purchased or produced to serve Ontario taking into account the transaction costs. With respect to exporters, the short-run marginal benefit is the price the exporter received or would have received on the sale of the power purchased from Ontario, taking into account the transaction costs. The IESO only considers actual after-the-fact costs.

The IESO will not consider fixed costs, sunk costs or operational expenses that are not directly incurred to undertake any specific transaction nor benefits that are not a direct result of undertaking any specific transaction.

If the IESO determines an alternative intertie reference level value, the IESO shall perform the conduct test and impact test using the alternative intertie reference level value. If the conduct test and impact test still fail using the alternative intertie reference level value, the IESO will issue a second notice of intertie economic withholding. If the conduct and impact tests do not fail when using the alternative intertie reference level value, the assessment concludes and no ex-post mitigation for economic withholding on uncompetitive interties settlement amount is issued.

### Applying Settlement Charge

(MR Ch.7 ss.22.19.6-22.19.8)

Settlement charges related to intertie economic withholding are applied after the IESO issues the second notice of intertie economic withholding to the market participant in accordance with MR Ch.7 s.22.19.6 or 22.19.7.

### Reporting on Intertie Economic Withholding

The IESO publishes a report each month with the following information:

monthly total and number of settlement amounts issued for intertie economic withholding;

timestamp of the report creation; and

the relevant month.

– End of Section –

List of Acronyms

| Acronym | Term |
| --- | --- |
| DCA | Dynamic constrained area |
| GOG | Generator Offer Guarantee |
| GSF | Generation shift factor |
| LMP | Locational marginal price |
| MR | Market rules |
| NCA | Narrow constrained area |
| NQS | Non-quick start |
| OSL | Operating security limit |
| QS | Quick-start |
| OR | Operating Reserve |
| DAM | Day-ahead market |
| RTM | Real-time market |

– End of Section –

References

| Document ID | Document Title |
| --- | --- |
| RUL-6 to RUL-24 | Market Rules |
| MAN-107 | Market Manual 1.3: Identity Management Operations Guide |
| MAN-116 | Market Manual 5.5: IESO-Administered Markets Settlement Amounts |

- – End of Document –

Refer to MM 1.5 for adding and updating contact roles with the IESO.

For more information on how reference quantities are determined, refer to MM 14.2: Reference Level and Reference Quantity Procedures.